Spatio-temporal Variation of Water Quality Parameters and Heavy Metals in Sukhna Lake

ABSTRACT

Sukhna Lake, a prominent man-made reservoir in Chandigarh, India, serves essential ecological and recreational roles but is increasingly vulnerable to pollution and seasonal stresses. This study investigates the spatio-temporal variations in the lake’s water quality by analysing key physicochemical parameters (pH, dissolved oxygen [DO], biochemical oxygen demand [BOD]) and heavy metal concentrations (Pb, Cd, Cu, Mn) across three seasons—winter, spring, and summer. Water samples were systematically collected from three representative locations and analysed using standard APHA protocols. Results revealed significant seasonal differences (p < 0.05) in most parameters. Spring exhibited the most favourable water quality, with the highest DO (12.50 ± 0.50 mg/L), lowest BOD (1.00 ± 0.00 mg/L), and moderately alkaline pH (7.50 ± 0.50), indicating high photosynthetic activity and lower organic load. Conversely, summer was the most ecologically stressed season, marked by low DO (6.83 ± 1.04 mg/L), high BOD (2.67 ± 1.44 mg/L), and acidic pH (6.32 ± 0.18), reflecting elevated microbial decomposition and pollutant concentration due to high temperatures and reduced dilution. Heavy metal concentrations remained within WHO/EPA limits but showed seasonal patterns. Lead and copper levels peaked slightly in spring (0.0050 mg/L and 0.0100 mg/L, respectively), while cadmium showed higher values in winter and summer (0.0017 mg/L), suggesting seasonal runoff and anthropogenic influence. The study concludes that Sukhna Lake’s water quality is highly sensitive to seasonal dynamics, particularly during summer months. These findings emphasize the need for season-specific management strategies, including enhanced runoff control, pollution mitigation, and continuous monitoring.

Keywords: Sukhna Lake, seasonal variation, water quality, BOD, DO, heavy metals, urban lake management.

INTRODUCTION

Freshwater systems play very vital role in balancing the nature, harbouring diverse ecosystems, and providing services of paramount importance. They provide clean drinking water and irrigation methods, are exploited for numerous recreational pursuits, and mainly, stabilizing the climate. The system has been increasingly internally threatened by their respective uses, especially through urbanization, industrial and agricultural developments. Pollution of water bodies thereby threatens their ecological well-being and also diminishes the value of the services they render. The case of Sukhna Lake in Chandigarh, India, is very indicative of growing pressure on an urban freshwater system. This artificial reservoir, constructed in 1958, provides several vital resources, including recreation, irrigation, and biodiversity. In recent decades, the lake ecosystem has been severely degraded due to intensified urbanization, recreational exploitation, and industrial encroachment around Sukhna Lake. Changes in the water quality of the lake are fairly evident and occur both spatially and temporally due to these pressures. Spatial problems are related to seasonal processes because sediments and contaminants accumulate near inlets. "Spatio-temporal variation in water quality" refers to water quality variations in parameters among various locations (spatial variation) and with time (temporal variation). Correct assessment of these variations can be important in water resource management, pollution control, and sustainable development. Sukhna lake remains a major wetland in climatic moderation, flood prevention, and water filtration. It lowers temperatures by as much as 3.52°C in summers and hence contributes to lessening the urban heat island effect. The lake supports a huge variety of flora and fauna, thereby increasing ecological resilience and biodiversity (Guang-ren, 2012) (Rahul Aditya, 2023).(Sun et al., 2023) (Saha et al., 2021) (Biswas et al., 2024)  (Zhao et al., 2012). Water quality is greatly impacted by urbanization, sedimentation, and seasonal fluctuations, which raises a number of environmental and health issues. Sedimentation can make these problems worse by changing the flow and quality of water, while urbanization brings pollutants from home and industry. Nutrients, pathogens, and heavy metals are entering water bodies as a result of urbanization, thereby impacting water quality significantly. Stormwater runoff increases as cities grow because impermeable surfaces like parking lots, sidewalks, and roadways block natural infiltration. Numerous pollutants, including as pesticides, fertilizers, automobile emissions, and industrial waste, are carried by this runoff straight into neighbouring lakes, rivers, and reservoirs. Further deterioration of water quality is also caused by untreated or inadequately treated wastewater discharges from homes, businesses, and industry. When these contaminants build up, they distort natural cycles of ecosystems, causing algal blooms, low oxygen levels, and damage to fish and other aquatic life and sometimes extinction of some aquatic species in extreme cases. According to scientific research, urbanization can cause water bodies to contain noticeably more pollutants than less developed areas. For example, metropolitan streams may have two to three times the quantities of bromide and chloride, which are frequently linked to road salt, industrial discharge, and wastewater. Long-term environmental hazards are posed by heavy metals like lead, mercury, and cadmium that build up in sediments from rusted pipelines, electronic trash, and vehicle emissions. Eutrophication can be accelerated by nutrient overload from phosphorus and nitrogen, which are frequently present in fertilizers and detergents. This can result in an overabundance of aquatic plants and deteriorating water quality. (Pang et al., 2022) (Valtanen et al., 2014) , (Matej-Lukowicz et al., 2020),(Pochodyła-Ducka et al., 2023) (Chauhan and Paliwal, 2009) (Estrada-Rivera et al., 2022) .This work offers baseline data for monitoring the ecological health of Sukhna Lake and supports efforts for effective water resource management, environmental protection, and policymaking in urban freshwater ecosystems.

EXPERIMENTAL PROTOCOL

This study examines the seasonal change in water quality at Sukhna Lake in Chandigarh by examining physicochemical characteristics and heavy metal concentrations during three separate seasons: rainy, winter, and summer. Water samples were taken seasonally to determine how environmental changes affect the lake's health and pollution levels. Seasonal changes can alter physicochemical qualities and increase pollutant discharge, particularly heavy metals, which pose environmental and health problems. By comparing data over seasons, this study hopes to identify trends that might help with sustainable lake management and pollution control techniques.

METHODOLOGY

Sample collection site

For the purpose of assessing spatial variation in water quality, water samples were systematically collected from three to four distinct sites across Sukhna Lake, which is situated in Chandigarh city, India. The selection of these sites was intended to capture a representative understanding of the lake’s hydrological and environmental conditions, taking into account possible differences due to location, anthropogenic influence, and surrounding land use.

Figure 1: Sample Collection Site

Sample collection

Water samples were collected from three strategically selected sites within Sukhna Lake to ensure representative coverage of spatial variability in water quality. The sampling sites were chosen based on factors such as proximity to inflow points, human activity, and vegetation cover. All samples were collected in pre-cleaned, sterile high-density polyethylene (HDPE) containers to prevent contamination and ensure the integrity of the samples. Standard sampling protocols were followed during collection, including rinsing the containers with the lake water prior to final sampling, labelling each sample accurately, and maintaining appropriate handling and storage conditions for subsequent laboratory analysis.

Determination of pH

The pH of water samples was measured using a calibrated digital pH meter (Model: [Insert Model Number], Manufacturer: [Insert Manufacturer]). Prior to each measurement session, the pH meter was calibrated using standard buffer solutions of pH 4.00, 7.00, and 10.00 to ensure accuracy. Measurements were performed in accordance with the APHA Standard Methods for the Examination of Water and Wastewater (23rd Edition, 2017), Method 4500-H⁺ B. The pH value provides crucial insight into the acidic or alkaline nature of water, influencing various chemical and biological processes within the aquatic system.

Determination of Dissolved Oxygen (DO)

Dissolved Oxygen (DO) concentrations were determined using the Winkler titration method, which remains a widely accepted technique for surface water analysis due to its accuracy and reliability. Water samples were fixed on-site by adding manganous sulfate and alkaline iodide-azide reagents. The fixed samples were then acidified, releasing iodine equivalent to the amount of oxygen present, which was titrated using a standard sodium thiosulfate solution. DO levels were recorded at five different time intervals (DO₁ to DO₅) to facilitate the calculation of Biochemical Oxygen Demand (BOD). All procedures conformed to the APHA (2017), Method 4500-O C.

Determination of Biochemical Oxygen Demand (BOD)

Biochemical Oxygen Demand (BOD₅) was evaluated using the 5-day BOD test following APHA Method 5210 B (23rd Edition, 2017). Water samples were incubated in airtight BOD bottles at 20°C ± 1°C for a duration of five days in a dark chamber to prevent photosynthetic oxygen interference. BOD was calculated by measuring the difference between the initial DO (DO₁) and the final DO (DO₅) using the following equation:

BOD ​=DO1​−DO5​

This test assesses the oxygen demand associated with the microbial degradation of organic matter under aerobic conditions. BOD serves as an important indicator of biodegradable organic pollution and treatment efficiency in wastewater systems.

To stabilize metal ions, water samples were collected in clean, acid-washed polyethylene bottles and immediately acidified with ultrapure nitric acid to lower the pH below 2. This ensured the preservation of dissolved metal species. Samples were stored at 4°C until laboratory analysis. The concentrations of lead (Pb) and mercury (Hg) were determined using Inductively Coupled Plasma Mass Spectrometry (ICP-MS) following the standard protocols of the APHA, 24th Edition (2023). Fluoride (F⁻), a non-metallic ion, was analyzed using an ion-selective electrode technique. At the same time, sulphate (SO₄²⁻) was measured using either the turbidimetric method or ion chromatography, depending on sample characteristics and equipment availability. All analyses were conducted at the Regional Advanced Water Testing Laboratory, Punjab (RAWTL, Punjab) under strict quality assurance protocols. These included the use of procedural blanks, sample duplicates, and certified reference standards to ensure analytical precision and accuracy. Final results were expressed in milligrams per liter (mg/L), as recommended by APHA (2023).

RESULTS

Table 1 presents the seasonal variation in key water quality parameters. The pH ranged from neutral in winter (7.00 ± 0.50) to slightly alkaline in spring (7.50 ± 0.50) and decreased to acidic levels in summer (6.32 ± 0.18). Dissolved oxygen (DO) concentrations were highest in spring (12.50 ± 0.50 mg/L), followed by winter (8.17 ± 0.76 mg/L), and lowest in summer (6.83 ± 1.04 mg/L), reflecting the influence of temperature and biological activity. Biochemical oxygen demand (BOD) was lowest in spring (1.00 ± 0.00 mg/L), indicating low organic pollution; however, it increased in winter (1.83 ± 1.44 mg/L) and peaked in summer (2.67 ± 1.44 mg/L), suggesting higher organic loads and microbial activity during warmer months. Heavy metal concentrations showed minor seasonal variations: lead (Pb) remained stable in winter and summer (0.0043 ± 0.0006 mg/L) but slightly increased in spring (0.0050 ± 0.0000 mg/L); cadmium (Cd) was lowest in spring (0.0010 ± 0.0000 mg/L) and higher in winter and summer (0.0017 ± 0.0006 mg/L); copper (Cu) was slightly elevated in spring (0.0100 ± 0.0000 mg/L) compared to winter (0.0097 ± 0.0006 mg/L) and summer (0.0090 ± 0.0000 mg/L); and manganese (Mn) followed a similar pattern, with a spring peak (0.0050 ± 0.0000 mg/L). All metal concentrations remained well below WHO and EPA safety limits, indicating no significant contamination.

The one-sample t-test results indicated that the seasonal mean values for most physicochemical parameters—pH, dissolved oxygen (DO), lead (Pb), cadmium (Cd), copper (Cu), and manganese (Mn)—were statistically significant (p < 0.05) across all seasons, suggesting reliable presence above zero levels. Biochemical oxygen demand (BOD) was not significant in winter (p = 0.0852), indicating greater variability or lower reliability in that season. In contrast, BOD values in spring and summer were significant. Constant values in spring (e.g., Pb, Cd, Cu) produced infinite t-values, yet these were still statistically significant. Overall, the analysis confirms consistent seasonal variation and corresponding variaton in detectable concentrations of the measured parameters. (Refers to Table 2)

The seasonal variations in key physicochemical parameters are illustrated in Figures 2, 3, and 4. As shown in Figure 2, pH values were slightly alkaline during winter (7.00) and spring (7.50) but decreased in summer (6.32), indicating a mild shift toward acidity in warmer months. Figure 3 highlights the Biochemical Oxygen Demand (BOD), which was lowest in spring (1.00 mg/L), suggesting reduced organic pollution during that period. BOD values increased in winter (1.83 mg/L) and peaked in summer (2.67 mg/L), likely due to elevated microbial decomposition and organic load. Figure 4 shows that Dissolved Oxygen (DO) concentrations were highest in spring (12.50 mg/L), possibly due to increased photosynthetic activity and lower water temperatures, whereas DO levels declined in winter (8.17 mg/L) and further in summer (6.83 mg/L), likely because of higher water temperatures reducing oxygen solubility and increasing consumption. Figures 5 to 8 illustrate the seasonal variation in the concentrations of selected heavy metals and ions. Figure 5 shows that cadmium (Cd) levels were higher in winter and summer (0.0017 mg/L) compared to spring (0.0010 mg/L), with noticeable variability in winter and summer. In Figure 6, lead (Pb) concentrations remained relatively stable across all seasons, slightly higher in spring (0.0050 mg/L), while winter and summer values were nearly identical (0.0043 mg/L). Figure 7 indicates a peak in copper (Cu) concentration during spring (0.0100 mg/L), with slightly lower values in winter (0.0097 mg/L) and a drop in summer (0.0090 mg/L), suggesting a seasonal influence on copper availability. As shown in Figure 8, manganese (Mn) levels were marginally higher in spring (0.0050 mg/L) compared to winter (0.0043 mg/L) and summer (0.0047 mg/L), with slight fluctuations but overall stable values across seasons.

Table 1: Seasonal variation in physicochemical parameters and heavy metals of water samples

| Parameter | Winter (Mean ± SD) | Spring (Mean ± SD) | Summer (Mean ± SD) |
| --- | --- | --- | --- |
| pH | 7.00 ± 0.50 | 7.50 ± 0.50 | 6.32 ± 0.18 |
| DO (mg/L) | 8.17 ± 0.76 | 12.50 ± 0.50 | 6.83 ± 1.04 |
| BOD (mg/L) | 1.83 ± 1.44 | 1.00 ± 0.00 | 2.67 ± 1.44 |
| Lead (Pb, mg/L) | 0.0043 ± 0.0006 | 0.0050 ± 0.0000 | 0.0043 ± 0.0006 |
| Cadmium (Cd, mg/L) | 0.0017 ± 0.0006 | 0.0010 ± 0.0000 | 0.0017 ± 0.0006 |
| Copper (Cu, mg/L) | 0.0097 ± 0.0006 | 0.0100 ± 0.0000 | 0.0090 ± 0.0000 |
| Manganese (Mn, mg/L) | 0.0043 ± 0.0006 | 0.0050 ± 0.0000 | 0.0047 ± 0.0006 |

Values are presented as mean ± standard deviation (SD) for winter, spring, and summer seasons. Parameters include pH, dissolved oxygen (DO), biochemical oxygen demand (BOD), and concentrations of lead (Pb), cadmium (Cd), copper (Cu), and manganese (Mn), measured in mg/L where applicable

Table 2: One-sample t-test results of physicochemical parameters across different seasons (N = 3).

| Parameter | Season | Mean ± SD | t-value | p-value | Significance (p < 0.05) |
| --- | --- | --- | --- | --- | --- |
| pH | Winter | 7.00 ± 0.50 | 24.25 | 0.0002 | Significant |
|  | Spring | 7.50 ± 0.50 | 25.98 | 0.0001 | Significant |
|  | Summer | 6.32 ± 0.18 | 61.10 | 0.0000 | Significant |
| Dissolved Oxygen (mg/L) | Winter | 8.17 ± 0.76 | 18.59 | 0.0003 | Significant |
|  | Spring | 12.50 ± 0.50 | 43.30 | 0.0000 | Significant |
|  | Summer | 6.83 ± 1.04 | 11.36 | 0.0008 | Significant |
| Biochemical Oxygen Demand (mg/L) | Winter | 1.83 ± 1.44 | 2.20 | 0.0852 | Not Significant |
|  | Spring | 1.00 ± 0.00 | ∞ (constant) | 0.0000 | Significant |
|  | Summer | 2.67 ± 1.44 | 3.21 | 0.0477 | Significant |
| Lead (Pb, mg/L) | Winter | 0.0043 ± 0.0006 | 12.30 | 0.0007 | Significant |
|  | Spring | 0.0050 ± 0.0000 | ∞ (constant) | 0.0000 | Significant |
|  | Summer | 0.0043 ± 0.0006 | 12.30 | 0.0007 | Significant |
| Cadmium (Cd, mg/L) | Winter | 0.0017 ± 0.0006 | 5.20 | 0.0343 | Significant |
|  | Spring | 0.0010 ± 0.0000 | ∞ (constant) | 0.0000 | Significant |
|  | Summer | 0.0017 ± 0.0006 | 5.20 | 0.0343 | Significant |
| Copper (Cu, mg/L) | Winter | 0.0097 ± 0.0006 | 28.07 | 0.0001 | Significant |
|  | Spring | 0.0100 ± 0.0000 | ∞ (constant) | 0.0000 | Significant |
|  | Summer | 0.0090 ± 0.0000 | ∞ (constant) | 0.0000 | Significant |
| Manganese (Mn, mg/L) | Winter | 0.0043 ± 0.0006 | 12.30 | 0.0007 | Significant |
|  | Spring | 0.0050 ± 0.0000 | ∞ (constant) | 0.0000 | Significant |
|  | Summer | 0.0047 ± 0.0006 | 12.30 | 0.0007 | Significant |

Values are expressed as mean ± standard deviation. Statistical significance tested against a null hypothesis of mean = 0. p < 0.05 is considered significant.

Graphical representation of results

Figure 2

Figure 3

Figure 4

Figure 5				            	Figure 6

Figure 7 						Figure 8

Fig. 2-8. Seasonal variation in physicochemical and heavy metal parameters of water samples

The seasonal variation in water quality parameters of Sukhna Lake shows distinct patterns. The pH is highest in spring (7.50) due to increased photosynthetic activity and lowest in summer (6.32), likely due to organic acid accumulation. Dissolved Oxygen (DO) also peaks in spring (12.50 mg/L), benefiting from enhanced algal activity and lower temperatures, while it drops in summer (6.83 mg/L) that might be due to higher temperatures and microbial respiration. Biochemical Oxygen Demand (BOD) is lowest in spring (1.00 mg/L), indicating better water quality, and increases in summer (2.67 mg/L) as organic matter decomposition intensifies. Among the heavy metals, lead (Pb) and manganese (Mn) concentrations are highest in spring, possibly due to seasonal runoff, while their levels are lower in winter and summer. Cadmium (Cd) and copper (Cu) are slightly reduced in spring, with higher values in winter and summer, likely influenced by sediment disturbance and reduced dilution during drier months.

DISCUSSION

The seasonal study of water quality indices in Sukhna Lake demonstrates dynamic ecological changes caused by both natural processes and human activities. Key physicochemical variables, including pH, dissolved oxygen (DO), and biochemical oxygen demand (BOD), as well as heavy metal concentrations, showed significant temporal fluctuations that corresponded to seasonal shifts. The pH values varied dramatically across seasons, ranging from somewhat acidic in summer (6.32 ± 0.18) to highly alkaline in spring (7.50 ± 0.50). The spring surge is due to increased photosynthetic activity and algae blooms, which reduce CO₂ and increase pH. In contrast, the decline in summer pH is most likely caused by increased microbial decomposition, which produces organic acids under warmer temperatures. These findings are consistent with the literature, which links increased temperatures and microbial respiration to acidification in aquatic environments. The pH values in aquatic systems follow different seasonal cycles. For example, in soft-water lakes in South Africa, pH readings are higher in the summer than in the winter, indicating a seasonal pH variation (Schutte and Elsworth, 1954). Similarly, research in northern Wisconsin lakes have revealed yearly pH ranges of 1.0 to 1.5 units, with the lowest values often occurring in late winter and early spring (Kratz et al., 1987). Temperature plays an important effect in seasonal pH dynamics. Changes in carbon dioxide and oxygen pressures have a greater influence on the physiological processes and behaviour of aquatic species (Gobler and Baumann, 2016). Dissolved oxygen (DO) values followed similar seasonal trends. Spring has the greatest DO (12.50 ± 0.50 mg/L) which could be ascribed to higher photosynthetic production and colder temperatures, which improve oxygen solubility. Summer's lowest DO (6.83 ± 1.04 mg/L) is due to oxygen depletion from microbial respiration and lower solubility in warm water. This cyclical hypoxia means that summer brings increased ecological stress, potentially endangering aquatic life. Dissolved oxygen concentrations frequently follow a seasonal pattern, peaking in the winter and falling to their lowest point in the summer. For example, in the Gulf of Aqaba, Red Sea, dissolved oxygen had a maximum and uniform distribution in winter and a minimum in summer (Badran, 2001). Similarly, in Sanya Bay, dissolved oxygen concentrations were typically lowest in the fall and highest in the winter for the majority of the seven years analyzed, from 2004 to 2010. (Xu et al. 2016). Surface water DO in a montane stream stayed near 100% saturation during years of observation, whereas hyporheic zone DO varied more dynamically over storm events, seasonal, and inter-annual timeframes (Soulsby et al., 2009).  The summer season has the greatest average BOD (2.67 ± 1.44 mg/L), indicating increased organic load and microbial activity. Spring had the lowest BOD (1.00 ± 0.00 mg/L), indicating excellent water quality. The increased BOD in the summer corresponds to increased biological metabolism and runoff-induced organic pollutants, establishing the negative link between DO and BOD. Similarly, in rivers such as the Chengi River, the greatest BOD was reported before to the monsoon season, while the lowest was found during the monsoon season. The greatest BOD levels were found in Kaptai Lake prior to the monsoon season, whereas the lowest levels were found after the monsoon. The Shuvolong and Risang waterfalls have the greatest BOD levels after the monsoon season, and the lowest during the monsoon season. (Islam et al.,2022).  Heavy metal analysis emphasizes seasonal impacts. Spring had greater levels of metals like lead (0.0050 ± 0.0000 mg/L) and copper (0.0100 ± 0.0000 mg/L), probably because to increased surface runoff from nearby metropolitan areas. Interestingly, Cadmium values were lowest in the spring, indicating diluting effects or lower input sources. Manganese levels were mostly steady but slightly elevated in spring, indicating seasonal runoff or sediment interactions. Seasonal changes in heavy metal concentrations in aquatic settings are a complicated phenomenon impacted by a variety of elements like as temperature, rainfall, runoff, and sediment dynamics. (balakrishnan et al., 2015a).

Heavy metal concentrations in sediments in the Muthupet lagoon, on India's southeast coast, were lowest during the post-monsoon and summer seasons, and highest during the pre-monsoon and monsoon seasons. The geographical distribution of heavy metals in the Muthupet lagoon revealed that iron was consistently the most prevalent metal across all seasons, followed by magnesium and manganese. Other metals, such as cadmium, chromium, copper, magnesium, manganese, nickel, lead, and zinc, exhibited substantial seasonal fluctuations in this lagoon. For example, during the monsoon season, station 6 had the highest cadmium concentration, whereas station 3 had the highest chromium concentration (Balakrishnan et al., 2015b). Overall, the findings show that Sukhna Lake's water quality is best in the spring, whereas the summer months bring issues owing to increased biological activity and pollution intake. These findings emphasize the importance of implementing proactive, season-specific management strategies—such as regulating runoff, improving aeration, and monitoring pollutant sources during the summer months—to maintain water quality and the lake's ecological integrity throughout the year.

CONCLUSION

This study provides a complete assessment of seasonal fluctuations in physicochemical parameters and heavy metal content in the waters of Sukhna Lake in Chandigarh. The findings show that seasonal dynamics have a significant impact on water quality, with spring being the most favorable time in terms of increased dissolved oxygen levels, reduced biochemical oxygen demand, and relatively constant pH. Summer, on the other hand, was the most stressful season, with decreased dissolved oxygen, increased organic load (BOD), and a move toward acidic pH conditions. These changes are most likely caused by higher temperatures, greater microbial activity, and increased pollution intake from runoff. The investigation also revealed slight but regular changes in heavy metal concentrations. While all measured elements were well under permitted limits, heightened lead and copper concentrations in the spring, as well as higher cadmium levels in the winter and summer, demonstrate the effect of both seasonal runoff and anthropogenic stresses. The statistically significant values of most measures across seasons demonstrate the susceptibility of Sukhna Lake's water quality to environmental changes.  In conclusion, the findings highlight the importance of season-specific monitoring and management measures. Mitigating pollution loads during important months like summer, enhancing stormwater management, and preserving biological balance through conservation initiatives can all have a substantial impact on Sukhna Lake's sustainability and health. This work provides baseline data for future monitoring, planning, and restoration efforts aimed at urban freshwater systems influenced by seasonal and anthropogenic influences.

Declaration of conflict of interest

No potential conflict of interest was reported by the authors.

Data availability statement

Data sharing is not applicable to this study as no datasets were generated or analysed during the current study.

REFERENCES

Badran, M.I., 2001. Dissolved Oxygen, Chlorophyll a and Nutrients: Seasonal Cycles in Waters of the Gulf of Aquaba, Red Sea. Aquat. Ecosyst. Health Manag. 4, 139–150. https://doi.org/10.1080/14634980127711

Balakrishnan, T., Sundaramanickam, A., Shekhar, S., Balasubramanian, T., 2015a. DISTRIBUTION AND SEASONAL VARIATION OF HEAVY METALS IN SEDIMENTS OF MUTHUPET LAGOON, SOUTHEAST COAST OF INDIA. J. Ecol. Eng. 16, 49–60. https://doi.org/10.12911/22998993/2805

Biswas, A., Debnath, P., Roy, S., Bhattacharyya, S., Mitra, S., Chaudhuri, P., 2024. Spatio-temporal variation in water quality due to the anthropogenic impact in Rudrasagar Lake, a Ramsar site in India. Environ. Monit. Assess. 196, 598. https://doi.org/10.1007/s10661-024-12736-6

Chauhan, P.K.S., Paliwal, R.K., 2009. Study of seasonal variation of heavy metal concentrations in bed sediments of Yamuna river. Curr. World Environ. 4, 439–442. https://doi.org/10.12944/CWE.4.2.31

Estrada-Rivera, A., Díaz Fonseca, A., Treviño Mora, S., García Suastegui, W.A., Chávez Bravo, E., Castelán Vega, R., Morán Perales, J.L., Handal-Silva, A., 2022. The Impact of Urbanization on Water Quality: Case Study on the Alto Atoyac Basin in Puebla, Mexico. Sustainability 14, 667. https://doi.org/10.3390/su14020667

Gobler, C.J., Baumann, H., 2016. Hypoxia and acidification in ocean ecosystems: coupled dynamics and effects on marine life. Biol. Lett. 12, 20150976. https://doi.org/10.1098/rsbl.2015.0976

Islam, Md.S., Chakma, P., Kabir, Md.H., Department of Environmental Science and Resource Management, Mawlana Bhashani Science and Technology University, Tangail-1902, Bangladesh. Email: xxxxx@gmail.com, Shammi, R.S., Department of Environmental Science and Resource Management, Mawlana Bhashani Science and Technology University, Tangail-1902, Bangladesh. Email: xxxxxx996@gmail.com, Hoque, M.Md.M., Department of Environmental Science and Resource Management, Mawlana Bhashani Science and Technology University, Tangail-1902, Bangladesh. Email: xxxxxx@gmail.com, Iqubal, K.F., EQMS Consulting Limited, House # 76, Road # 5, Block # F, Banani, Dhaka-1213, Bangladesh.  Email: xxxxxx@gmail.com, 2022. Seasonal Variation of Water Qualities in River, Lake and Waterfall in Rangamati and Khagrachhari Hill Tracts, Bangladesh. Grassroots J. Nat. Resour. 05, 59–79. https://doi.org/10.33002/nr2581.6853.050205

Kratz, T.K., Cook, R.B., Bowser, C.J., Brezonik, P.L., 1987. Winter and Spring pH Depressions in Northern Wisconsin Lakes Caused by Increases in p CO2. Can. J. Fish. Aquat. Sci. 44, 1082–1088. https://doi.org/10.1139/f87-129

Matej-Lukowicz, K., Wojciechowska, E., Nawrot, N., Dzierzbicka-Głowacka, L.A., 2020. Seasonal contributions of nutrients from small urban and agricultural watersheds in northern Poland. PeerJ 8, e8381. https://doi.org/10.7717/peerj.8381

Pochodyła-Ducka, E., Glińska-Lewczuk, K., Jaszczak, A., 2023. Changes in Stormwater Quality and Heavy Metals Content along the Rainfall–Runoff Process in an Urban Catchment. Water 15, 3505. https://doi.org/10.3390/w15193505

Saha, A., Ramya, V.L., Jesna, P.K., Mol, S.S., Panikkar, P., Vijaykumar, M.E., Sarkar, U.K., Das, B.K., 2021. Evaluation of Spatio-temporal Changes in Surface Water Quality and Their Suitability for Designated Uses, Mettur Reservoir, India. Nat. Resour. Res. 30, 1367–1394. https://doi.org/10.1007/s11053-020-09790-5

Schutte, K.H., Elsworth, J.F., 1954. The Significance of Large pH Fluctuations Observed in Some South African Vleis. J. Ecol. 42, 148. https://doi.org/10.2307/2256983

Soulsby, C., Malcolm, I.A., Tetzlaff, D., Youngson, A.F., 2009. Seasonal and inter‐annual variability in hyporheic water quality revealed by continuous monitoring in a salmon spawning stream. River Res. Appl. 25, 1304–1319. https://doi.org/10.1002/rra.1241

Sun, X., Rosado, D., Hörmann, G., Zhang, Z., Loose, L., Nambi, I., Fohrer, N., 2023. Assessment of seasonal and spatial water quality variation in a cascading lake system in Chennai, India. Sci. Total Environ. 858, 159924. https://doi.org/10.1016/j.scitotenv.2022.159924

Valtanen, M., Sillanpää, N., Setälä, H., 2014. The Effects of Urbanization on Runoff Pollutant Concentrations, Loadings and Their Seasonal Patterns Under Cold Climate. Water. Air. Soil Pollut. 225, 1977. https://doi.org/10.1007/s11270-014-1977-y

Zhao, Q., Liu, S., Deng, L., Yang, Z., Dong, S., Wang, C., Zhang, Z., 2012. Spatio-temporal variation of heavy metals in fresh water after dam construction: a case study of the Manwan Reservoir, Lancang River. Environ. Monit. Assess. 184, 4253–4266. https://doi.org/10.1007/s10661-011-2260-y