HEAVY METAL PROFILE OF A SOUTHERN RIVER, KERALA, INDIA

- A QUANTITATIVE ASSESSMENT

ABSTRACT

Heavy metals in riverine ecosystems pose long-term risks to water security and aquatic organisms. A quantitative analysis of seven heavy metals—Cr, Cd, Fe, Pb, Zn, Cu, and Ni—in water and sediment samples at 18 strategic locations of Kerala's Kallada River over a 12-month period (Feb 2018–Jan 2019) in the present study was done. Using atomic absorption spectroscopy and statistically intensive treatment, the research aimed to determine patterns of contamination as well as seasonality. Significantly, the chromium levels in water were above limits in all monsoon stations, including an upper limit of 0.5684 ppm in upstream segment (S6). Copper was above the limits in midstream and downstream areas in postmonsoon, while the highest value of iron in sediment was 140.1 mg/kg at S18. Sediment analysis showed levels of lead (2.315 mg/kg) and chromium (6.14 mg/kg) above normal at certain downstream locations. Pearson correlation indicated strong relationships, i.e., Cu-S and Pb-S (r = 0.86753). Implications include industrial effluents, agricultural runoff (copper fungicides in particular), and municipal solid waste. Although water samples as a whole were in conformity with standards, sediment displayed latent ecological threats. The study concludes on recommending routine monitoring of water quality, strict control of effluent disposal, and implementation of sustainable waste management policy to promote aquatic and human health safety.

Keywords: Chromium, Cadmium, Copper, Heavy metals, Iron, Lead, Nickel, Sediment, Zinc Analysis

1.INTRODUCTION

Water quality studies often include physicochemical water quality checking, trace and toxic metal analysis, microbiological analysis, etc. Non-degradable and persistent nature of heavy metals makes them one of the most widespread and hazardous contaminants of great ecological concern (CWC, 2019). Mining activities, growth of industries, expulsion of industrial effluents, domestic waste discharges and activities that speed up urbanization are the main sources of metal contamination (Mishra et al., 2021;. Rohini et al., 2016). After entering rivers, the majority of heavy metals immediately settle into the sediment, where they are much more concentrated (Alao, 2023; Joseph et al., 2025). The concentration of heavy metals in water is typically low, but their increased concentrations have been found as a result of human activity (Sofia et al.,2016). Metals like nickel enter river water from the leaching of rocks and sediments. High concentrations of nickel and chromium were found in sediments that were downstream of landslides (Joseph, 2025; Geertsema et al., 2019).). Anthropogenic sources of chromium contamination include leather industries, tanning, and chrome plating, while fertilizers and tanning contribute heavy metal copper (Joseph et al., 2024). Less soluble heavy metals either stay suspended as particles or become entrained in the sediments of the bed. However, soluble heavy metals are thought to be extremely hazardous because of their mobility and bioavailability (Baby et al., 2022).The delicate equilibrium of the aquatic systems is known to be disturbed when metal pollutants are present in freshwater (Ambedkar and Muniyan, 2012).The present study aims to quantify the heavy metals in water and sediment of the river to evaluate the status of contamination.

2. STUDY AREA

Kallada River, the longest river in Kollam District of Kerala, was selected for study. Sampling was carried out from February 2018 to January 2019. This tropical freshwater river comprises the Kallada Irrigation Project and Thenmala Ecotourism. Along the upstream, midstream and downstream segments of the river, 18 stations were selected for heavy metal analysis. S1, S3, S5, S7, S9, S11, S13, S15 and S17 represent reference sites and S2, S4, S6, S8, S10, S12, S14, S16 and S18 were test sites. Reference and test sites were chosen based on some observable criteria in the vicinity of the river. They were types of land such as forest, urban, or agricultural land, the density of roads, including off-road paths, the nature of the riparian zone and the rate of physical habitat destruction of the selected area.

Table 1 Coordinate of the sampling sites in the three segments of the Kallada River

| Upstream Sites | Latitude | Longitude | Midstream Sites | Latitude | Longitude | Downstream Sites | Latitude | Longitude |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S1 | 8.5736 | 77.0518 | S7 | 8.998628 | 76.95996 | S13 | 9.081353 | 76.72517 |
| S2 | 8.958282 | 77.06347 | S8 | 9.0054 | 76.5621 | S14 | 9.058490 | 76.71102 |
| S3 | 8.5745 | 77.0217 | S9 | 9.03696 | 76.91734 | S15 | 9.056896 | 76.68894 |
| S4 | 8.5809 | 77.0102 | S10 | 9.04431 | 76.8966 | S16 | 9.026013 | 76.66482 |
| S5 | 8.982719 | 76.9904 | S11 | 9.0455 | 76.4518 | S17 | 9.0134 | 76.3848 |
| S6 | 8.5922 | 76.584 | S12 | 9.0518 | 76.452 | S18 | 9.002742 | 76.62472 |

3. MATERIALS AND METHODS

3.1 Water and Sediment Sampling

For heavy metal analyses, water samples and sediments were collected from all the selected sites along the Kallada River. To remove metal contaminants, the water sampling plastic bottles were washed with 10% HNO3 and rinsed thoroughly with distilled water. Soon after collection, all samples were acidified by adding a few drops of concentrated HNO3 . Petersen Grab Sampler was used to collect sediments and was stored in a polythene cover. Collected sediments were transferred to the laboratory for further processing. Determination of chromium (Cr), cadmium (Cd), iron (Fe), lead (Pb), zinc (Zn), copper (Cu) and nickel (Ni) was carried out by atomic absorption spectroscopy (APHA, 2017).

3.2 Data Analysis

Analysis of variance was done to find out seasonal variation among sites and t-test was done for site-wise variations at 5% level of significance using the software SPSS (SPSS Statistics 21.0). Heavy metal concentration in water and sediment was correlated using Pearson Correlation Analysis in software PAST (ver. 4.03)

4. RESULT AND DISCUSSION

Heavy metal analysis of water and sediment from upstream, midstream and downstream segments of the river is shown in Tables 1 to 10. The highest mean value of zinc in the water sample was 0.1286 ± 0.008 ppm and was observed in site S6 of upstream during postmonsoon and the lowest value was in site S13 of midstream during monsoon season. In sediment, the highest value of zinc was found in site S10 (3.935 ± 0.016 ppm) of midstream and the lowest in site S17 of downstream (0.0186 ± 0.0002 ppm) during postmonsoon and monsoon respectively. However, all the values of zinc in water and sediment were within the permissible limit of national and international drinking standards. Concentration of copper in the water samples of the midstream segment during the postmonsoon season was observed to be above the permissible limit (0.05 ppm). In the downstream segment, the mean value of copper at sites S15 and S16 also exceeded the acceptable limit. But at the same time, it was below detectable levels during monsoon in all sites. Though the mean value of copper in the sediment was within the permissible limit of standards (USEPA, 1995), it reached a concentration of 1.2367± 0.15 ppm (Site S18) in pre-monsoon season. In the water sample, the concentration of iron was observed to be within the permissible limit of the water quality standard (USEPA,1999), and it ranged from 0.0011 ± 0.0001ppm in site S4 during the premonsoon to 0.1544 ± 0.014 ppm in site S3 during the postmonsoon season. In the sediment, the value of iron exceeded the permissible limit of the standard value. Site S18 of the downstream segment showed a maximum value of iron in sediment (140.1 ± 7.14ppm) during pre-monsoon season. Chromium in the water sample ranged from 0.0087 ± 0.003 ppm in site S18 of downstream during postmonsoon to 0.5684 ± 0.029 ppm in site S6 of upstream during monsoon. During monsoon, chromium concentrations in the sediment sample were lowest at site S15 of the downstream (0.254 ± 0.10 ppm) and highest at S6 of upstream (6.14 ± 0.281ppm) during post-monsoon.

As per the t-test result, in water, there was no significant variation in the mean concentration of zinc observed between sites during the monsoon season. In sediments, it showed significant variation between sites such as S1and S2; S9 and S10; S11and S12; S13and S14; and S17and S18 in all the seasons. The concentration of copper in the midstream segment showed no significant variation between sites in all seasons. Iron showed significant variation between sites during pre-monsoon and post-monsoon seasons or both, except for sites such as S5 and S6 of upstream and S9 and S10 of midstream. In the upstream segment, chromium showed significant variation between sites in all seasons except for sites S3 and S4 during monsoon and postmonsoon seasons. In the midstream segment, no significant variation was observed between sites during the monsoon season.

Figure 1 Annual Mean Concentration of Heavy Metals in River Sediment

Figure 2 Annual Mean Concentration of Heavy Metals in River Sediment

Correlation Analysis of Heavy Metals in Water and Sediments

Pearson Correlation Analysis was done to understand the relationship between heavy metals in water and sediment (Table 1). Results showed that a significant positive correlation (p< 0.05) exists among different metals. Zinc in sediment showed a significant correlation with lead (r = 0.61832) in water. Copper in sediment displayed a significant positive correlation with iron (r = 0.51573) and lead (r = 0.86753) in sediment. Iron in sediment shows a high correlation with lead (r = 0.71774) in sediment, whereas chromium in sediment showed a significant correlation with nickel (r = 0.7066) in sediment and chromium (r = 0.5157) in water. Zinc in water showed a positive correlation with chromium (r = 0.5060) and nickel (r = 0.5716) in water.

Table 1 Correlation analysis of Annual Mean Concentration of Heavy Metals in Water and Sediment of Kallada River

|  | Zn-S | Cu-S | Fe-S | Cr-S | Ni-S | Pb-S | Zn-W | Cu-W | Fe-W | Cr-W |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Zn-S | 1 |  |  |  |  |  |  |  |  |  |
| Cu-S | -0.05356 | 1 |  |  |  |  |  |  |  |  |
| Fe-S | 0.235981 | 0.515738* | 1 |  |  |  |  |  |  |  |
| Cr-S | 0.25158 | 0.209816 | -0.01084 | 1 |  |  |  |  |  |  |
| Ni-S | 0.262823 | 0.19197 | -0.38006 | 0.70661* | 1 |  |  |  |  |  |
| Pb-S | 0.150709 | 0.867531* | 0.717745* | 0.093859 | -0.005 | 1 |  |  |  |  |
| Zn-W | -0.06701 | -0.02456 | -0.04585 | 0.367394 | 0.351756 | -0.18593 | 1 |  |  |  |
| Cu-W | 0.056458 | -0.07076 | 0.144884 | -0.42339 | -0.26492 | 0.221718 | -0.125 | 1 |  |  |
| Fe-W | -0.30436 | 0.280878 | 0.255984 | -0.21531 | -0.26068 | 0.250802 | 0.082278 | 0.063907 | 1 |  |
| Cr-W | 0.202435 | -0.40355 | -0.23387 | 0.51571* | 0.344488 | -0.41977 | 0.506094* | -0.41861 | 0.060184 | 1 |

*Significance level p<0.05

Table 2 Mean concentration of Zinc (ppm) in Kallada River water in three segments (Mean ± SD)

| Upstream | Pre monsoon | Monsoon | Post monsoon | Mid Stream | Pre monsoon | Monsoon | Post monsoon | Down stream | Pre monsoon | Monsoon | Post monsoon |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S1* | 0.0353 ± 0.006 | 0.00194 ± 0.0001 | 0.0342 ± 0.008* | S7* | 0.0372 ± 0.007 | 0.00202 ± 0.0001 | 0.0564 ± 0.005* | S13* | 0.0337 ± 0.005 | 0.00139 ± 0.0004 | 0.0884 ± 0.002 |
| S2* | 0.0369 ± 0.002 | 0.00175 ± 0.0004 | 0.0670 ± 0.020* | S8* | 0.0355 ± 0.001 | 0.00185 ± 0.0004 | 0.0789 *± 0.009 | S14* | 0.0369 ± 0.001 | 0.00166 ± 0.0005 | 0.0856 ± 0.003 |
| S3* | 0.0311 ± 0.001* | 0.00195 ± 0.0002 | 0.0327 ± 0.008* | S9* | 0.0327 ± 0.002 | 0.00208 ± 0.0002 | 0.0547 ± 0.001 | S15* | 0.0316 ± 0.002 | 0.00162 ± 0.0002 | 0.0753 ± 0.006 |
| S4* | 0.0409 ± 0.005* | 0.00181 ± 0.0006 | 0.0671 ± 0.002* | S10* | 0.0312 ± 0.002 | 0.00185 ± 0.0006 | 0.0672 ± 0.004 | S16* | 0.0335 ± 0.001 | 0.00177 ± 0.0003 | 0.0763 ± 0.005 |
| S5* | 0.0308 ± 0.009* | 0.00191 ± 0.0006 | 0.0674 ± 0.002* | S11* | 0.0355 ± 0.003 | 0.00198 ± 0.0006 | 0.04208 ± 0.002* | S17* | 0.0285± 0.009 | 0.00221± 0.0002 | 0.0575± 0.002 |
| S6* | 0.0526 ± 0.003* | 0.00202 ± 0.0002 | 0.1286 ± 0.008* | S12* | 0.0376 ± 0.004 | 0.00216 ± 0.0002 | 0.0565 ± 0.003* | S18* | 0.0307 ± 0.008 | 0.00236 ± 0.0001 | 0.0584 ± 0.001 |

Table 3 Mean concentration of Copper (ppm) in Kallada River water in three segments (Mean ± SD)

| Upstream | Pre monsoon | Monsoon | Post monsoon | Mid Stream | Pre monsoon | Monsoon | Post monsoon | Down stream | Pre monsoon | Monsoon | Post monsoon |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S1* | 0.0023 ± 0.0008 * | BDL | 0.0147 ± 0.002 * | S7* | 0.0036 ± 0.0005 | BDL | 0.0604 ± 0.003 | S13* | 0.0035 ± 0.0005 | BDL | 0.0152± 0.001 |
| S2* | 0.0034 ± 0.0003 * | BDL | 0.0201 ± 0.001 * | S8* | 0.0033 ± 0.0002 | BDL | 0.0541 ± 0.008 | S14* | 0.0033 ± 0.0003 | BDL | 0.0178± 0.003 |
| S3* | 0.0033 ± 0.0002 | BDL | 0.0215 ± 0.005 | S9* | 0.0035 ± 0.0002 | BDL | 0.0546 ± 0.005 | S15* | 0.0035 ± 0.0003 | BDL | 0.0554 ± 0.005 |
| S4* | 0.0036± 0.0007 | BDL | 0.0202 ± 0.004 | S10* | 0.0034± 0.0003 | BDL | 0.0483 ± 0.002 | S16* | 0.0036 ± 0.0007 | BDL | 0.0645 ± 0.003 |
| S5* | 0.0021 ± 0.0001* | BDL | 0.0163 ± 0.003* | S11* | 0.0034 ± 0.0001 | BDL | 0.0538 ± 0.002 | S17* | 0.00205 ± 0.0001* | BDL | 0.0375 ± 0.002 |
| S6* | 0.0035 ± 0.0004* | BDL | 0.0253 ± 0.007* | S12* | 0.0033 ± 0.0003 | BDL | 0.0532 ± 0.001 | S18* | 0.0028 ± 0.0004* | BDL | 0.0401 ± 0.004 |

*singnificance level  p< 0.05 		BDL-Below Detectable Level

Table 4 Mean concentration of Iron (ppm) in Kallada River water in three segments (Mean ± SD)

| Upstream | Pre monsoon | Monsoon | Post monsoon | Mid Stream | Pre monsoon | Monsoon | Post monsoon | Down Stream | Pre monsoon | Monsoon | Post monsoon |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S1* | 0.0043 ± 0.0005* | BDL | 0.0324 ± 0.012* | S7* | 0.0063 ± 0.005* | BDL | 0.0639 ± 0.002 | S13* | 0.0068 ± 0.0008* | BDL | 0.0836± 0.005* |
| S2* | 0.0099 ± 0.0002* | BDL | 0.0732 ± 0.036* | S8* | 0.0088 ± 0.002* | BDL | 0.0532 ± 0.006 | S14* | 0.0236 ± 0.0009* | BDL | 0.1247± 0.008* |
| S3* | 0.0016 ± 0.0005* | BDL | 0.1544 ± 0.014* | S9* | 0.0013 ± 0.005 | BDL | 0.0514 ± 0.004 | S15* | 0.0087 ± 0.0005* | BDL | 0.0996± 0.004* |
| S4* | 0.0011 ± 0.0001* | BDL | 0.0536 ± 0.028* | S10* | 0.0015 ± 0.001 | BDL | 0.0536 ± 0.008 | S16* | 0.0426 ± 0.0007* | BDL | 0.1452 ± 0.003* |
| S5* | 0.0053 ± 0.0001 | BDL | 0.0764 ± 0.022 | S11* | 0.0028 ± 0.002* | BDL | 0.0764 ± 0.002 | S17* | 0.0109 ± 0.0004* | BDL | 0.1058± 0.004 |
| S6* | 0.0064 ± 0.0008 | BDL | 0.0839 ± 0.013 | S12* | 0.0036 ± 0.008* | BDL | 0.0839 ± 0.003 | S18* | 0.0078 ± 0.0002 * | BDL | 0.0983 ± 0.006 |

Table 5 Mean concentration of Copper (ppm) in Kallada River water in three segments (Mean ± SD

| Upstream | Pre monsoon | Monsoon | Post monsoon | Mid Stream | Pre monsoon | Monsoon | Post monsoon | Down Stream | Pre monsoon | Monsoon | Post monsoon |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S1* | 0.0156 ± 0.008* | 0.1232 ± 0.021* | 0.0887 ± 0.016* | S7* | 0.0152 ± 0.005* | 0.3348 ± 0.023 | 0.1058 ± 0.006* | S13* | 0.0637± 0.005 | 0.5428 ± 0.020 | 0.3639± 0.008 |
| S2* | 0.0854 ± 0.006* | 0.5432 ± 0.045* | 0.3154 ± 0.012* | S8* | 0.0758 ± 0.002* | 0.3432 ± 0.037 | 0.1288 ± 0.002* | S14* | 0.0652± 0.008 | 0.5648 ± 0.019 | 0.3893± 0.002 |
| S3* | 0.0695 ± 0.005* | 0.4762 ± 0.033 | 0.1874 ± 0.040 | S9* | 0.0869 ± 0.003* | 0.3862 ± 0.035 | 0.1647 ± 0.004* | S15* | 0.0269 ± 0.002 | 0.2659 ± 0.017 | 0.1557± 0.005 |
| S4* | 0.0886 ± 0.002* | 0.5532 ± 0.017 | 0.2226 ± 0.053 | S10* | 0.0796 ± 0.002* | 0.4204 ± 0.028 | 0.2062 ± 0.005* | S16* | 0.0277 ± 0.001 | 0.2847 ± 0.021 | 0.1673± 0.007 |
| S5* | 0.0434 ± 0.005* | 0.5132 ± 0.080* | 0.1434 ± 0.011* | S11* | 0.0487 ± 0.005* | 0.3524 ± 0.021 | 0.1629 ± 0.002* | S17* | 0.0236 ± 0.007 | 0.2187 ± 0.015 | 0.0163 ± 0.002* |
| S6* | 0.0679 ± 0.001* | 0.5684 ± 0.029* | 0.2483 ± 0.023* | S12* | 0.0584 ± 0.000* | 0.4357 ± 0.018 | 0.2693 ± 0.003* | S18* | 0.0241 ± 0.009 | 0.2369 ± 0.024 | 0.0087 ± 0.003* |

*singnificance level  p< 0.05 	BDL-Below Detectable Level

Table 6 Mean concentration of Zinc (mg/kg) in Kallada River sediment in three segments (Mean ± SD

| Upstream | Pre monsoon | Monsoon | Post monsoon | Mid Stream | Pre monsoon | Monsoon | Post Monsoon | Down Stream | Pre monsoon | Monsoon | Post monsoon |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S1* | 0.1267 ± 0.005* | 0.0592 ± 0.0002* | 0.1543 ± 0.006* | S7* | 0.1485 ± 0.006* | 0.0256 ± 0.0007* | 0.1864 ± 0.009 | S13* | 0.1947 ± 0.002* | 0.0839 ± 0.0008* | 0.2543 ± 0.007* |
| S2* | 3.158 ± 0.008* | 1.137 ± 0.006* | 3.235 ± 0.012* | S8* | 0.1743 ± 0.008* | 0.0327± 0.0004* | 0.1875 ± 0.005 | S14* | 0.3295 ± 0.004* | 0.1038 ± 0.009* | 0.4069 ± 0.003* |
| S3* | 0.0887 ± 0.003 | 0.0337 ± 0.005 | 0.1054 ± 0.011* | S9* | 1.847 ± 0.015* | 0.764 ± 0.003* | 3.177 ± 0.012* | S15* | 0.0781± 0.0008* | 0.0742 ± 0.0005* | 0.1587 ± 0.008 |
| S4* | 0.0964 ± 0.001 | 0.0308 ± 0.004 | 0.1211 ± 0.003* | S10* | 2.956 ± 0.002* | 0.975 ± 0.007* | *3.935 ± 0.016 | S16* | 0.1054± 0.009* | 0.0769 ± 0.0003* | 0.1671± 0.006 |
| S5* | 0.1085 ± 0.011* | 0.0873 ± 0.008* | 0.1532 ± 0.006 | S11* | 0.0876 ± 0.011* | 0.0268 ± 0.008* | 0.1216 ± 0.003* | S17* | 0.0782 ± 0.0005* | 0.0186 ± 0.0002* | 0.0564 ± 0.004* |
| S6* | 0.1398 ± 0.005* | 0.1021 ± 0.006* | 0.1764 ± 0.002 | S12* | 0.1121 ± 0.005* | 0.0527 ± 0.009* | 0.1545± 0.006* | S18* | 0.867± 0.004* | 0.464 ± 0.002* | 1.112 ± 0.012* |

Table 7 Mean concentration of Copper (mg/kg) in Kallada River sediment in three segments (Mean ± SD)

| Upstream | Pre monsoon | Monsoon | Post monsoon | Mid Stream | Pre monsoon | Monsoon | Post monsoon | Down stream | Pre monsoon | Monsoon | Post monsoon |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S1* | 0.1271± 0.008* | BDL | 0.1062 ± 0.018* | S7* | 0.1345 ± 0.009* | BDL | 0.1162 ± 0.216* | S13* | 0.1341± 0.21* | BDL | 0.0546± 0.12* |
| S2* | 0.3194 ± 0.008* | BDL | 0.2536 ± 0.023* | S8* | 0.1896 ± 0.015* | BDL | 0.1694 ± 0.121* | S14* | 0.1986 ± 0.20* | BDL | 0.1467 ± 0.13* |
| S3* | 0.3681 ± 0.006 | BDL | 0.2781 ± 0.043 | S9* | 0.1476 ± 0.022 | BDL | 0.1281 ± 0.039 | S15* | 0.2435 ± 0.23* | BDL | 0.1538 ± 0.15* |
| S4* | 0.3921± 0.005 | BDL | 0.3252 ± 0.027 | S10* | 0.1564± 0.008 | BDL | 0.1217 ± 0.209 | S16* | 0.3319 ± 0.17* | BDL | 0.2887± 0.14* |
| S5* | 0.2694 ± 0.011* | BDL | 0.2163 ± 0.022* | S11* | 0.1485 ± 0.013 | BDL | 0.1393 ± 0.128 | S17* | 0.8782 ± 0.16* | 0.1437 ± 0.005 | 0.5432 ± 0.17* |
| S6* | 0.4427 ± 0.008* | BDL | 0.3461 ± 0.021* | S12* | 0.1793 ± 0.006 | BDL | 0.1489 ± 0.082 | S18* | 1.2367 ± 0.15* | 0.1688 ± 0.007 | 0.6643 ± 0.19* |

*singnificance level  p< 0.05 	BDL-Below Detectable Level

Table 8 Mean concentration of Iron (mg/kg) in Kallada River sediment in three segments (Mean ± SD)

| Upstream | Pre monsoon | Monsoon | Post monsoon | Mid Stream | Pre monsoon | Monsoon | Post Monsoon | Down Stream | Pre monsoon | Monsoon | Post monsoon |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S1* | 30.51 ± 2.23 | 16.26 ± 1.15 | 30.78 ± 2.37* | S7* | 28.78 ± 2.34* | 17.82 ± 1.57* | 37.18 ± 1.39* | S13* | 129.2 ± 6.19* | 54.61± 3.20* | 73.65 ± 4.13 |
| S2* | 32.45 ± 2.11 | 18.37 ± 1.64 | 37.93 ± 1.35* | S8* | 40.37 ± 2.67* | 23.26 ± 1.34* | 44.91 ± 3.28* | S14* | 90.16 ± 4.21* | 44.12 ± 2.24* | 66.46 ± 2.17 |
| S3* | 37.22 ± 1.56* | 18.66 ± 0.56 | 28.66 ± 2.34 | S9* | 54.12 ± 1.79* | 26.32 ± 1.15* | 83.28 ± 2.57* | S15* | 59.74 ± 3.22* | 32.27 ± 1.20 | 47.35 ± 3.18* |
| S4* | 26.34 ± 1.25* | 15.27 ± 1.17 | 27.23 ± 1.38 | S10* | 84.52 ± 2.37* | 56.48 ± 1.29* | 109.5 ± 3.21* | S16* | 81.87 ± 4.19* | 38.64 ± 1.32 | 65.33 ± 4.21* |
| S5* | 28.66 ± 1.43 | 16.37 ± 1.54 | 32.25 ± 1.28 | S11* | 48.14 ± 2.28* | 24.45 ± 1.43* | 63.37 ± 2.24* | S17* | 110.8 ± 6.18* | 56.71 ± 4.16* | 89.63 ± 4.89 |
| S6* | 28.32 ± 1.09 | 16.19 ± 1.17 | 28.68 ± 2.37 | S12* | 30.15 ± 2.16* | 17.58 ± 1.30* | 32.75 ± 2.09* | S18* | 140.1 ± 7.14* | 76.43 ± 5.15* | 122.56 ± 6.18 |

Table 9 Mean concentration of Chromium (mg/kg) in Kallada River sediment in three segments (Mean ± SD)

| Upstream | Pre monsoon | Monsoon | Post monsoon | Mid Stream | Pre monsoon | Monsoon | Post monsoon | Downstream | Pre monsoon | Monsoon | Post monsoon |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S1* | 2.21 ± 0.241* | 0.415 ± 0.153* | 2.51 ± 0.357* | S7* | 2.23 ± 0.208 | 0.541 ± 0.121 | 3.18 ± 0.245* | S13* | 3.36 ± 0.12 | 0.576 ± 0.15 | 3.517 ± 0.13 |
| S2* | 3.56 ± 0.445* | 0.710 ± 0.184* | 4.81 ± 0.448* | S8* | 2.67 ± 0.152 | 0.667 ± 0.118 | 4.54 ± 0.256* | S14* | 3.76 ± 0.15 | 0.668± 0.11 | 4.01 ± 0.11 |
| S3* | 3.72 ± 0.326 | 0.562 ± 0.162 | 4.11 ± 0.355 | S9* | 2.91 ± 0.142* | 0.514 ± 0.124* | 3.36 ± 0.236* | S15* | 1.472 ± 0.16 | 0.254± 0.10* | 1.734 ± 0.17 |
| S4* | 3.92 ± 0.257 | 0.671± 0.356 | 5.13 ± 0.459 | S10* | 3.562 ± 0.235* | 0.682 ± 0.127* | 5.31 ± 0.219* | S16* | 1.681 ± 0.19 | 0.342± 0.14* | 1.921 ± 0.19 |
| S5* | 2.53 ± 0.263* | 0.662 ± 0.225 | 4.78 ± 0.268* | S11* | 2.516 ± 0.271* | 0.719± 0.131 | 5.91 ± 0.281* | S17* | 3.412 ± 0.22 | 0.469 ± 0.17 | 3.64 ± 0.15 |
| S6* | 4.87 ± 0.214* | 0.665 ± 0.238 | 6.14 ± 0.281* | S12* | 3.012 ± 0.223* | 0.796 ± 0.119 | 2.38 ± 0.231* | S18* | 3.92 ± 0.19 | 0.482 ± 0.15 | 4.012 ± 0.18 |

*singnificance level  p< 0.05 		BDL-Below Detectable Level

Table 10 Mean concentration of Nickel (mg/kg) in Kallada River sediment in three segments (Mean ± SD)

| Upstream | Pre monsoon | Monsoon | Post monsoon | Mid Stream | Pre monsoon | Monsoon | Post monsoon | Downstream | Pre monsoon | Monsoon | Post monsoon |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S1* | 0.0721 ± 0.014* | BDL | 0.0521 ± 0.014* | S7* | 0.2658 ± 0.003* | BDL | 0.1477 ± 0.002* | S13* | 0.0307 ± 0.0002 | BDL | 0.0613 ± 0.0007* |
| S2* | 0.4831 ± 0.012* | BDL | 0.3571 ± 0.015* | S8* | 0.3698 ± 0.006* | BDL | 0.2253 ± 0.005* | S14* | 0.0316± 0.0001 | BDL | 0.0711 ± 0.0008* |
| S3* | 0.3574 ± 0.024* | BDL | 0.3274 ± 0.027 | S9* | 0.3202 ± 0.008 | BDL | 0.2098 ± 0.011 | S15* | 0.0336 ± 0.0001 | BDL | 0.0771 ± 0.0009 |
| S4* | 0.4688 ± 0.021* | BDL | 0.3523 ± 0.016 | S10* | 0.3727 ± 0.007 | BDL | 0.1781 ± 0.008 | S16* | 0.0363 ± 0.0006 | BDL | 0.0897 ± 0.0003 |
| S5* | 0.4863 ± 0.018 | BDL | 0.3701 ± 0.017* | S11* | 0.1851 ± 0.006 | BDL | 0.1241 ± 0.006 | S17* | 0.0412 ± 0.0002* | BDL | 0.0722 ± 0.0007* |
| S6* | 0.6298 ± 0.015 | BDL | 0.5698 ± 0.024* | S12* | 0.1812 ± 0.004 | BDL | 0.1486 ± 0.012 | S18* | 0.2541 ± 0.003* | BDL | 0.3996 ± 0.013* |

Table 11 Mean concentration of Lead (mg/kg) in Kallada River sediment in three segments (Mean ± SD)

| Upstream | Pre monsoon | Monsoon | Post monsoon | Mid Stream | Pre monsoon | Monsoon | Post monsoon | Downstream | Pre monsoon | Monsoon | Post monsoon |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S1* | 0.2156 ± 0.015* | BDL | 0.1824 ± 0.021* | S7* | 0.6759 ± 0.117* | BDL | 0.5166 ± 0.124* | S13* | 0.5173 ± 0.046 | BDL | 0.5815 ± 0.018 |
| S2* | 0.8826 ± 0.019* | BDL | 0.8236± 0.015* | S8* | 0.8355± 0.128* | BDL | 0.5275 ± 0.137* | S14* | 0.5725 ± 0.028 | BDL | 0.6723 ± 0.011 |
| S3* | 0.6738± 0.012 | BDL | 0.5232 ± 0.003* | S9* | 0.8762 ± 0.126* | BDL | 0.5573 ± 0.118* | S15* | 0.5502 ± 0.015 | BDL | 0.6227 ± 0.026 |
| S4* | 0.6267 ± 0.026 | BDL | 0.5234 ± 0.003* | S10* | 0.7708 ± 0.134* | BDL | 0.6348 ± 0.129* | S16* | 0.8315 ± 0.085 | BDL | 0.9214 ± 0.024 |
| S5* | 0.5243 ± 0.022 | BDL | 0.5225 ± 0.008* | S11* | 0.8812 ± 0.215* | BDL | 0.8341 ± 0.204 | S17* | 1.242 ± 0.136* | BDL | 1.568 ± 0.018* |
| S6* | 0.8346 ± 0.018 | BDL | 0.0324± 0.005* | S12* | 0.6643± 0.176* | BDL | 0.5721 ± 0.227 | S18* | 2.143 ± 0.242* | BDL | 2.315 ± 0.038* |

*singnificance level  p< 0.05 	BDL-Below Detectable Level

Discussion

The concentration of heavy metals varied significantly in water and sediments of all segments of river. The concentration of zinc was high in water samples from upstream segment and high in sediment from the midstream segment. Though zinc is found naturally in Earth’s crust its high occurrence in upstream segment might be due to heavy rainfall and consequent leaching.  It is also widely used as an anti-corrosive agent to prevent corrosion of metal but when it is exposed to hydrogen and oxygen it make various zinc compounds (Wunaand Okieimen, 2021;Rajappa et al., 2008).Copper concentrations in water samples were found below detectable levels during monsoon season. More than 60% of the samples in the midstream segments and 10% of the samples in the downstream segments showed copper concentrations above the acceptable limit during the postmonsoon season. Copper concentrations in river water become higher when it is used as agrochemical in combinations with sulphate as copper sulphate and chlorine as copper oxychloride (Jio et al., 2022; George et al., 2017, Lalfakawma et al., 2014). Rubber cultivation was found in the study area mainly in upper and middle stretches of river. For the control of fungus, copper fungicides are frequently employed in rubber plantations (Mathew and Radhika, 2022; Prasannakumari and Jessy, 2017). In the current study, upstream stretches of river had substantial concentrations of chromium in sediment and water along with nickel in sediment, which might be due to the frequent landslides in the catchment area of river’s origin because landslides typically contain high levels of nickel and chromium (Geertsema et al., 2019). However, the presence of these metals in lower stretches may be due to anthropogenic activities such as disposal of municipal garbage, laundry detergents, paints, leather tanning, electroplating etc. (Yahiya et al., 2023; Banerjee et al.,2016). The study of Rahman and Hassan (2020) also revealed the significant contamination in water and sediments due to industrial discharge.

Lead concentration also exceeded the acceptable limit and it was observed as high in middle segment, particularly at site S8, where plumbing and car wash operations were prevalent. Lead paints and automobile batteries are two potential sources of lead for entry in to water bodies (Chakraborty and Bhattacharya, 2021; Abagale et al., 2013). In the water sample, iron was detected during the premonsoon and postmonsoon seasons. In the present result of sediment analysis, the value of iron exceeded the permissible limit of 30 ppm (Alao, 2025; Alao et al., 2024; USEPA,1995). Weathering of rocks generates iron oxides in great extent and at the same time domestic effluents also contribute to the increased concentration of iron in sediment and water (Nair et al., 2010; Nair and Kumar, 2019).

Pearson Correlation Matrixunveils a delightful synergy, revealing that zinc in sediment showed positive correlation with lead found in water. This notable connection may stem from the geological tapestry that nurtures the coexistence of lead and zinc. Due to their akin chemical characteristics and their affinity for sulphur, zinc and lead compounds often appear hand in hand (Nayak et al., 2022). The surface runoff cascading from agricultural landscapes may serve as a pivotal reservoir for these metals (Klake et al., 2012). In this exploration, chromium and nickel also showed positive correlations, aligning perfectly with earlier investigations into heavy metals (Luo et al., 2021). The significant interrelations between specific heavy metal concentrations in both water and sediment could arise from the metals' mutual reliance, pollution emissions from identical sources, contamination at comparable rates, or a blend of these influences (Day et al., 2021;Ali et al., 2016).

5. Conclusion

The present study on heavy metal analysis of Kallada River water indicates significant insights into the contamination status and potential ecological risks associated with anthropogenic activities. Elevated concentrations of certain heavy metals such as Chromium and Copper at specific sampling points suggest inputs from industrial effluents, domestic sewage, agricultural runoff, and urban drainage systems. However,the concentrations of all the metals, except iron, in the sediments were within the acceptable limit. The spatial variation in metal concentration reflects the influence of local land use and point or non-point pollution sources. Regular monitoring and the implementation of pollution control measures are crucial to safeguard the river's ecological integrity and ensure safe water quality for dependent communities. Furthermore, there is a pressing need for strict regulation of effluent discharges and promotion of sustainable waste management practices in the catchment area.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

References

- Central Water Commission (CWC). (2019). Status of trace and toxic metals in Indian rivers. Ministry of Jal Shakti, Department of Water Resources, River Development, and Ganga Rejuvenation.

- Mishra, S., Kumar, A. and Shukla, P. (2021). Estimation of heavy metal contamination in the Hindon River, India: An environmetric approach. Applied Water Science, 11(1), 1–9. https://doi.org/10.1007/s13201-020-01342-z

- Rohini, B. R., Jayan, A. and BR, P. K. (2018). Analysis of heavy metal pollution on ParvathyPuthanar, an artificial river canal in South Kerala. International Journal of Research and Analytical Reviews, 5, 310–312.

- Geertsema, M., Highland, L.andVaugeois, L. (2009). Environmental impact of landslides. In K. Sassa & P. Canuti (Eds.), Landslides – Disaster risk reduction (pp. 589–607). Springer. https://doi.org/10.1007/978-3-540-69970-5_30

- Alao, J. O. (2023). Impacts of open dumpsite leachates on soil and groundwater quality. Groundwater for Sustainable Development 20, 100877: https://doi.org/10.1016/j.gsd.2022.100877

- Alao  (2025). Assessing the community knowledge on waste management practices, drinking water source systems, and the possible implications on public health systems. Cleaner Waste Systems, 11, 100295. https://doi.org/10.1016/j.clwas.2025.10029

- Sofia, S., and Merlee Teresa, M. V. (2016). Assessment of heavy metals in water and sediments by ICP-AES in coastal area of Chirackal, Ernakulam District, Kerala. International Journal of Current Research, 8(9), 37635–37640.

- Jospeh. O. (2025). The Evolving Roles of Geophysics in Environmental Assessment, Monitoring, and Management of Landfill Leachate Contaminant Plumes: An Overview. Case Studies in Chemical and Environmental Engineering, 101124. https://doi.org/10.1016/j.cscee.2025.101124

- Joseph. O,, Bello, A., Lawal, H. and  Abdullahi, D. (2024). Assessment of groundwater challenge and the sustainable management strategies. Results in Earth Sciences, 2, 100049. https://doi.org/10.1016/j.rines.2024.100049

- Baby, R., Hussein, M. Z., Abdullah, A. H. and Zainal, Z. (2022). Nanomaterials for the treatment of heavy metal contaminated water. Polymers, 14(3), 583. https://doi.org/10.3390/polym14030583

- Ambedkar, G. and Muniyan, M. (2012). Analysis of heavy metals in water, sediments and selected freshwater fish collected from Gadilam River, Tamil Nadu, India. International Journal of Toxicological and Applied Pharmacological, 2(2), 25–30.

- American Public Health Association(APHA). (2017). Standard methods for the examination of water and wastewater (23rd ed.). American Public Health Association.

- Alao, J.O.(2024). The Factors Influencing the Landfill Leachate Plume Contaminants in Soils, Surface and Groundwater and Associated Health Risks: A Geophysical and Geochemical View. Public Health and Environment. 1 (1): 20-43: https://doi.org/10.70737/7ejde2

- Alao, J. O. and Abubakar, F. (2025). Groundwater exploration, management strategies and sustainability: Geophysical approaches. Geosystems and Geoenvironment, 100395. https://doi.org/10.1016/j.geogeo.2025.100395

- United States Environmental Protection Agency(USEPA).(1995). QA/QC guidance for sampling and analysis of sediments, water, and tissues for dredged material evaluations: Chemical evaluations. U.S. Environmental Protection Agency.

- United States Environmental Protection Agency(USEPA). (1999). Alternative disinfectants and oxidants guidance manual (EPA 815-R-99-014). U.S. Environmental Protection Agency, Office of Water.

- Wuana, R. A. and Okieimen, F. E. (2011). Heavy metals in contaminated soils: A review of sources, chemistry, risks and best available strategies for remediation. International Scholarly Research Notices, 2011, Article ID 402647. https://doi.org/10.5402/2011/402647

- Rajappa, S. K., Venkatesha, T. V. and Praveen, B. M. (2008). Chemical treatment of zinc surface and its corrosion inhibition studies. Bulletin of Materials Science, 31(1), 37–41. https://doi.org/10.1007/s12034-008-0007-z

- Jiao, Y., Zhang, W., Kim, J. Y. R., Deventer, M. J., Vollering, J. and Rhew, R. C. (2022). Application of copper (II)-based chemicals induces CH₃Br and CH₃Cl emissions from soil and seawater. Nature Communications, 13, Article 6263. https://doi.org/10.1038/s41467-022-34041-z

- George, P., Joseph, S. and Moses, S. A. (2017). Heavy metal monitoring and health risk assessment of a tropical mountainous river in Kerala, India. International Journal of River Basin Management, 15(4), 475–484. https://doi.org/10.1080/15715124.2017.1372447

- Lalfakawma, C., Nath, B. C., Bora, L. C., Srivastava, S. E. and Singh, J. P. (2014). Integrated disease management of Zingiber officinale Rosc. rhizome rot. The Bioscan, 9, 265–269.

- Mathew, S. M. and Radhika, R. (2022). Status of heavy metal contamination in major rivers of Kerala, south India – a review. Uttar Pradesh Journal of Zoology, 43(21), 23–31. https://doi.org/10.56557/upjoz/2022/v43i213207

- Prasannakumari, P. and Jessy, M. D. (2017). Effect of increasing levels of soil copper on growth of young rubber (Hevea brasiliensis) plants. Journal of Plantation Crops, 45(3), 173–179.

- Yahiya, M., Lal, S. S. and Latha, C. (2023). Environmental risk of heavy metals in the southern coast of Kerala, India. Journal of Chemical Health Risks, 13(4s).

- Banerjee, S., Kumar, A., Maiti, S. K., and Chowdhury, A. (2016). Seasonal variation in heavy metal contaminations in water and sediments of Jamshedpur stretch of Subarnarekha River, India. Environmental Earth Sciences, 75(3), Article 213. https://doi.org/10.1007/s12665-015-4902-1

- Rahman, M. M. and Hassan, M. M. (2020). The amount of selected heavy metals in water, sediments and fish species from the Rupsha River, Khulna, Bangladesh. Asian Journal of Fisheries and Aquatic Research, 6(2), 1–9. https://doi.org/10.9734/ajfar/2020/v6i230091

- Chakraborty, S. and Bhattacharya, T. (2021). Seasonal variation of heavy metals in river water and sediment in relation to land use in eastern India. Environmental Monitoring and Assessment, 193(6), 371. https://doi.org/10.1007/s10661-021-09176-3

- Abagale, F. K., Sarpong, D. A., Ojediran, J. A., Osei-Agyemang, R., Shaibu, A. G. and Birteeb, P. T. (2013). Heavy metal concentration in wastewater from car washing bays used for agriculture in the Tamale Metropolis, Ghana. International Journal of Current Research, 5(6), 1571–1576.

- Nair, I., Singh, K., Arumugam, M. and Clarson, D. (2010). Monitoring of trace metal pollution in Meenachil River at Kottayam, Kerala (India). E-Journal of Chemistry, 8(1), 257–263. https://doi.org/10.1155/2011/519185

- Nair, V. M. and Kumar, R. B. (2019). Assessment of heavy metal concentration in river sediments along Vamanapuram River Basin, South Kerala, India. Nature Environment and Pollution Technology, 18(2), 593–597.

- Nayak, A., Jena, M. S. and Mandre, N. R. (2022). Beneficiation of lead-zinc ores–A review. Mineral Processing and Extractive Metallurgy Review, 43(5), 564–583. https://doi.org/10.1080/08827508.2021.2013672

- Klake, R. K., Nartey, V. K., Doamekpor, L. K. and Edor, K. A. (2012). Correlation between heavy metals in fish and sediment in Sakumo and Kpeshie Lagoons, Ghana. Journal of Environmental Protection, 3(9), 1070–1076. https://doi.org/10.4236/jep.2012.39125

- Luo, P., Xu, C., Kang, S., Huo, A., Lyu, J., Zhou, M. and Nover, D. (2021). Heavy metals in water and surface sediments of the Fenghe River Basin, China: Assessment and source analysis. Water Science and Technology, 84(10–11), 3072–3090. https://doi.org/10.2166/wst.2021.358

- Dey, S., Das, S. and Hazra, S. (2021). Source apportionment and ecological risk assessment of heavy metals in water and sediment of a tropical estuarine system in eastern India. Environmental Science and Pollution Research, 28(25), 33183–33199. https://doi.org/10.1007/s11356-021-13099-0

- Ali, M. M., Ali, M. L., Islam, M. S., and Rahman, M. Z. (2016). Preliminary assessment of heavy metals in water and sediment of Karnaphuli River, Bangladesh. Environmental Nanotechnology, Monitoring & Management, 5, 27–35. https://doi.org/10.1016/j.enmm.2016.01.002