Multidimensional Evaluation of Underground Water Quality in Rooma, Kanpur ,U.P., India: A Physico-Chemical and Microbiological Approach

### ABSTRACT

This study aims to assess the underground water quality of Rooma, an industrial and semi-urban area in Kanpur, Uttar Pradesh. With the growing urbanization and industrial activities, concerns about groundwater pollution have increased. The research focuses on key physical and chemical parameters, including pH, alkalinity, total hardness, fluoride, iron, and arsenic, to understand the current status of water quality. Biological indicators were also considered to provide a more complete picture of water safety. Samples were collected from various hand pumps, submersibles, and bore wells across the area. Laboratory analysis revealed that while some parameters were within permissible limits, others—especially potassium, magnesium, Total hardness, and arsenic—exceeded safe levels in most of the months recommended by the BIS and WHO. As well as the presence of microbes in terms of Total bacterial count and total Yeast count also indicated the health hazards among human beings. The findings suggest a potential risk to public health and the urgent need for regular monitoring and treatment of groundwater in the region.

Keywords-  Physico-chemical Parameters, Microbiological analysis, Industrial and semi-urban areas, Water quality assessment for drinking, public health risks etc.

### INTRODUCTION

Groundwater is one of the most critical natural resources for sustaining life, agriculture, and industry in India. In urban and semi-urban regions like Rooma in Kanpur district, groundwater is the principal source of drinking and domestic use due to the lack of a continuous piped water supply (Singh et al., 2024; BIS, 2021). However, growing industrialization and unregulated waste disposal have raised serious concerns over groundwater quality in such areas (Gupta & Singh, 2023; Yadav & Yadav, 2025).

Rooma is a rapidly developing locality located on the southern periphery of Kanpur city. Over the past two decades, several small- and medium-scale industries, including leather processing, textile dyeing, plastic manufacturing, electroplating, and chemical units, have emerged in and around this region (Bishnoi & Malik, 2008; Manna et al., 2024). These units often release untreated or partially treated effluents directly into the surrounding environment, increasing the risk of groundwater contamination through percolation (Ganiyu et al., 2018; Chaturwedi et al., 2024). Heavy metal contamination and microbial contamination in groundwater toxicity has been reported which indicating severe public health risks (Gupta et al., 2023; Thakur et al., 2024). Geochemical evaluations highlight arsenic contamination and propose treatment technologies tailored to groundwater conditions (Kanel et al., 2023).

The presence of toxic substances such as fluoride, iron, and arsenic, along with increased hardness, alkalinity, and microbial load, can severely degrade water quality and pose long-term health risks (Adimalla & Venkatayogi, 2018; Mukhopadhyay et al., 2025; Kapoor et al., 2024).

Studies have shown that prolonged consumption of water with high fluoride leads to dental and skeletal fluorosis, while arsenic is linked with skin disorders, cancers, and neurological issues (WHO, 2022). Therefore, assessing the physical, chemical, and biological characteristics of groundwater in areas like Rooma is essential to understand the pollution levels and provide evidence-based recommendations for safe water use (Shaji et al., 2024).

In Kanpur, groundwater vulnerability mapping using the DRASTIC model revealed zones of high susceptibility to pollution (Sahoo et al., 2016). Regulatory standards, such as the National Primary Drinking Water Regulations, provide benchmarks for safe drinking water and act as a guideline for monitoring contamination (Environmental Protection Agency, 2021).

2. METHODOLOGY

This study was conducted in Rooma, a semi-urban and rapidly industrializing locality located in the southern fringe of Kanpur city, Uttar Pradesh, India. Geographically, Rooma lies between latitude 260 21‘ 58“ N and longitude 800.25’ 25“ ‘E, covering an area of approximately 6 to 8 square kilometers. The average elevation is around 125 meters above sea level, and the region falls under the alluvial plains of the upper Ganga basin (Photograph 1).

The area experiences a moderate climate with hot summers and mild winters. Over the past two decades, Rooma has witnessed significant industrial expansion. The locality is home to various small and medium-scale industries, including leather tanning units, textile dyeing and printing factories, electroplating and metal processing industries, rubber, foam, and plastic molding units, and chemical-based manufacturing plants. These industries, often located close to residential areas, discharge waste into the environment, contributing to soil and groundwater contamination. The main source of water for residents is groundwater, accessed through hand pumps, submersibles, and deep bore wells. The groundwater depth varies between 25 and 45 meters, depending on the season and usage patterns (Misra, 2013; Fathi et al., 2021).

Pic 1- Satellite view of the site Rooma

To evaluate the quality of groundwater in this area, water samples were collected randomly from public hand pumps, submersibles, and private bore wells monthly from residential, industrial, and semi-rural zones. The collected samples were stored in pre-cleaned, sterilized polyethylene bottles and labeled accurately for laboratory testing for getting a complete picture on variation in parameters. The analysis focused on key physical, chemical, and biological parameters, which are important indicators of groundwater quality. During post-monsoon season, it reflects a relatively stabilized aquifer condition after natural recharge.

The temperature was measured by a calibrated digital thermometer.  pH was also measured by using a calibrated digital meter to assess the acidity or alkalinity of water. Acceptable limits for pH lie between 6.5 to 8.5 as per BIS (2021) and WHO (2022) guidelines.

The APHA (2012) standard operating procedures will be used to the analysis of some chemical parameters by laboratory procedures, and few parameters were ascertained using analytical testing kits. For several metrics, such as hardness, total alkalinity, free carbon dioxide, and chloride, the titration method was used for analysis. Few are from digital meter as well as Analyzing kit, alike TDS by TDS Meter, Sulphates, Potassium, Magnesium, Nitrate, Iron, Fluoride, Arsenic are also determined respectively.

Microbiological analysis was performed using a BACTASLYDE(BS101) method to detect the Total bacterial count and total Yeast count. All the values obtained were compared with permissible limits set by the Bureau of Indian Standards (IS 10500:2021) and the World Health Organization (2022). The data were analyzed to determine pollutant distribution and identify risk-prone zones.

3. RESULTS AND DISCUSSION

The analysis of groundwater samples from Rooma, Kanpur provides a comprehensive picture of its Physico-chemical and microbial quality. Physico-chemical parameters offer insights into the inorganic pollution load and long-term geochemical processes affecting groundwater. The finding highlights seasonal fluctuations as well as the influence of local industrial and domestic activities on water parameters.

Table 1: Month wise water quality parameters monitoring data (May2024-April 2025)

| Months/ Parameters | May 2024 | June 2024 | July 2024 | August 2024 | September 2024 | October 2024 | November 2024 | December 2024 | January 2025 | February 2025 | march 2025 | April 2025 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pH | 7.2 | 7.4 | 7.3 | 8.4 | 7.9 | 7.8 | 9.5 | 8.3 | 8.5 | 8.36 | 8.03 | 8.5 |
| TDS | 220 | 265 | 250 | 302 | 250 | 170 | 180 | 250 | 460 | 436 | 225 | 250 |
| Temp | 20 | 23 | 25 | 23 | 22 | 20 | 25 | 23 | 20 | 21 | 25 | 25 |
| Total alkalinity | 250 | 350 | 525 | 350 | 250 | 300 | 400 | 400 | 350 | 350 | 350 | 350 |
| Total Hardness | 300 | 350 | 600 | 750 | 650 | 550 | 750 | 500 | 760 | 600 | 450 | 400 |
| Chloride | 60 | 80 | 100 | 80 | 160 | 60 | 160 | 60 | 80 | 100 | 80 | 60 |
| Fluoride | 0.5 | 0.2 | 0.6 | 0.5 | 0.3 | 0.2 | 0.5 | 0.2 | 0.3 | 0.6 | 0.8 | 0.3 |
| Nitrate | 5.0 | 5.0 | 0.5 | 2.5 | 0.5 | 1.5 | 5.0 | 5.0 | 2.5 | 2.5 | 5.0 | 2.5 |
| Iron | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Arsenic | 25 | 25 | 25 | 25 | 25 | 25 | 1.2 | 50 | 25 | 25 | 25 | 25 |
| Potassium | 17 | 15 | 15 | 17 | 18 | 15 | 15 | 17 | 17 | 15 | 18 | 15 |
| Magnesium | 130 | 125 | 117 | 126 | 117 | 125 | 134 | 125 | 117 | 127 | 122 | 125 |
| Sulphate | 27 | 28 | 25 | 20 | 25 | 25 | 21 | 28 | 27 | 25 | 55 | 25 |
| Carbon dioxide | 20 | 10 | 30 | 25 | 35 | 35 | 45 | 35 | 25 | 20 | 10 | 20 |

pH Indicates acidity/alkalinity. Extreme pH alters taste, corrodes pipelines, and changes solubility of toxic metals. pH of water samples analyzed was ranged from 7.2 (May 2024) to 9.5 (November 2024). Common rage of pH is 7.3-8.5 for monthly water samples. Values beyond 8.5 in multiple months indicate alkaline water, possibly due to dissolution of carbonate-rich strata or industrial effluent discharge. Mostly values were within permissible BIS/WHO limits, but exceed only in November (BIS/WHO-6.5–8.5). Reflects total inorganic salts and organic matter. High TDS affects taste, scaling in pipes, and may indicate pollution. Likewise, TDS values vary from 170 (October) to 460(January), with most values between 220 and 302. Mostly values were within permissible BIS limits (500 mg/L). TDS increases post-monsoon suggest leaching of salts and industrial pollutants into the aquifer (Fig.1).

Fig.1- Monthly Fluctuation in TDS and pH

Temperature affects palatability, microbial growth, and solubility of gases/minerals. Warm water favors bacterial contamination. It ranges from 20 to 25 degrees Celsius. Generally, warmer in Monsoon and Spring. There was no significant role of temperature in evaluation of water quality directly. There is no strict limit for this. Total alkalinity shows buffering capacity due to carbonates/bicarbonates. High alkalinity can affect solubility and toxicity of pollutants. It’s values range from 250 to 525, with most values between 250 and 400. Total alkalinity also crossed 500 mg/L (July 2024), supporting the observation of alkaline water chemistry. Alkalinity, although not toxic, can alter the solubility and toxicity of other compounds like ammonia, metals, and arsenic (Singh & Srivastava, 2023), Mostly values were within permissible BIS limits (200-600 mg/L) (Fig.2).

Fig.2 -Monthly Fluctuation in Temperature and Total Alkalinity

Total hardness caused by calcium and magnesium salts. Hard water causes kidney stone risk, poor soap lathering, and scaling in household use (Tyagi & Bansal, 2021). Its values vary from 300 to 760, with most values between 350 and 650. Hardness peaked at 760 mg/L (January 2025), much above the BIS safe limit (300-600 mg/L), indicating high calcium and magnesium content. Chloride indicates mineral content; excess often linked with sewage/industrial discharge. High levels cause salty taste and corrosion. Chloride values range from 60 to 160, with most values between 60 and 100. Its peak values found in Post-monsoon & winter (Sep–Nov). Safe limit,250 mg/L (desirable), 1000 mg/L (permissible)(Fig.3).

Fig.3-Monthly Fluctuation in Total Hardness and Chloride.

Fluoride is essential in trace amounts for maintaining dental health, but excessive intake can lead to adverse health effects, including dental fluorosis and skeletal fluorosis, highlighting the importance of optimal fluoride levels. Fluoride values vary from 0.2 to 0.8, with most values between 0.2 and 0.6. levels were moderate (0.2–0.8 mg/L), mostly within safe ranges, but occasional spikes (March 2025: 0.8 mg/L) Fluoride require monitoring.

Nitrate reached in ground water from fertilizers, sewage, and leaching by many natural and manmade practices. High nitrate causes “blue baby syndrome” (methemoglobinemia) in infants Nitrate values range from 0.5 to 5.0, with most values between 0.5 and 2.5 Nitrate levels, ranging from 0.5 to 5.0 mg/L with common values below 2.5 mg/L and seasonal peaks during monsoon and spring (May, June, November, and March), are considered safe according to BIS/WHO guidelines, as they fall well below the permissible limit of 45 mg/L (Fig.4).

Fig.4- Monthly Fluctuation in Fluoride and Nitrate

Iron was consistently absent (0 mg/L) throughout the study, suggestion-bearing strata or corroded pipelines A toxic metalloid. Long-term exposure linked to cancers, skin lesions, and neurological disorders. Arsenic remained persistently high (25–50 µg/L), with November 2024 peaking at 50 µg/L, which exceeds both BIS (10 µg/L) and WHO (10 µg/L) limits. Arsenic values range from 1.2 to 50 (ppm), with most values at 25. Chronic arsenic exposure is a serious public health hazard associated with cancers, skin lesions, and neurological dysfunction(Fig.5).

Fig.5- Monthly Fluctuation in Iron and Arsenic

A natural mineral. Normally harmless, but very high levels may affect individuals with kidney/cardiac problems. Potassium values vary from 15 to 18 mg/L, with most values between 15 and 17. Slight rise in Sep & January may affect kidney patients if in excess, and according to BIS/WHO guidelines, there's no strict limit making it acceptable.

Elevated concentrations of this essential nutrient can paradoxically contribute to increased water hardness, gastrointestinal disturbances characterized by laxative effects, and the formation of scales, underscoring the complex relationship between nutrient levels and water quality dynamics. It is with a range of 117–134 mg/L and a common range of 125–130 mg/L, exhibits a peak concentration in November (134 mg/L), and according to BIS guidelines, the levels are slightly high, exceeding the desirable limit of 30–100 mg/L but within the permissible limit of 200 mg/L (Fig.6).

Fig.6- Monthly Fluctuation in Potassium and Magnesium.

Sulphate concentrations exceeding permissible limits can lead to a bitter taste in water, promote corrosion of infrastructure, and cause gastrointestinal issues, including laxative effects such as diarrhea, thereby impacting both water quality and public health. Sulphate levels in the water range from 20 to 55 mg/L, typically between 25-28 mg/L, with a peak of 55 mg/L in March, and are considered safe according to BIS standards, as they fall well below the desirable limit of 200 mg/L and the permissible limit of 400 mg/L, posing no risk of bitter taste or laxative effects.) Carbon dioxide is a dissolved gas indicates a potential impact on water chemistry, as high levels can lower pH, increase corrosiveness, and alter aquatic chemistry, thereby affecting the overall water quality and ecosystem balance. its values vary from 10 to 45. mostly range 25 to 35 mg/L, with higher concentrations observed post-monsoon in November, and are considered acceptable as per BIS/WHO guidelines, which do not specify a fixed limit for these parameters (Fig No.7).

Fig 7  -Monthly Fluctuation in Sulphate and Carbon dioxide

Microbial contamination is one of the most critical indicators of drinking water safety. The presence of bacteria (especially coliforms), yeasts, and other microorganisms directly threatens public health. Unlike chemical contaminants that usually cause long-term effects, microbes can cause immediate illnesses such as diarrhea, cholera, dysentery, typhoid, and gastrointestinal infections. WHO recommends zero coliform bacteria in 100 ml of drinking water, as even a small presence can be unsafe for mankind.

Table 2: Microbial contamination level in water samples.

| Month (2024-2025) | Total yeast count/ml | Total Bacterial count /ml | Time |
| --- | --- | --- | --- |
| May2024 | < 102 | < 102 | 24 hours |
| June2024 | < 102 | < 102 | 24 hours |
| July2024 | < 102 | 102 | 24 hours |
| August2024 | <102 | 103 | 24 hours |
| September2024 | < 102 | <102 | 24 hours |
| October2024 | < 102 | <102 | 24 hours |
| Novmber2024 | <102 | <102 | 24 hours |
| December2024 | <102 | <102 | 24 hours |
| January2025 | <102 | <102 | 24 hours |
| February2025 | < 102 | <102 | 24 hours |
| March2025 | <102 | < 102 | 24 hours |
| April2025 | < 102 | <102 | 24 hours |

Microbial contamination levels in water samples over a year, from (May2024 to April 2025) showed in terms of total yeast count and total Bacterial count. The total yeast count per milliliter remains consistently below <102 throughout the year, Yeasts are generally non-pathogenic, but their persistent presence in groundwater may point towards Organic matter contamination, possibly from decomposing waste or industrial sludge. Biofilm development within hand-pump infrastructure due to irregular cleaning. Persistent low-level yeast detection necessitates regular disinfection of water extraction points and infrastructure to prevent biofilm-associated infections.

Likewise, total bacterial count per milliliter is also generally below <102, except for July and August 2024, where it reaches <102 and <103, respectively time measurement taken within 24 hours, indicating a standardized testing protocol. This fluctuation may be attributed to monsoonal runoff and leaching of surface contaminants into shallow aquifers. Sometimes, overflow or seepage from improperly constructed septic tanks, common in semi-urban zones. Increased anthropogenic activity and poor waste management during wet seasons also responsible for contamination. According to WHO (2022) guidelines, zero coliform or pathogenic bacterial presence is desirable in drinking water. Although the bacterial levels were generally low, the observed seasonal peaks reflect a moderate microbial threat, especially for immunocompromised individuals.

Generally, post-monsoon months (August–October 2024) showed increased microbial counts, TDS, and alkalinity, suggesting percolation from industrial and surface runoff sources and Winter months recorded peak values of hardness and arsenic, which may be linked to low groundwater recharge and increased concentration of dissolved salts.

CONCLUSION

The findings of this study reveal a complex and concerning picture of groundwater quality in the Rooma area of Kanpur. While some Physico-chemical parameters, such as pH, TDS, and chloride, remained within acceptable limits, several critical indicators—particularly total hardness, alkalinity, and arsenic levels—consistently exceeded safe thresholds. This indicates a growing chemical burden in the local aquifer, likely driven by unregulated industrial activity and inadequate environmental safeguards.

What makes the situation more alarming is the biological assessment, which, although showing relatively low levels of microbial contamination, exposed seasonal spikes in bacterial load—especially during and after the monsoon season. Even these low or moderate microbial presences are unacceptable in drinking water, especially for vulnerable populations like children and the elderly. Such contamination hints at leakages from sanitation systems, surface runoff, or poor infrastructure hygiene, all of which reflect systemic neglect.

The recurring presence of arsenic in elevated concentrations, along with episodic increases in fluoride, poses a long-term public health threat. These toxic elements are linked to irreversible conditions, including skin disorders, bone deformities, and various cancers. If left unchecked, continued reliance on this water source without treatment could silently fuel chronic health crises in the region.

While the groundwater in Rooma remains a vital lifeline for the local community, its quality is under serious stress from both industrial pollutants and microbial threats. Microbial safety is moderate but unstable, warranting seasonal chlorination or UV treatment. High arsenic levels demand immediate community-level awareness and installation of arsenic filters or alternative water sources. Policy-level interventions for industrial waste regulation and source-wise water quality zoning are urgently needed. In addition, there is an urgent need for regular, multi-parameter monitoring, community-level awareness, and sustainable water management strategies. Safe drinking water should not be a privilege—it is a basic right. For Rooma, that right can only be protected through science-backed policies, better infrastructure, and community participation in safeguarding this essential resource.

Disclaimer (Artificial intelligence)

Author(s) hereby declare that NO generative AI technologies such as Large Language Models (ChatGPT, COPILOT, etc.) and text-to-image generators have been used during the writing or editing of this manuscript.

REFRENCES

- Adimalla, N., & Venkatayogi, S. (2018). Geochemical Characterization and Evaluation of Groundwater Suitability for Domestic and Agricultural Utility in Semi-Arid Region of Basara, Telangana State, South India. Applied Water Science, 8, Article No. 44.

- https://doi.org/10.1007/s13201-018-0682-1.

- American Public Health (Apha). American Water Works Association (awwa) and water pollution control federation (wpcf) (2012). Standard Methods for the Examination of Water and Wastewaters 22nd edition, American Public Health Association Inc. New York, p.4).

- BIS. (2021). IS 10500: Drinking Water — Specification (Second Revision). Bureau of     Indian Standards.

- Bishnoi M and Malik R (2008). Groundwater quality in environmentally degraded localities of Panipat city, India. Journal of Environmental Biology, Vol. 29 (6), pp: 881-886.

- Chaturwedi, A. K., Kashyap, N. K., Gupta, P., Biswas, S., Shriwas, S. K., Vaishnav, M. M., & Hait, M. (2024). Industrial effluents andtheir impact on water pollution: An overview. ES General, 5, 1203.

- https://doi.org/10.30919/esg1203

- Environmental Protection Agency. (2021). National Primary Drinking Water Regulations. United States EPA.

- Fathi, Ahmed & Abdelaziz, Shokry & Gad, Mohamed & Youssef, Tahani. (2021). Evaluation of Groundwater Quality for drinking and irrigation purposes Case Study (El-Tur City).

- Ganiyu, S.A., Badmus, B.S., Olurin, O.T. et al. (2018). Evaluation of seasonal variation of water quality using multivariate statistical analysis and irrigation parameter indices in the Ajakanga area, Ibadan, Nigeria. Appl water sci ,8, 357.

- https://doi.org/10.1007/s13201-018-0677-y.

- Gupta, Rachana & Singh, Deepesh. (2023). Impact of Urbanization on Groundwater Level in Kanpur City, Uttar Pradesh. 10.55084/grinrey/ERT/978-81-964105-3-7_2.

- Gupta, V., Kumar, D., Dwivedi, A., & others. (2023). Heavy metal contamination in river water, sediment, groundwater and human blood, from Kanpur, Uttar Pradesh, India. Environmental Geochemistry and Health, 45(5), 1807–1818.

- https://doi.org/10.1007/s10653-022-01290-0

- Kanel, S. R., Das, T. K., Varma, R. S., Kurwadkar, S., Chakraborty, S., Joshi, T. P., Bezbaruah, A. N., & Nadagouda, M. N. (2023). Arsenic contamination in groundwater: Geochemical basis of treatment technologies. ACS Environmental Au, 3(3), 135–152.

- https://doi.org/10.1021/acsenvironau.2c00053

- Kapoor, K., Kumar, S., & Vishwakarma, D. K. (2024). Health risk assessment of heavy metals in groundwater sources: carcinogenic and non-carcinogenic evaluation. Journal of Water and Health, 22(10), 1972-1987.

- https://doi.org/10.2166/wh.2024.243

- Manna, S., Das, P., & De, S. (2024). Groundwater hardness and alkalinity as risk factors for kidney stone disease. Journal of Nephrology and Renal Health, 8(1), 5.

- https://doi.org/10.17352/2455-5495.000105

- Misra, A. K. (2013). Physico-chemical analysis of drinking water in some rural areas of Kanpur. Biospectra, 8(1), 105–108.

- Mukhopadhyay, A., Ghosh, P., Jana, J., & Chatterjee, D. (2025). Arsenic and microbial contamination in drinking water wells: A field study in West Bengal, India. Journal of Environmental Science, Health & Sustainability, 1(1), 27–37.

- https://doi.org/10.63697/jeshs.2025.011

- Shaji, E., Sarath, K. V., Santosh, M., Krishnaprasad, P. K., Arya, B. K., & Babu, M. S. (2024). Fluoride contamination in groundwater: A global review of the status, processes, challenges, and remedial measures. Geoscience Frontiers, 15(2), Article 101734.

- https://doi.org/10.1016/j.gsf.2023.101734

- Singh, A., Durbha, K. S., Sinha, A., & Pasupuleti, S. (2024). Comparative assessment of fluoride and arsenic mobilization mechanisms among the groundwater of the major affected river basins of India. Water Supply, 24(9), 2969–2998.

- https://doi.org/10.2166/ws.2024.196

- Thakur, D., Sharma, P., & Singh, R. (2024). Microbial assessment of drinking water sources in Himachal Pradesh, India: Prevalence of coliforms and Escherichia coli in groundwater. Indian Journal of Public Health, 68(2), 210-217.

- https://doi.org/10.4103/ijph.ijph_372_24

- WHO. (2022). Guidelines for drinking-water quality (4th ed., updated 2022). World Health Organization.

- Sahoo, M., Sahoo, S., Dhar, A., & Pradhan, B. (2016). Effectiveness evaluation of objective and subjective weighting methods for aquifer vulnerability assessment in urban context. Journal of Hydrology, 541, 1303-1315.

- Yadav, S., & Yadav, E. (2025). Assessment of heavy metal pollution in groundwater in the industrial area of Kanpur Nagar, Uttar Pradesh, India. Journal of Experimental Zoology, India, 28(2), 1837.

- https://doi.org/10.51470/jez.2025.28.2.1837