## Review Article

## THE IMPACT OF MICROPLASTICS ON AQUATIC ECOSYSTEMS AND THEIR IMPLICATIONS FOR HUMAN HEALTH: A REVIEW

ABSTRACT

Microplastics (MPs) are pollutants that are smaller than 5 mm in size. Microplastics, derived primarily and secondarily from plastics have negative impact on aquatic biodiversity, food chain and human health. This review paper highlights the physical and chemical properties, classification, environmental interactions and fate of microplastics in the environment. It also reviews how MPs act as vectors for various pollutants, such as heavy metals and persistent organic pollutants (POPs), their ingestion and bioaccumulation and their associated negative effects. Furthermore, this paper address present microplastic detection techniques, as well as the limitations and inconsistencies within existing regulatory frameworks. The findings emphasize the critical need of standardizing monitoring protocols and interdisciplinary studies on the effect of MPs on human health and policies aimed to reduce microplastic pollution in aquatic bodies. The paper also deals with mitigations strategies, including the development of biodegradable plastics, advancements in water treatment technologies and establishment of setting regulatory limits to safeguard ecosystem and human health.

Keywords: Microplastics, Nano-plastics, Aquatic ecosystem, Human exposure Ecotoxicology, Bioaccumulation, Plastic degradation, Heavy metals, Regulatory framework.

### 1. INTRODUCTION

Anthropogenic pollutants plethora like pesticides, herbicides, heavy metals, hydrocarbons, plastics, hydrocarbons pose threat to aquatic ecosystems. Plastic particles less than 5 mm in size are known as Microplastics (MPs) and they pose serious environmental concern due to their widespread presence, durability and tendency to accumulate in living organisms (Yarahmadi et ​al., 2024). The dynamic nature of freshwater and coastal environments, driven by intricate physicochemical and oceanographic interactions, makes it more difficult to understand and assess pollution and to implementing effective remediation strategies, thereby positioning MPs contamination as a significant concern in ecotoxicology and environmental management.

Picture 1 : Sources of microplastics and their associated health effects

Over the past seven decades, the world’s dependence on plastic materials has increased and by 2060 global plastic output is expected to reach 1.2 billion tonnes under a business-as-usual model (Li et al., 2023; OECD, 2022). Worldwide, plastic waste from packaging accounts for 40% of total plastic waste and single-use items single-use items represents a significant source of MPs pollution (Our World in Data, 2023). Exposure of plastic materials to various environmental factors, such as UV radiation and mechanical abrasion, leads to their degradation over time, resulting in the formation of secondary microplastics (MPs) (Packnode, 2024). Microplastics hampers various physiological functions, imparting growth and reproduction and may even causes death of aquatic life through ingestion and entanglement (Rakib et al., 2023). Moreover, many toxic substances such as heavy metals and persistent organic pollutants (POPs) become adsorbed onto microplastics, thereby increasing their ecological toxicity (Amelia et al., 2021; Narwal et al., 2024).

Muhib & Rahman (2024) observed ingesstion of MPs in tilapia (Oreochromis niloticus) causing stress, growth inhibition, reproductive impairment and in severe cases mortality. Ingestion of MPs have been documented in various commercially important fishes. Microplastics act as vectors for contaminants such as organic pollutants (POPs) and heavy metals, which are transferred through food webs and ultimately enter the human body (Narwal et al., 2024; Rahman et al., 2025). Microplastics may cause serious health concerns in humans, such as inflammation, disruption of endocrine, and potential genetic effects. In humans, these MPs may cause serious health concerns including as endocrine disruption, inflammation and potential genetic effects. Given the scale and complex nature of MPs, this paper contributes to a better understanding of their classification, sources, environmental fate and biological effects in aquatic systems. In addition, this paper examines human exposure pathways, current detection techniques and regulatory frameworks and highlights key research gaps in this field. This paper also highlights emerging issues and emphasizes the importance of interdisciplinary collaboration and policy innovation aimed at mitigating the risk associated with microplastic.

2. CHARACTERISTICS OF MICROPLASTICS

MPs are synthetic polymers of size less than 5 mm and have wide range of physical and chemical properties such as size, shape, density, surface density, colour and polymer composition which affect their environmental behaviour, persistence and biological interactions. These characters also decide their transport, degradation rate and toxicity.

2.1 Plastic Composition and Environmental Relevance

Figure 1. Global plastic use by economic sector in 2021

Plastics are synthetic polymers, mainly derived from petrochemical sources, composed of long chains of repeating monomers with varying molecular weights and degrees of plasticity (Costa et al., 2016). Approximately 99% of plastics are derived from fossil fuels, and the plastic industry is intrinsically linked to the oil and gas industry and its carbon footprint (Walker & Fequet, 2023). The polymerization of monomers, whether through natural or synthetic methods, is the backbone of macroplastics, which subsequently degrade into microplastics (MPs) (Figure 1). Globally speaking, plastic production is led by specific industries: packaging materials (39.5%), building materials (20.1%), automotive parts (8.6%), electricity and electronics components (5.7%), and agriculture (3.4%) (Du et al., 2021; Kreiger et al., 2014). Other applications include textiles, consumer products, and household appliances.

A wide diversity of plastic polymers are used in these applications, including polyethylene (PE), polypropylene (PP), polyvinyl chloride (PVC), polystyrene (PS), polyethylene terephthalate (PET), low-density polyethylene (LDPE), polymethyl methacrylate (PMMA) and polyamide (PA) (Du et al., 2021; Geyer et al., 2017). These polymers vary in density, durability, tendency to degrade, and additive content, which affect how they behave once they enter the environment. Up on entering aquatic ecosystems, these plastics starts breaking down gradually after decades to centuries under the influence of mechanical abrasion, microbial activity, UV radiation thus forming secondary microplastics.

Over time, toxic leachates and adsorbed pollutants from these materials cause serious ecological risks, with evidence of long-term accumulation and adverse impacts on aquatic organisms and ecosystems.

Figure 2. Production process of the artificial and natural polymers and generation of macro plastics and microplastics from synthetic polymers (Sharma and Chatterjee, 2017)

2.2 Size, Shape, Colour, and Polymer Types of Microplastics

2.2.1 Size

Thompson et al. (2004) defined ‘microplastic’ approximately 20 years ago. MPs are plastic particles with an effective diameter of less than 5 mm (Du et al., 2021; S. Sharma and Chatterjee, 2017). The National Oceanic and Atmospheric Administration (NOAA) defines MPs as <5 mm with no lower limit (Masura et al., 2015), while the U.S. EPA specifies 5 mm to 1 nm (U.S. Environmental Protection Agency, 2024). Small MPs range from 0.001 to 1 mm, and prominent MPs range from 1 to 5 mm. The lower limit is determined by sampling and identification, usually 500 μm (Ivleva et al., 2017). For sediment and surface water samples, the standard ranges are 0.5 to 2 mm and 53 mm to 3 mm, respectively (Di & Wang, 2018). Bulk sample filters their MP down to 1 mm, though MP < 1 mm and nanoplastics (< 100 nm) need to be considered (Pan et al., 2019). But according to IUPAC, the MP lower limit is 0.1 mm (Revel et al., 2018), and current technology can only detect levels of 1μm in aquatic environments.

2.2.2 Shape

The morphology of microplastics exhibit a wide range of morphologies including fragments, foams, fibres, sphere, pellets, films and foams, originating from various sources. Fibers mostly come from synthetic clothing during washing, and the fragments result from the breakdown of larger plastic pieces. Spheres are commonly found in cosmetics and industrial abrasives, in films from the disintegration of plastic bags and packaging, and in foams and pellets, which are frequently linked to insulation materials and plastic manufacturing processes (Yuan et al., 2022a).

2.2.3 Color

Microplastics can be transparent or white, blue, red, black, or green, and are often similar to the original product or additives used in the production of the plastic. These visual clues can affect the likelihood of being ingested by aquatic organisms, as some colored particles mimic prey. Moreover, highly colored plastics can contain toxic additives, e.g., heavy-metal-based dyes, thereby raising concerns about environmental toxicity (Yuan et al., 2022a).

2.3 Physical and chemical characteristics

Microplastics contain various polymers, each with distinct properties, as shown in Table 2. The density, durability, hydrophobicity, and degradation resistance of polymers vary, affecting their buoyancy and environmental fate. For example, PE and PP are less dense than water, so they float, whereas denser plastics sink and are deposited on the seabed. Knowledge of these properties is critical for evaluating the environmental behaviour, bioavailability, and potential toxicity of microplastics in water ecosystems.

| Table 1: Classification of microplastics (MPs) (Yuan et al., 2022b) |  |
| --- | --- |
| Class | Description |
| Source | Primary microplastics (PMPs) and Secondary microplastics (SMPs) |
| Type | Films, spheres, fibres, foams, and particles |
| Shape | Film: fragments, crystals, fluff, powder, granules, shavings, round, sub-round, subangular, angular. Fibers: Polystyrene expanded Polystyrene. Spheres: beads, grains, microbeads, microspheres, cylinder, disc, flat, ovate, pellet, and ellipse. Particles: resin pellets, nurdles, pre-production pellets, nibs. General: irregular, elongated, degraded, rough, and broken-edge flakes. |
| Colour | Transparent, crystalline, white, cream-white-cream, red, orange, blue, opaque, black, grey, brown, green, pink, tan, yellow, and pigmented. |
| Erosion | Fresh, unweather, initial change, degree of cracking, weathering, grooves, surface irregularity, jagged fragments, linear fractures, subparallel ridges, and degradation |

| Table 2: Properties of main microplastics found in global aquatic environments |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Polymer type |  | Characteristics (g cm -3) (Lusher et al., 2017) | Monomer (Lithner et al., 2011) | Chemical structure (Urbanek et al., 2018) | Main use (Lusher et al., 2017) | Approx. global production million tonnes year-1 (Lithner et al., 2011) |
| Polypropylene (PP) |  | Low density (0.85-0.94) | Propylene |  | Reusable food containers and packaging, bottle caps, drinking straws, laboratory equipment | 45 |
| Polyethylene (PE) | Low-density PE (LDPE) | LDPE: 0.92 | Ethylene |  | Food wrap film, shopping bags, water pipes | 39 |
|  | High-density PE (HDPE) | HDPE: 0.96 |  |  | Toy, milk bottles, pipes, plastic bags, detergent and oil bottles, cable insulation | 32 |
| Polyvinyl chloride (PVC) |  | High density (1.38-1.50) | Vinyl chloride |  | Pipes, floors, window frames, and a shower curtains, car seat covers, raincoats, bottles, visors, shoe soles, garden hoses, electricity pipes | 37 |
| Polyethylene terephthalate (PET) |  | High density, (1.38-1.41) | Terephthalic acid, ethylene glycol |  | Plastic Soda beverage bottles and packaging, processed meat packages, peanut butter/jam jars, pillow, and sleeping bag filling, textile fibers | 33 |
| Polystyrene (PS) |  | 1.04-1.08 | Styrene |  | Fast food container, disposable plastic cups and lids, foam (i.e., “Styrofoam”), CD crystal cases, service ware, packaging materials, laboratory ware, electronic uses | 13 |
| Polyurethane (PUR) |  | Very low density (0.40-0.60) | Di/tri-isocyanate, polyol |  | Upholstery, sports mats, and packaging bags. | 9 |
| Acrylonitrile-butadiene-styrene (ABS) |  | 1.02-1.08 | Acrylonitrile, 1,3-Butadiene, styrene |  | Automotive applications, pipes | 7* |
| Polycarbonate (PC) |  | 1.20-1.22 | Bisphenol A |  | Construction materials, medical equipment, reusable beverage bottles, CDs, DVDs, street and car lights, sky-lights, baby bottles, and roofs of greenhouses, glass lenses, water pipes | 2.1 |
| Polyamide (nylon, PA) |  | 1.12-1.15 | Adipic acid |  | Textile, carpets, sportswear, miniature bearings, windshield wipers, water-hose nozzles, helmets, racehorse shoes, inks, rainwear | 1.2 |
| Styrene acrylonitrile (SAN) |  | 1.06-1.10 | Styrene, Acrylonitrile |  | Cosmetic containers, ballpoint pens, and lighters | 0.5* |

Note: Production volumes marked with an asterisk (*) indicate variable or uncertain data

Additives and Surface Chemistry

Microplastics (MPs) contain known chemical additives such as phthalates, brominated flame retardants, and bisphenol A (BPA), which can leach into the environment and interfere with the endocrine systems of aquatic life and humans. Also, plastic materials are chemically complex, and some contain more than 10,000 chemical elements, including harmful plasticizers and monomers. The large surface area-to-volume ratio and the small size of MPs and nanoplastics (NPs) render them highly effective at adsorbing environmental pollutants, including heavy metals and persistent organic pollutants (POPs). This combined exposure to the additives contained in the plastic itself, as well as to the contaminants it may carry, may result in toxic effects in the exposed organism that are amplified or synergistic (Wu et al., 2022; Yuan et al., 2022b).

3. SOURCES AND PATHWAYS OF MICROPLASTICS IN AQUATIC ENVIRONMENTS

Microplastics (MPs) have numerous sources, and they enter water bodies through complex pathways. Knowledge of these sources and modes of transportation is crucial for designing effective mitigation measures. The deposition of atmospheric particles has also become a major pathway, and MPs are transported by wind and rain to distant marine and freshwater systems. Commercial shipping, aquaculture, and abandoned fishing equipment are also sources of marine-based pollution that contribute to MP pollution through direct discharge and the degradation of plastic infrastructure (Barnes et al., 2009).

3.1 Primary vs secondary microplastics

Based on source, MPs can be classified as primary and secondary microplastics. The MPs that are produced with a specific purpose artificially are known as primary MPs and those are produced from degradation of larger plastics are called secondary MPs (Sawma et al., 2024; Ziani et al., 2023).

3.1.1 Primary Microplastics

Specially designed and manufactured microscopic primary microplastics (PMPs) are present in various consumer goods (e.g., microbeads in cosmetics and toothpaste), medicines and industrial abrasives. They are further generated through laundering, which releases synthetic fibers, as well as through the degradation of materials used in paints, adhesives, electronics and 3D printing (An et al., 2020; Costa et al., 2016; Sawma et al., 2024).

3.1.2 Secondary Microplastics

Many processes drive the degradation of plastics such as mechanical abrasion, UV induced photo-oxidation, microbial activity that produces MPs and secondary microplastics (SMPs). Plastic bags, bottles, packing materials and fishing gears main contributors to SMPs (An et al., 2020; Costa et al., 2016).

3.2 Transport Pathways

MPs can be found in terrestrial, atmospheric and aquatic environments but they primarily originate from terrestrial sources. They are transported to the oceans through rivers, which are the primary carriers, and ocean currents and winds drive their horizontal spread and accretion. In the vertical direction, heavy MPs are buried in sediments, while light ones remain on the surface. Textile MPs or degraded plastics move across terrestrial and aquatic systems in the atmosphere over long distances through wet and dry deposition, connecting remote locations.

3.2.1 Terrestrial inputs

The primary source of plastic pollution in global ecosystems is the terrestrial sector, which introduces microplastics (MPs) and nanoplastics (NPs) into soils, freshwater, and the sea. (Walker & Fequet, 2023). It is estimated that about 80% of the marine (micro) plastic pollution originates on land. It has been proposed that on land, terrestrial ecosystems contain 4 to 23 times as much microplastic (nano) waste per annum as the sea. (Horton et al., 2017; Sana et al., 2020).

3.2.1.1 Agricultural Practices

The agricultural sector plays an important role and is a particular source of soil MPs (Zhou et al., 2020). The primary means of incorporation of MPs into agricultural soil is by applying biosolids/sewage sludge (with synthetic fibers and deposited MPs) as fertilizer (Palmer, 2000; Yoon et al., 1975). The other sources are the use of plastic mulch films to protect crops, compost, and irrigation (Katsumi et al., 2021; Yang et al., 2021). These MPs, bound to the soil, can then enter nearby rivers and surface water through runoff from rain and irrigation.

3.2.2 Aquatic and Marine sources

Microplastics enter water systems through fishing practices, primarily via plastic ropes, nets, and pots. Microplastic fiber waste is a product of mechanical wear on fishing nets and ropes during use. Fishing equipment lacks proper disposal procedures and is frequently discarded in the ocean. According to US statistics, 30 million pounds are disposed of in the ocean each year. Fishing nets deteriorate, forming secondary microplastics in the sea (Environmental Investigation Agency, 2024).

Fishing equipment accounts for 10 percent of total marine plastic pollution worldwide, and in specific components, it may reach 50-100 percent, while chemical additives are even more dangerous (Environmental Investigation Agency, 2024). Since 1983, Norway's program has recovered more than 22,000 gillnets, though not evenly across regions and with varying levels of difficulty (Standal et al., 2020). Issues in tracing gear and preventing losses have remained (Deshpande & Haskins, 2021; Richardson et al., 2021).

3.2.3 Atmospheric deposition

Atmospheric deposition is now finally recognized as a significant source of microplastics (MPs) to freshwater systems, including remote and pristine systems. Terrestrial sources of microplastics, such as textile fibers and eroded plastic particles, may be transported far in the atmosphere before being deposited in water bodies via both wet and dry processes.

3.2.3.1 Remote and Pristine Locations

Other papers have reported the presence of atmospheric microplastics deposited in isolated mountain catchments (Table 2), including the French Pyrenees, the Tibetan Plateau, and the Canadian boreal lakes. These results show that microplastics may be carried over long distances from their point of origin (Allen et al., 2019; Dong et al., 2021).

3.2.3.2 Quantitative Impact

Atmospheric fallout on the Tibetan Plateau is estimated to be 3.3 tons of MPs in one lake alone during the monsoon, more than that from glacial or non-glacial runoff (Dong et al., 2021). The daily deposition rate at atmospheric sites in the French Pyrenees was hundreds of particles per square meter, and analysis of air mass transport indicates transport distances of 95km (Allen et al., 2019).

3.2.3.3 Ubiquity and Composition

Synthetic textiles and urban activities contribute to MPs in the atmosphere in the form of fibers and fragments, which are common. Their existence in both cities and rural areas promotes the universal nature of this transport channel (Miller et al., 2024).

3.2.3.4 Deposition Mechanisms

The processes of wet (rain, snow) and dry (settling) deposition can transfer MPs into the freshwater and terrestrial systems (Allen et al., 2019; Dong et al., 2021)

| Table 3. Key Findings on Atmospheric Microplastic Deposition |  |  |  |
| --- | --- | --- | --- |
| Location/Study Area | Deposition Rate/Impact | Key Findings | Citations |
| Tibetan Plateau (Nam Co) | 3.3 tons/monsoon season | Atmospheric fallout is the dominant MP source | (Dong et al., 2021) |
| French Pyrenees | Up to 249 fragments/m²/day | MPs transported up to 95 km via the atmosphere | (Allen et al., 2019) |
| Southern Blue Ridge, USA | 7.6-449.8 MPs/m²/day | Atmospheric input is significant, even in forests | (Miller et al., 2024) |
| Boreal Lakes, Canada | 0.4 ± 0.2 particles/m²/day | Atmospheric deposition is the main MP pathway | (McIlwraith et al., 2024) |
| Global/Meta-analyses | Ubiquitous, mostly fibers/fragments | Long-range transport confirmed worldwide | (Evangeliou et al., 2020; Fox et al., 2024) |

3.4 wastewater treatment plants

Major transport routes for MPs into waterways include rivers and wastewater treatment plants (WWTPs). Although wastewater treatment plants (WWTPs) are effective in the elimination of 90-99% of MPs, they are still a significant source of microplastics (nano-plastics) in the environment, whether directly through the release of effluents into water bodies or indirectly through biosolids application to the land (Walker & Fequet, 2023). Cities are also sources of inputs, including wastewater discharge, rainwater discharge, wastewater overflow, and littering into rivers, which are the primary means of transporting plastics to marine habitats (Murphy et al., 2016).

### 4.  FATE AND BEHAVIOUR OF MICROPLASTICS IN AQUATIC ENVIRONMENTS

The physical and chemical characteristics of microplastics (MPs) make them behave in complex ways in water bodies. Their destiny is determined by the type of polymer, size, density, environment, and contact with biotic and abiotic elements of the environment. After entering aquatic environments, MPs undergo various transformations that influence their distribution, persistence, and ecological effects (Witczak et al., 2024).

4.1 Environmental persistence and degradation

The slow degradation rate and strong resistance of plastics to environmental factors are the primary cause of their massive accumulation. Polyethylene (PE), polypropylene (PP), and polyvinyl chloride (PVC) and most other plastics produced in large quantities do not readily biodegrade and may take several centuries to degrade and breakdown (Geyer et al., 2017). Due to their persistent nature, microplastics may take hundreds of years to undergo complete mineralization in aquatic environments (Walker & Fequet, 2023).

Mechanisms of fragmentation and degradation

4.1.1 Physical Degradation

Weathering, commonly referred to as physical degradation, is a mechanical process involving abrasion, impact, and flexing caused by factors such as water, sand, wind and mechanical shocks. Through this process micro- and nanoplastics are produced from plastics. Synthetic fiber use accelerates the release of MPs into the environment. Physical degradation of plastics changes the structural integrity of plastics thereby making them brittle and more susceptible to further breakdown (Pfohl et al., 2022; Yuan et al., 2022a).

4.1.2 Chemical Degradation

Chemical degradation is primarily caused by environmental exposure, which includes:

Photodegradation

Photo-oxidation is triggered by ultraviolet (UV) radiation from sunlight, which breaks chemical bonds in the polymer matrix, leading to brittleness and fragmentation. It is a significant factor of surface aging and embrittlement of plastics (Pfohl et al., 2022).

Hydrolysis

Of particular importance to ester- or amide-linked polymers (e.g., PET, PU, polyamides), hydrolysis breaks these bonds in the presence of water, further enhancing degradation. Oxidation and hydrolysis can also occur synergistically, significantly accelerating chain scission and fragmentation (Deshoulles et al., 2022).

4.1.3 Biological Degradation

Biological degradation is carried out by microorganisms such as bacteria, fungi, and algae. The organisms colonize biofilm surfaces that secrete enzymes (e.g., esterases, lactases, hydrolases) that break the polymer chain, and the monomers may be mineralized into CO2 and H2O (aerobically) or into CH4 (anaerobically). Biological degradation usually takes a long time in most commercial plastics, but advances in microbial and enzyme technologies are under consideration to accelerate this process (Yuan et al., 2022a).

Microbial Activity

Biodegradation involves microorganisms, such as bacteria, molds (fungi), and algae.

Enzymatic Catalysis

They destroy MPs by hydrolysis and enzyme catalysis. Long-chain polymers are broken down into monomer components by extracellular enzymes (such as esterases, lipases, and laccases). These monomers are then normally mineralized to carbon dioxide (CO2) and water (H2O) in aerobic environments (or methane (CH4) anaerobically). The use of microorganisms, including Bacillus cereus and Agromyces mediterraneus, and marine fungi, including Zalerion maritimum, is under research as a promising greener approach to the removal and degradation of MP (Tang et al., 2022).

4.2 Biofouling and sinking behaviour

In aquatic systems, interconnected process such as biofouling and sinking determine the fate and vertical transport of microplastics (MPs) and nanoplastics (NPs) (Zettler et al., 2013). Community of fouling organisms colonize the surface of plastics, a process referred to as biofouling and these communities alter the hydrophobicity, density and degradation behaviour of the particles (Cole et al., 2011). Microbial colonization on MPs is influenced by multiple factors such as temperature, nutrient levels and surface roughness. The formation of biofilms on buoyant plastics, such as polyethylene (PE) and polypropylene (PP), increases their density and mass, causing them to sink. Denser plastics particles such as polystyrene (PS) and polyvinyl chloride (PVC), tend to settle naturally. On average 70-80% of microplastics settle on the seabed over time, although they become resuspended due to turbulence, biological activity or gas generation within biofilms. Therefore, biofouling is a key process governing vertical transport of microplastics influencing the sinking of buoyant particles, their residence time and their ecological impacts within aquatic ecosystems (Du et al., 2021).

4.3 Interaction with pollutants

Microplastics and nanoplastics may endanger ecosystems and human health by functioning as vectors for chemical and biological pollutants, facilitating their toxicity and transfer across food webs. MPs can accumulate contaminants at a significantly higher because of their distinct surface properties. Pollutants interactions may be driven by plastic additives leaching and adsorption of environmental contaminants (Du et al., 2021; Sana et al., 2020). Many plastic additives act as endocrine disrupters for instance, phthalates, flame retardants and BPA, while many pollutants like heavy metals, POPs and various other emerging pollutants get readily adsorbed by MPs. The biofilms formed by colonizing bacteria on MPs may contain pathogenic bacteria, thereby facilitating harmful microbial dispersal. This adsorption process is also influenced by various factors. Further, bioaccumulation of these pollutants and additives may happen at higher trophic levels upon their release from MPs. Multiple physiological changes are induced by these pollutants in aquatic organisms, this may include organ toxicity, oxidative stress and endocrine disruption. Thus, MPs helps in distribution of pollutants by acting as vectors and enhances the ecological risk by transferring pollutants through the food webs (Sana et al., 2020).

| Table 4: Types of pollutants adsorbed by microplastics, their bioaccumulation and associated toxicity in aquatic organisms |  |  |  |
| --- | --- | --- | --- |
| Aquatic Organisms | Pollution | Toxicity | References |
| Blood clam (Tegillarca granosa) | Bisphenol A | Neurotoxicity, immunotoxicity, an increase in neurotransmitter concentration, decrease in gene expression, and an effect on DNA methylation | (Tang et al., 2020; Weis & Alava, 2023) |
| Copepods (Acartia tonsa, Calanus finmarchicus) | PAHs fluromethane and phenanthrene | Bioaccumulation in lipid-rich tissues, MP-sorbed PAHs do not significantly accumulate or contribute to toxicity in marine organisms | (Sørensen et al., 2020) |
| Goldfish (Carassius auratus) | PAH benzo(a)pyrene | Disrupted lipid metabolism, liver damage, significantly higher Casp3 mRNA expression, oxidative stress, which leads to apoptosis | (Kim et al., 2024) |
| Mussel (Mytilus coruscus) | Dechlorane Plus | Reduced photosynthetic efficiency (reduced Fv/Fm by 0.03%), higher growth inhibition (16.15%), and oxidative damage (increased ROS by 152%). Co-exposure significantly downregulated amino acid metabolism and the tricarboxylic acid cycle (TCA), and upregulated fatty acid metabolism | (Xu et., 2023) |
| Atlantic cod (Gadus morhua) | PCB-126 | Bioaccumulation in livers and muscles, minor differences in liver and skin histology | (Bogevik et al., 2023) |
| European seabass (Dicentrarchus labrax) | DDE, BP-3, chlorpyrifos | Bioaccumulation in livers and muscles, no effect on fish condition indicators was observed | (Herrera et al., 2023) |
| Blue discus (Symphysodon aequifasciatus) | Cadmium | Oxidative stress, stimulation of innate immunity in young individuals, and antagonistic interaction between the two stressors (MP and cadmium) | (Wen et al., 2018) |
| Crucian carp (Carassius carassius) | Cadmium | Inflammation of liver and spleen cells, reduction in the diversity and number of intestinal microflora organisms, oxidative stress, and a significant upregulation in the gene expression levels of IL-8 and hsp70 | (Wei et al., 2023) |

5. ECOLOGICAL IMPACTS OF MICROPLASTICS

MPs pose a significant threat to aquatic ecosystem due to their minute size, physiochemical properties, vector nature, and their ability to interact with organisms and the environment. Both organisms and ecosystem are affected by MPs through their interactions at the organism and trophic levels, thereby hampering overall ecosystem functioning.

5.1 Effects on Aquatic Organisms

5.1.1 Ingestion and entanglement

Aquatic organisms readily ingest MPs and nanoplastics, as these particles closely resemble  natural food particles (in shape and size) (Derraik, 2002). Numerous reports have documented the bioaccumulation of MPs and naoplastics in various body parts and organs of aquatic organisms (Rist & Hartmann, 2018; Wang, AL-Hasni, et al., 2024). Particles with a size of less than 150 0.5 are highly bioavailable and can easily penetrate biological barriers (Yuan et al., 2021).

Digestive Tract Impairment

Consumed plastic debris may block the digestive system, resulting in starvation and malnourishment, and can also cause a false feeling of fullness, inhibiting feeding behaviour (Dada & Bello, 2023).

Physical Injury

Abrasion and internal injury to the digestive system may occur after ingestion, often due to mechanical forces. The long-term exposure is found to result in intestinal holes in experimental models, including zebrafish and nematodes (Speirs et al., 2024).

Size-Dependent Penetration

Smaller MPs (less than 1.5 mm) can enter deeper into internal organs and block physiological pathways. Translocation of MPs between the gut and the circulatory system may occur in the bivalves (Ward et al., 2024).

Entanglement

Big plastic debris can trap aquatic organisms, limiting their movement, harming, or even killing them. The entanglement poses a severe threat to large marine animals such as seals and cetaceans (Pinzone et al., 2021; Ward et al., 2024).

5.1.2 Physiological and cellular toxicity

MPs and NPs can have various toxic effects at the cellular and physiological levels and depend on particle size, composition, and surface chemistry.

Oxidative Stress and Inflammation

At the cellular level, microplastics also induce oxidative stress, inflammation, and autophagy, leading to tissue damage. For example, exposure to microplastics from polystyrene interferes with the BTB. This key barrier prevents spermatogenic cells from entering the outside by allowing the entry of pollutants and immune cells, a leading cause of reproductive system toxicity in males. They are presumably oxidative stress and inflammatory mechanisms, though they are not fully understood (Jiang et al., 2024).

Organ and Tissue Damage

The buildup causes histopathological changes in the gills, liver, and intestines, including necrosis and degenerative changes. Polystyrene MPs have been shown to cause liver swelling and hepatitis in zebrafish, but NPs cause chronic hepatotoxicity in medaka fish.

Neurotoxicity

Neuronal function can be disturbed by exposure to MP and NP, leading to inhibition of acetylcholinesterase (AChE) and impairing normal nervous system function. Even though there is little literature on the prevalence of the common goby, other species of aquatic life have been observed to exhibit comparable neurotoxicity in its presence, and MPs and NPs have been associated with neuroinflammation, oxidative stress, and reduced neural cell viability. To illustrate, neural stem cells exposed to polystyrene nano- and microplastics exhibit perturbed gene expression, depending on neuroinflammation and poor cellular function (Marcellus et al., 2024; Proca et al., 2024).

Developmental and Reproductive Impairment

Microplastics (MPs) and nanoplastics (NPs) have been shown to cause developmental retardation, growth retardation, and metabolic abnormalities in aquatic organisms, including crustaceans and embryonic zebrafish. MPs reduce fecundity and delay moulting in crustaceans, thereby altering reproductive and developmental mechanisms. Exposure to NP in zebrafish embryos causes increased mortality and lowered heart rate, which underscores cardiotoxicity and enhanced risk of mortality. Zebrafish research has described various mechanistic actions of MPs and NPs. Polystyrene nanoparticles (PS-NPs) have the potential to increase the toxicity of other pollutants, including silver nanoparticles, thereby altering gene expression of antioxidant defense and metabolism and altering apoptosis and immunotoxicity (Yan et al., 2023). Exposure to PS-NPs during developmental stages disrupts energy metabolism, leading to increased feeding, oxygen uptake rate, and locomotor behaviour indicative of a metabolic imbalance (Chackal et al., 2022). Co-exposure studies reveal that there are additive or synergistic metabolic disturbances, which make developmental toxicity even worse.

5.1.3 Trophic transfer and bioaccumulation

Microplastics are subject to trophic transfer and bioaccumulation in aquatic ecosystems and thus a major environmental issue. Trophic transfer is the movement of microplastics via the food web, from lower to higher trophic levels. Simultaneously, bioaccumulation is the rise in concentration of microplastics in an organism over time.

Bioaccumulation Mechanism

Controlled laboratory-based studies show that microplastics bioaccumulate at lower trophic levels of the food chain. To illustrate, mixotrophic flagellates have been shown to take up microplastics. When consumed preferentially by filter-feeding ascidians, they enrich the microplastic content in the latter's digestive tracts and support bioaccumulation, identifying unicellular organisms as an essential player in microplastic trophic transfer at the micro-level (Pennati et al., 2022). Heterotrophic marine protists (e.g., some dinoflagellates) also do the latter, so the predator grows and produces less. It transfers to microplastics, which then move further up the microbial food web (Fulfer & Menden-Deuer, 2021).

Translocation and Transfer

Microplastic transfer has been experimentally established at various trophic levels in aquatic food chains. As an example, in Baltic Sea littoral food chains such as zooplankton, chameleon shrimp, and rockpool prawn, the ingestion of liter levels of fluorescent microplastic particles is seen in each trophic level, with larger ingraction seen in predators that have prey exposure compared to environmental exposure, indicating that trophic transfer can enhance prey exposure to microplastics (Kangas et al., 2023). Also, in field studies of commercially significant marine species, including deep-sea rose shrimp, tangles of microplastic fibers have been found in the digestive tracts, suggesting the presence of microplastics and their transfer within natural populations (Yücel, 2022).

Carrier Role

Microplastics facilitate the transport of multiple environmental pollutants by adsorbing hydrophobic pollutants, such as persistent organic pollutants (POPs), heavy metals, and plastic additives. The surface characteristics of MPs promote the adsorption of pollutants and contaminants onto their surface. It is further driven by multiple factors like UV aging and biofilm colonization. Thus, MPs enhance the bioavailability and toxicity of pollutants by accumulating and distributing them within aquatic ecosystems. The above-stated interactions between MPs and pollutants contribute to biomagnification across various trophic levels, thereby posing ecological and health risks to organisms that ingest MPs (Abihssira-García et al., 2022; Chen et al., 2024; Da Costa et al., 2024; Mittal et al., 2023a).

5.2 Effects on Ecosystem Functions

Microplastics significantly disrupts food-web functions, trophic cascades, primary producers and feeding behaviour in both aquatic and terrestrial ecosystems.

5.2.1 Alteration of food webs

Impact on Primary Producers

Microplastics enter food webs, beginning with primary producers. Trophic transfer can occur, for example, through microplastic contamination of freshwater plants (e.g., Lemna minuta) by herbivorous larvae (Cataclysta lemnata). Such a transfer will lead to high mortality and the absence of typical life cycles in consumers, indicating adverse effects on producers along the food chain (Mariani et al., 2023). Besides, microplastics through contaminants degrade the growth and photosynthesis of freshwater phytoplankton (e.g., Scenedesmus armatus and Microcystis aeruginosa) and thereby alter primary production, and possibly the synthesis of the desired toxins (e.g., Microcystins). This influences the foundation of the aquatic food webs and the productivity of the entire ecosystem (Sánchez-Fortún et al., 2022). Manipulations of feeding behaviour can alter trophic dynamics. Microplastics affect the heterotrophic feeding of coral reef organisms, including large benthic foraminifera, in which organisms discriminate against pristine microplastics but interact more with conditioned (biofilm-coated) ones, and increased ecological risks may be expected over time (Joppien et al., 2022).

Trophic Cascades

Another area of concern due to such disruption is known as Trophic cascades. Indirectly, via the feeding selectivity, it influences meso-zooplankton that mediate between the microbial and classic food webs. They affect the structure of phytoplankton communities, with effects of predation pressure and feeding preferences mediating biomass of various phytoplankton communities, and any perturbation (such as due to microplastics) could alter these cascading effects (Chen et al., 2021)

Feeding Behaviour

Alterations in prey (e.g., purple urchins) behaviour under the influence of environmental stressors may alter the amount of grazing that algae experience, thereby indirectly altering primary producers and trophic cascades (Belleza et al., 2021).

5.2.2 Impact on microbial communities

Microplastics have significant effects on microbial communities, including reorganizing their structure and dynamics in water ecosystems. It has been found that microplastic diversity can positively stabilise microbial network complexity and stability by promoting greater species interactions and network connectivity, especially in warmer environments. This increased complexity can also affect microbial ecology by altering the equilibrium among microbial species and their interactions in ecosystems (Wu et al., 2024).

Microbiota Dysbiosis

The imbalance in microbial communities within the host organism can be expressed as microbiota dysbiosis. Dysbiosis in the oral cavity may promote various diseases such as dental caries and periodontal diseases, which facilitates the growth of pathogenic microorganisms. Local environmental conditions and microbial dysbiosis are modified by microbial metabolic activity and immune responses. Thus, microbiota balance is crucial to maintain health and dysbiosis is considered a key contributing factor to disease (Zhu et al., 2022).

Microbial Ecology

By functioning as vectors and facilitating or disrupting interaction among microbial communities, microplastics alter biochemical processes, antibiotic resistance and microbial communities’ structure. Through the growth of biofilms on MPs surfaces, MPs impact nutrient cycling and microbial communities in urban waters, which consequently influence microbial pathogen dynamics and ecological processes in both natural and urban aquatic ecosystems (Entezari et al., 2022).

5.2.3 Changes in sediment structure and nutrient cycling

In areas of high anthropogenic pressure, microplastics may accumulate in sediments and disrupt nutrient cycling and material turnover, thereby affecting bottom sediment composition, structure and porosity (Mutshekwa et al., 2023). MPs hamper natural nutrient cycles by sheltering pathogenic microbial communities. This not only affects biogeochemical cycles such as carbon (C) and nitrogen (N) but also alters enzymatic activities involved in these cycles and in organic matter degradation, as shifts in microbial communities occur (Wang et al., 2024). In addition, MPs alter the microalgal photosynthetic capacity, causing a significant decrease in photosynthetic efficiency and subsequent decline in nutrient uptake and carbon fixation (Jalaudin et al., 2023). They also have detrimental effects on the filtration of benthic organisms, such as mussels, thereby interfering with the nutrient supply to both sediments and water (Thomsen et al., 2024).

### 6. HUMAN HEALTH IMPLICATIONS

Microplastics are minute plastic particles less than 5 mm in size that have become ubiquitous environmental pollutants, with growing reports of human exposure through inhalation, ingestion of contaminated water and food, and dermal contamination. Microplastics and their implications for human health are generally worrisome.

6.1 Routes and Levels of Exposure

The human population is exposed to microplastics through inhalation of atmospheric particles, consumption of food and water, and the use of plastic or PTFE-coated cookware, which may release micro- and nano-plastics into food during preparation. Research estimates that, each year, exposure by inhalation and the inclusion of dust in urban centres varies between 7.37 ×104 items in children and 1.06 ×105 items in adults, comparable to ingestion via water and food. An individual's use of plastic cookware might add a few thousand microplastic particles to the home-cooked food prepared annually (Cole et al., 2024; Xu et al., 2024).

6.2 Tissue Accumulation and Potential Toxicity

The presence of microplastics has been documented in various body parts and tissues, including blood, the gastrointestinal tract, lungs, the placenta and fetal meconium, indicating their ability to penetrate biological barriers. Microplastics interactions with tissues and organs is governed by various factors such as their shape, size and degree of weathering. The bioaccumulation and toxicity of MPs are further enhanced by their ability to adsorb various contaminants and pollutants, such as heavy metals, POPs. In addition to this, the potential risk of MPs is further enhanced by biofilm formation on the surface (Hunt et al., 2024a; Mittal et al., 2023b).

6.3 Mechanisms of Harm and Health Outcomes

Cellular functions are altered by microplastics, as they induce oxidative and endoplasmic reticulum (ER) stress, inflammation, impaired function of mitochondria, organ fibrosis and cytotoxicity. Through disrupting gut microbiota, they may cause systemic metabolic and immunological effects. Exposure to microplastics, causes immunotoxicity and adversely affects reproductive performance, resulting in altered expression of reproductive genes and prolonged gamete developmental periods in animal and in-vitro models (DiBona et al., 2022; Mittal et al., 2023b).

6.4 Human Health Concerns and Evidence

Observational studies in humans suggest that microplastic exposure may result in adverse reproductive outcomes, for instance underweight birth and developmental impairments in infants. Apart from this microplastics affect overall physiological performances causing immunosuppression, inflammation, oxidative stress, lung injury and reducing sperm quality. An Enhanced risk of colon and lung cancer in organisms exposed to MPs suggests the carcinogenic potential of microplastics. Despite growing concerns, the lack of evidence and data underscores the urgent need for studies that clarify exposure levels and associated related health effects in organisms (Chartres et al., 2024; Hunt et al., 2024b).

6.5 Environmental and Regulatory Context

To better understand the origins and distribution of MPs, as well as their impacts on aquatic ecosystems, organisms and human health, studies are ongoing, as MPs are present in air, water, soil and food webs. The need for standardized exposure assessment and comprehensive toxicological studies has been emphasized by the World Health Organization (WHO). To reduce human exposure to MPs policy-driven regulatory and mitigation strategies such as reducing plastics use and replacing plastics with biodegradable materials are recommended (Boccia et al., 2024; Ma et al., 2024).

6.6 Knowledge Gaps and Risk Assessment Challenges

Significant knowledge gaps persist in the risk assessment of MPs despite of studies undertaken, such as lack of standardized protocols for microplastic detection in food items and biological tissues. Poor standardization, lack of enough reference materials and non-uniform protocols constrain advances in the understanding of MPs, especially for nanoplastics (NPs). Safe exposure level determination remains unresolved because of methodological shortcomings, highlighting gaps in toxicological studies and inconsistencies across study designs. The chronic effect of MPs on human health is not well understood due to the lack of long-term epidemiologic studies. The adsorption of various pollutants and contaminants onto the surface of MPs complicates the understanding and assessment of their effect. Induction of oxidative stress upon translocation across biological barriers by MPs has been revealed by various studies, but the precise mechanism involved remains unclear. Biodistribution modelling of microplastics is challenging due to their heterogeneity in shape, size and composition. Despite advancements in physiologically based kinetic models, data limitations remain a major challenge. Secondary microplastics are generated through the daily use of plastic products and their exposure pathways are inadequately quantified and underrepresented in current risk assessment frameworks.

## 7. DETECTION AND ANALYSIS OF MICROPLASTICS

## The study of microplastics in aquatic ecosystems has emerged as a rapidly expanding area of research, highlighting the need to understand their distribution, composition, and potential impacts. Despite increased awareness, the lack of standardized protocols and field-deployable methodologies poses a significant challenge for researchers and regulatory bodies worldwide.

## 7.1 Sampling Techniques

## Sampling techniques depend on the type of environmental matrix, such as water, sediment, or biota.

## Water

## Generally, various devices are used to collect surface water samples, including trawls, filtration systems and neuston nets. Sampling nets with a 300 μm mesh size defines the smallest particle size that can be captured.

## Sediment

## Bottom sediments are collected using grab samplers, such as core samplers and Van Veen grab samplers. Collected samples are subjected to drying and sieving, after which microplastics are isolated by density separation using saturated salt solutions such as NaCl and ZnCl₂.

## Biota

## In the case of biota, organisms are dissected, and their gastrointestinal tracts are processed using enzymatic or chemical treatments, such as KOH and H₂O₂, to extract MPs.

## 7.2 Identification Methods

## After isolation, microplastics (MPs) are referred to and defined using visual and analytical methods.

## Visual Inspection

## The samples were also studied under a stereomicroscope or an optical microscope, which is convenient for initial sorting by size, shape, and color. Subjectivity, misidentification vulnerability, and a lack of effectiveness with particles below 300 mm are among the limitations.

## Spectroscopic Techniques

## Fourier-transform infrared spectroscopy (FTIR) identifies the type of polymer based on distinct infrared absorption bands and can be used on particles larger than 20 mm. It may be used in conjunction with microscopy (m-FTIR) to increase spatial resolution. Raman Spectroscopy is sensitive to molecular vibrations through inelastic light scattering and is appropriate for smaller particles (<1 mm). It is not as erythrocyte-latching by water, and hence it is recommended in wet samples.

## Emerging Technologies

## Pyrolysis-GC/MS involves the thermal decomposition of MPs and the analysis of the resultant gases to determine the composition of the polymers. Hyperspectral Imaging is an imaging technique that uses spectroscopy to analyze a specimen quickly and non-destructively. In AI-assisted classification, spectral libraries used to train machine learning models enable more accurate, faster identification. On-site detection of MPs has been implemented by developing portable field sensors that require minimal laboratory work.

## 7.3 Challenges and Limitations

## Several challenges persist despite technological advancements. A lack of universally standardized protocols for sampling, processing and analysis has been observed. Existing analytical methods struggle to detect and quantify nanoplastics, posing a significant challenge. The use of plastic sampling nets may lead to cross-contaminate of samples by releasing microplastics. Additionally, most analytical methods are expensive, labor-intensive and rely on specialized equipment. Due to the small size of nanoplastics, chemical validation is required to avoid false-positive detections.

8. POLICIES, REGULATIONS, AND MITIGATION STRATEGIES

Mitigating microplastic contamination at a global scale necessitates a multilayered approach encompassing international collaboration, national regulatory frameworks, industrial accountability and public involvement. Although international treaties have yielded limited success, regional and local initiatives are increasingly emerging as viable pathways for mitigation.

8.1 Global and Regional Frameworks

Initiatives are underway to formulate a global treaty on plastic pollution, with the United Nations Environment Assembly (UNEA) at the center of these efforts. Despite ongoing efforts, negotiations toward a binding international agreement were unsuccessful in 2025 due to persistent divisions among key plastic-producing countries. Restrictions have been implemented on the use of non-biodegradable plastic glitter and the inclusion of microplastics in products has been regulated. The Barcelona Convention and HELCOM (Helsinki Commission) have implemented the action plans at a regional level to reduce marine litter and microplastic emissions in the Mediterranean and Baltic seas, respectively. In 2019, the Basel Convention was revised to cover plastic waste, targeting microplastic pollution indirectly by controlling the export of plastic waste.

8.2 Waste Management and Reduction

Waste management is critical to reducing microplastic emissions. To address this problem, several strategies have been implemented worldwide. First of all, bans on microbeads in cosmetic products have been introduced in many countries, including the United States, Canada, the United Kingdom, France, and South Korea. Secondly, the EU and other states in the US have recommended specific goals for reducing and recycling plastic packaging. Finally, greater focus is placed on research and policy incentives to promote the development and use of biodegradable plastics as alternatives to conventional plastics.

8.3 Industry and Innovation

Extended Producer Responsibility (EPR) policies put manufacturers in a position to take lifecycle responsibility for their products and manage their waste at the end of the consumption period. Perfect examples could include SB 54 in California, which requires manufacturers to fund recycling and reduction programs for plastic wrapping. Moreover, EPR newcomers in both Europe and Asia are claiming convergence of global standards and improved producer accountability.

Circular Economy Approaches aim to reduce plastic production, encourage reuse, and redesign products to be recyclable and reusable. These efforts are further supported by national action plans, notably the U.S. National Strategy to Prevent Plastic Pollution, which prioritizes innovation in product design and material recovery.

8.4 Public Awareness and Citizen Science

### Public awareness and participation play a vital role in the prevention of microplastics. Several educational campaigns and eco-labelling initiatives have been implemented to raise consumer awareness of the sources of microplastics and promote sustainable decision making. The Big Microplastic Survey and similar citizen science initiatives facilitate community-based data collection and regional pollution monitoring thereby helping to narrow the gap between the scientific community and the general public. School-based programs, media campaigns and community clean-up activities have demonstrated effectiveness in increasing public awareness and fostering behavioural change, while simultaneously contributing to educational initiatives and community-based surveillance.

### 9. RESEARCH GAPS AND FUTURE DIRECTIONS

Despite the growing volume of research on microplastics (MPs), critical knowledge gaps persist across analytical, ecological, toxicological and policy areas. Addressing these knowledge gaps is crucial for the design and implementation of effective mitigation strategies and regulatory frameworks.

9.1 Analytical and Monitoring Gaps

Data comparability and reproducibility are hindered by the lack of standardized protocols for sampling, pretreatment and analysis of microplastics (MPs) and nanoplastics (NPs) across various environment matrices such as water, sediment, and biota. The detection of nanoplastics is constrained by the limitations of existing analytical methods. Various developed techniques are optimized for microscale particles and are not sufficiently sensitive to achieve the required nanoscale resolution. False positives or negatives, and sample contamination, are common due to the extensive use of plastic in the laboratory and the properties of environmental matrices. However, new technologies, including AI-based classification, nanotech-based sensors, and built-in spectromicroscopic systems, have potential but require further confirmation and validation.

9.2 Ecological and Toxicological Gaps

Although most studies have focused on the acute toxicity of model organisms, the conditions of chronic and low-dose, and long-term exposures are not thoroughly studied. Species-specific studies are required, particularly on freshwater and benthic organisms, to clarify differences in sensitivity and exposure routes. Experiments conducted in laboratories do not always reflect realistic exposure conditions, such as particle aging, biofouling, and co-contaminants, and therefore lack significant ecological applicability. Moreover, the effects of mixes of microplastics and other pollutants, such as heavy metals, persistent organic pollutants (POPs), and antibiotics, should also be studied in greater detail to define possible synergistic or antagonistic toxicological responses.

9.3 Human Health and Risk Assessment

At this point, microplastics (MPs) and nanoplastics (NPs) have no established limits or reference doses for humans. There is a dearth of epidemiological research comparing MP exposure to specific health outcomes, whereas most of the information comes from in vitro or animal models. New issues have been raised on placental transfer, neurotoxicity, and disrupted endocrinology, but the mechanisms of action, as well as dose-response patterns, are poorly comprehended. The risk assessment models for MPs are still in development and do not have harmonised exposure and hazard characterisation approaches.

9.4 Policy and Governance Needs

Today, there are no global standards for microplastics in food, water, or air. There is an urgent need for harmonized international policies covering upstream interventions, such as production and design, and downstream interventions, such as waste management and remediation. The systems for controlling and regulating microplastic pollution from industries, textile industries, and waste management facilities are poorly developed. Moreover, community participation and education are not widespread, although these are the most crucial steps towards reducing plastic use and promoting behavioural change (Sharma et al., 2025).

### 10. CONCLUSION

Microplastics (MPs) are ubiquitous contaminants in aquatic environments, with significant ecological and human health implications. This review explores their sources, classification, and environmental behavior, with a focus on their distribution in freshwater and marine ecosystems. Studies have shown that ingestion of MPs by aquatic organisms can induce physiological stress and mortality. Toxic substances may also be transported with MPs, as they adsorb onto the particle surface. These pollutants including persistent organic pollutants (POPs), and heavy metals can bioaccumulate through food webs. A major human health concern is the impact of MPs on the body, particularly the placenta and circulatory system,

potentially leading to disorders such as endocrine disruption and inflammation.

Despite some progress in understanding microplastics (MPs), significant research gaps remain, including standardized MP detection, analysis of nanoplastics (NPs), and comprehensive assessment of human health risks. Uncoordinated global policies hinder a unified response to microplastic contamination in the environment. Addressing microplastic issues requires research on the development of safe and sustainable alternatives to conventional plastics, improvements in wastewater treatment technologies, implementation of the Extended Producer Responsibility strategy, and adoption of circular economy principles, including pay-or-produce systems. Public awareness initiatives can help reduce plastic use. To mitigate microplastic contamination and its adverse effects on human health and the environment, coordinated action by scientists, policymakers, industries, and governments is essential.

REFERENCES

Abihssira-García, I.S., Kögel, T., Gomiero, A., Kristensen, T., Krogstad, M. and Olsvik, P.A., 2022. Distinct polymer-dependent sorption of persistent pollutants associated with Atlantic salmon farming to microplastics. Marine Pollution Bulletin, 180, p.113794.

Allen, S., Allen, D., Phoenix, V.R., Le Roux, G., Durántez Jiménez, P., Simonneau, A., Binet, S. and Galop, D., 2019. Atmospheric transport and deposition of microplastics in a remote mountain catchment. Nature geoscience, 12(5), pp.339-344.

Amelia, T.S.M., Khalik, W.M.A.W.M., Ong, M.C., Shao, Y.T., Pan, H.J. and Bhubalan, K., 2021. Marine microplastics as vectors of major ocean pollutants and its hazards to the marine ecosystem and humans. Progress in earth and planetary science, 8(1), p.12.

An, L., Liu, Q., Deng, Y., Wu, W., Gao, Y. and Ling, W., 2020. Sources of microplastic in the environment. In Microplastics in terrestrial environments: Emerging contaminants and major challenges (pp. 143-159). Cham: Springer International Publishing.

Barnes, D.K., Galgani, F., Thompson, R.C. and Barlaz, M., 2009. Accumulation and fragmentation of plastic debris in global environments. Philosophical transactions of the royal society B: biological sciences, 364(1526), pp.1985-1998.

Belleza, D.F.C., Kawabata, Y., Toda, T. and Nishihara, G.N., 2021. Effects of dead conspecifics, hunger states, and seasons on the foraging behavior of the purple urchin Heliocidaris crassispina. Marine Ecology Progress Series, 664, pp.133-148.

Boccia, P., Mondellini, S., Mauro, S., Zanellato, M., Parolini, M. and Sturchio, E., 2024. Potential effects of environmental and occupational exposure to microplastics: an overview of air contamination. Toxics, 12(5), p.320.

Bogevik, A.S., Ytteborg, E., Madsen, A.K., Jordal, A.E.O., Karlsen, O.A. and Rønnestad, I., 2023. PCB-126 spiked to polyethylene microplastic ingested by juvenile Atlantic cod (Gadus morhua) accumulates in liver and muscle tissues. Marine Pollution Bulletin, 187, p.114528.

Buzenchi Proca, T.M., Solcan, C. and Solcan, G., 2024. Neurotoxicity of some environmental pollutants to zebrafish. Life, 14(5), p.640.

Chackal, R., Eng, T., Rodrigues, E.M., Matthews, S., Pagé-Lariviére, F., Avery-Gomm, S., Xu, E.G., Tufenkji, N., Hemmer, E. and Mennigen, J.A., 2022. Metabolic consequences of developmental exposure to polystyrene nanoplastics, the flame retardant BDE-47 and their combination in zebrafish. Frontiers in Pharmacology, 13, p.822111.

Chartres, N., Cooper, C.B., Bland, G., Pelch, K.E., Gandhi, S.A., BakenRa, A. and Woodruff, T.J., 2024. Effects of microplastic exposure on human digestive, reproductive, and respiratory health: a rapid systematic review. Environmental Science & Technology, 58(52), pp.22843-22864.

Chen, M., Si, Y., Han, L., Liu, X., Huang, B. and Kang, C.K., 2021. Effect of prey selectivity and trophic cascades induced by mesozooplankton on the dynamics of phytoplankton. Marine Ecology Progress Series, 662, pp.35-51.

Chen, X., Chen, C.E., Cheng, S. and Sweetman, A.J., 2024. Bisphenol A sorption on commercial polyvinyl chloride microplastics: Effects of UV-aging, biofilm colonization and additives on plastic behaviour in the environment. Environmental Pollution, 356, p.124218.

Cole, M., Gomiero, A., Jaén-Gil, A., Haave, M. and Lusher, A., 2024. Microplastic and PTFE contamination of food from cookware. Science of the Total Environment, 929, p.172577.

Cole, M., Lindeque, P., Halsband, C. and Galloway, T.S., 2011. Microplastics as contaminants in the marine environment: a review. Marine pollution bulletin, 62(12), pp.2588-2597.

da Costa, J.P., Avellan, A., Tubić, A., Duarte, A.C. and Rocha-Santos, T., 2024. Understanding interface exchanges for assessing environmental sorption of additives from microplastics: current knowledge and perspectives. Molecules, 29(2), p.333.

Da Costa, J.P., Santos, P.S., Duarte, A.C. and Rocha-Santos, T., 2016. (Nano) plastics in the environment–sources, fates and effects. Science of the total environment, 566, pp.15-26.

Dada, O.A. and Bello, J.O., 2023. Microplastics in carnivorous fish species, water and sediments of a coastal urban lagoon in Nigeria. Environmental Science and Pollution Research, 30(19), pp.55948-55957.

Derraik, J.G., 2002. The pollution of the marine environment by plastic debris: a review. Marine pollution bulletin, 44(9), pp.842-852.

Deshoulles, Q., Le Gall, M., Dreanno, C., Arhant, M., Priour, D. and Le Gac, P.Y., 2022. Chemical coupling between oxidation and hydrolysis in polyamide 6-A key aspect in the understanding of microplastic formation. Polymer Degradation and Stability, 197, p.109851.

Deshpande, P.C. and Haskins, C., 2021. Application of systems engineering and sustainable development goals towards sustainable management of fishing gear resources in Norway. Sustainability, 13(9), p.4914.

Di, M. and Wang, J., 2018. Microplastics in surface waters and sediments of the Three Gorges Reservoir, China. Science of the Total Environment, 616, pp.1620-1627.

DiBona, E., Haley, C., Geist, S. and Seemann, F., 2022. Developmental polyethylene microplastic fiber exposure entails subtle reproductive impacts in juvenile Japanese Medaka (Oryzias latipes). Environmental Toxicology and Chemistry, 41(11), pp.2848-2858.

Dong, H., Wang, L., Wang, X., Xu, L., Chen, M., Gong, P. and Wang, C., 2021. Microplastics in a remote lake basin of the Tibetan Plateau: Impacts of atmospheric transport and glacial melting. Environmental science & technology, 55(19), pp.12951-12960.

Du, S., Zhu, R., Cai, Y., Xu, N., Yap, P.S., Zhang, Y., He, Y. and Zhang, Y., 2021. Environmental fate and impacts of microplastics in aquatic ecosystems: a review. RSC advances, 11(26), pp.15762-15784.

Entezari, S., Al, M.A., Mostashari, A., Ganjidoust, H., Ayati, B. and Yang, J., 2022. Microplastics in urban waters and its effects on microbial communities: a critical review. Environmental Science and Pollution Research, 29(59), pp.88410-88431.

Environmental Investigation Agency. (2024). A new global treaty: Fishing gear. https://reports.eia-international.org/a-new-global-treaty/fishing-gear/

Evangeliou, N., Grythe, H., Klimont, Z., Heyes, C., Eckhardt, S., Lopez-Aparicio, S. and Stohl, A., 2020. Atmospheric transport is a major pathway of microplastics to remote regions. Nature communications, 11(1), p.3381.

Fox, S., Stefánsson, H., Peternell, M., Zlotskiy, E., Ásbjörnsson, E.J., Sturkell, E., Wanner, P. and Konrad-Schmolke, M., 2024. Physical characteristics of microplastic particles and potential for global atmospheric transport: A meta-analysis. Environmental Pollution, 342, p.122938.

Fulfer, V.M. and Menden-Deuer, S., 2021. Heterotrophic dinoflagellate growth and grazing rates reduced by microplastic ingestion. Frontiers in Marine Science, 8, p.716349.

Geyer, R., Jambeck, J.R. and Law, K.L., 2017. Production, use, and fate of all plastics ever made. Science advances, 3(7), p.e1700782.

Herrera, A., Acosta-Dacal, A., Pérez-Luzardo, O., Martínez, I., Rapp, J., Reinold, S., Montesdeoca-Esponda, S., Montero, D. and Gómez, M., 2023. Trophic transfer of DDE, BP-3 and chlorpyrifos from microplastics to tissues in Dicentrarchus labrax. Science of the Total Environment, 882, p.163295.

Horton, A.A., Walton, A., Spurgeon, D.J., Lahive, E. and Svendsen, C., 2017. Microplastics in freshwater and terrestrial environments: Evaluating the current understanding to identify the knowledge gaps and future research priorities. Science of the total environment, 586, pp.127-141.

Hunt, K., Davies, A., Fraser, A., Burden, C., Howell, A., Buckley, K., Harding, S. and Bakhbakhi, D., 2024. Exposure to microplastics and human reproductive outcomes: A systematic review. BJOG: An International Journal of Obstetrics & Gynaecology, 131(5), pp.675-683.

Ivleva, N.P., Wiesheu, A.C. and Niessner, R., 2017. Microplastic in aquatic ecosystems. Angewandte Chemie International Edition, 56(7), pp.1720-1739.

Jalaudin Basha, N.N., Adzuan Hafiz, N.B., Osman, M.S. and Abu Bakar, N.F., 2023. Unveiling the noxious effect of polystyrene microplastics in aquatic ecosystems and their toxicological behavior on fishes and microalgae. Frontiers in Toxicology, 5, p.1135081.

Jiang, J., Shu, Z. and Qiu, L., 2024. Adverse effects and potential mechanisms of polystyrene microplastics (PS-MPs) on the blood-testis barrier. Environmental Geochemistry and Health, 46(7), p.238.

Joppien, M., Westphal, H., Stuhr, M. and Doo, S.S., 2022. Microplastics alter feeding strategies of a coral reef organism. Limnology and Oceanography Letters, 7(2), pp.131-139.

Kangas, A., Setälä, O., Kauppi, L. and Lehtiniemi, M., 2023. Trophic transfer increases the exposure to microplastics in littoral predators. Marine Pollution Bulletin, 196, p.115553.

Katsumi, N., Kusube, T., Nagao, S. and Okochi, H., 2021. Accumulation of microcapsules derived from coated fertilizer in paddy fields. Chemosphere, 267, p.129185.

Kim, J.A., Park, Y.S., Kim, J.H. and Choi, C.Y., 2024. Toxic effects of polystyrene microbeads and benzo [α] pyrene on bioaccumulation, antioxidant response, and cell damage in goldfish Carassius auratus. Ecotoxicology and Environmental Safety, 270, p.115825.

Kreiger, M.A., Mulder, M.L., Glover, A.G. and Pearce, J.M., 2014. Life cycle analysis of distributed recycling of post-consumer high density polyethylene for 3-D printing filament. Journal of cleaner production, 70, pp.90-96.

Li, Y., Tao, L., Wang, Q., Wang, F., Li, G. and Song, M., 2023. Potential health impact of microplastics: a review of environmental distribution, human exposure, and toxic effects. Environment & Health, 1(4), pp.249-257.

Lithner, D., Larsson, Å. and Dave, G., 2011. Environmental and health hazard ranking and assessment of plastic polymers based on chemical composition. Science of the total environment, 409(18), pp.3309-3324.

Lusher, A., Hollman, P. and Mendoza-Hill, J., 2017. Microplastics in fisheries and aquaculture: status of knowledge on their occurrence and implications for aquatic organisms and food safety. FAO.

Ma, H., Chao, L., Wan, H. and Zhu, Q., 2024. Microplastic pollution in water systems: Characteristics and control methods. Diversity, 16(1), p.70.

Marcellus, K.A., Bugiel, S., Nunnikhoven, A., Curran, I. and Gill, S.S., 2024. Polystyrene nano-and microplastic particles induce an inflammatory gene expression profile in rat neural stem cell-derived astrocytes in vitro. Nanomaterials, 14(5), p.429.

Mariani, F., Di Lernia, D., Venditti, I., Pelella, E., Muzzi, M., Di Giulio, A. and Ceschin, S., 2023. Trophic transfer of microplastics from producer (Lemna minuta) to primary consumer (Cataclysta lemnata) in a freshwater food chain. Science of the Total Environment, 891, p.164459.

Masura, J., Baker, J.E., Foster, G.D., Arthur, C. and Herring, C., 2015. Laboratory methods for the analysis of microplastics in the marine environment: recommendations for quantifying synthetic particles in waters and sediments.

McIlwraith, H.K., Dias, M., Orihel, D.M., Rennie, M.D., Harrison, A.L., Hoffman, M.J., Provencher, J.F. and Rochman, C.M., 2024. A Multicompartment Assessment of Microplastic Contamination in Semi‐remote Boreal Lakes. Environmental toxicology and chemistry, 43(5), pp.999-1011.

Miller, J., Barrett, N., Love, J., Gray, A., Youker, R., Hall, C., Meiri, N., Gaesser, M., Randall, G., Jarrett, R. and Spafford, J., 2024. Temporal and Spatial Variations in Microplastic Concentrations in Small Headwater Basins in the Southern Blue Ridge Mountains, North Carolina, USA. Environments, 11(11), p.240.

Mittal, N., Tiwari, N., Singh, D., Tripathi, P. and Sharma, S., 2024. Toxicological impacts of microplastics on human health: a bibliometric analysis. Environmental Science and Pollution Research, 31(46), pp.57417-57429.

Muhib, M.I. and Rahman, M.M., 2024. How do fish consume microplastics? An experimental study on accumulation pattern using Nile tilapia (Oreochromis niloticus). Environmental Science and Pollution Research, 31(27), pp.39303-39317.

Murphy, F., Ewins, C., Carbonnier, F. and Quinn, B., 2016. Wastewater treatment works (WwTW) as a source of microplastics in the aquatic environment. Environmental science & technology, 50(11), pp.5800-5808.

Mutshekwa, T., Munyai, L.F., Mugwedi, L., Cuthbert, R.N., Dondofema, F. and Dalu, T., 2023. Seasonal occurrence of microplastics in sediment of two South African recreational reservoirs. Water Biology and Security, 2(3), p.100185.

Narwal, N., Kakakhel, M.A., Katyal, D., Yadav, S., Rose, P.K., Rene, E.R., Rakib, M.R.J., Khoo, K.S. and Kataria, N., 2024. Interactions between microplastic and heavy metals in the aquatic environment: Implications for toxicity and mitigation strategies. Water, Air, & Soil Pollution, 235(9), p.567.

OECD. (2022). Global Plastics Outlook: Policy Scenarios to 2060. Packnode Publishing. https://doi.org/10.1787/aa1edf33-en

Our World in Data. (2023). Packaging is the source of 40% of the planet’s plastic waste. Our World in Data. https://ourworldindata.org/data-insights/packaging-is-the-source-of-40-of-the-planets-plastic-waste

Packnode. (2024). The Overlooked Threat: Microplastics in Packaging and Their Environment. PACKNODE.

Palmer, R.J. and Updated by Staff, 2000. Polyamides, plastics. Kirk‐Othmer Encyclopedia of Chemical Technology.

Pan Zhong, P.Z., Guo HuiGe, G.H., Chen HongZhe, C.H., Wang SuMin, W.S., Sun XiuWu, S.X., Zou QingPing, Z.Q., Zhang YuanBiao, Z.Y., Lin Hui, L.H., Cai ShangZhan, C.S. and Huang Jiang, H.J., 2019. Microplastics in the Northwestern Pacific: abundance, distribution, and characteristics.

Pennati, R., Castelletti, C., Parolini, M., Scarì, G. and Mercurio, S., 2022. Mixotrophic flagellate ingestion boosts microplastic accumulation in ascidians. Journal of Experimental Zoology Part A: Ecological and Integrative Physiology, 337(6), pp.639-644.

Pfohl, P., Wagner, M., Meyer, L., Domercq, P., Praetorius, A., Hüffer, T., Hofmann, T. and Wohlleben, W., 2022. Environmental degradation of microplastics: how to measure fragmentation rates to secondary micro-and nanoplastic fragments and dissociation into dissolved organics. Environmental Science & Technology, 56(16), pp.11323-11334.

Pinzone, M., Nordøy, E.S., Eppe, G., Malherbe, C., Das, K. and Collard, F., 2021. First record of plastic debris in the stomach of a hooded seal pup from the Greenland Sea. Marine Pollution Bulletin, 167, p.112350.

Rahman, S.M., Zohora, F.T., Mohona, T.S., Khanom, M., Rahman, M.M., Sarower, M.G., Rouf, M.A., Islam, M.N., Mathew, R.T., Alkhamis, Y.A. and Alrashada, Y.N., 2025. Impact of ingested microplastics on phenotypic traits and biochemical parameters in Nile tilapia (Oreochromis niloticus) juvenile. Aquaculture International, 33(3), p.225.

Rakib, M.R.J., Sarker, A., Ram, K., Uddin, M.G., Walker, T.R., Chowdhury, T., Uddin, J., Khandaker, M.U., Rahman, M.M. and Idris, A.M., 2023. Microplastic toxicity in aquatic organisms and aquatic ecosystems: a review. Water, Air, & Soil Pollution, 234(1), p.52.

Revel, M., Châtel, A. and Mouneyrac, C., 2018. Micro (nano) plastics: a threat to human health?. Current Opinion in Environmental Science & Health, 1, pp.17-23.

Richardson, K., Hardesty, B.D., Vince, J.Z. and Wilcox, C., 2021. Global causes, drivers, and prevention measures for lost fishing gear. Frontiers in Marine Science, 8, p.690447.

Rist, S. and Hartmann, N.B., 2018. Aquatic ecotoxicity of microplastics and nanoplastics: lessons learned from engineered nanomaterials. Freshwater microplastics, 58, pp.25-49.

Sánchez-Fortún, A., D’ors, A., Fajardo, C., Martín, C., Nande, M., Mengs, G., Costa, G., Martín, M. and Sánchez-Fortún, S., 2022. Influence of contaminant-spiked polyethylene-type microplastics on the growth and primary production of the freshwater phytoplankton species Scenedesmus armatus and Microcystis aeruginosa. Environmental and Experimental Botany, 203, p.105061.

Sharma, B., Kour, D., Bassi, A., Gusain, M., Sharma, A., Kaur, T., Basu, A., Kumari, S., Khan, S.S. and Yadav, A.N., 2025. Microplastic Pollution: Challenges, Ameliorating Strategies and Governance Policies. Water, Air, & Soil Pollution, 236(9), p.585.

Sharma, S. and Chatterjee, S., 2017. Microplastic pollution, a threat to marine ecosystem and human health: a short review. Environmental Science and Pollution Research, 24(27), pp.21530-21547.

Sørensen, L., Rogers, E., Altin, D., Salaberria, I. and Booth, A.M., 2020. Sorption of PAHs to microplastic and their bioavailability and toxicity to marine copepods under co-exposure conditions. Environmental Pollution, 258, p.113844.

Speirs, Z.C., Loynes, C.A., Mathiessen, H., Elks, P.M., Renshaw, S.A. and Jørgensen, L.V.G., 2024. What can we learn about fish neutrophil and macrophage response to immune challenge from studies in zebrafish. Fish & Shellfish Immunology, 148.

Standal, D., Grimaldo, E. and Larsen, R.B., 2020. Governance implications for the implementation of biodegradable gillnets in Norway. Marine policy, 122, p.104238.

Tang, K.H.D., Lock, S.S.M., Yap, P.S., Cheah, K.W., Chan, Y.H., Yiin, C.L., Ku, A.Z.E., Loy, A.C.M., Chin, B.L.F. and Chai, Y.H., 2022. Immobilized enzyme/microorganism complexes for degradation of microplastics: A review of recent advances, feasibility and future prospects. Science of the Total Environment, 832, p.154868.

Tang, Y., Zhou, W., Sun, S., Du, X., Han, Y., Shi, W. and Liu, G., 2020. Immunotoxicity and neurotoxicity of bisphenol A and microplastics alone or in combination to a bivalve species, Tegillarca granosa. Environmental Pollution, 265, p.115115.

Thompson, R.C., Olsen, Y., Mitchell, R.P., Davis, A., Rowland, S.J., John, A.W., McGonigle, D. and Russell, A.E., 2004. Lost at sea: where is all the plastic? Science, 304(5672), pp.838-838.

Thomsen, E.S., Almeda, R. and Nielsen, T.G., 2024. Tire particles and their leachates reduce the filtration rate of the mussel Mytilus edulis. Marine Environmental Research, 195, p.106348.

U.S. Environmental Protection Agency (2024). Microplastics Research [Overviews and Factsheets]. EPA. https://www.epa.gov/water-research/microplastics-research

Urbanek, A.K., Rymowicz, W. and Mirończuk, A.M., 2018. Degradation of plastics and plastic-degrading bacteria in cold marine habitats. Applied microbiology and biotechnology, 102(18), pp.7669-7678.

Walker, T.R. and Fequet, L., 2023. Current trends of unsustainable plastic production and micro (nano) plastic pollution. TrAC Trends in Analytical Chemistry, 160, p.116984.

Wang, F., Pei, L., Zhang, S., Sun, J. and Han, L., 2024. Microplastics affect ecosystem multifunctionality: Increasing evidence from soil enzyme activities. Land Degradation & Development, 35(15), pp.4379-4405.

Wang, S., Al-Hasni, N.S., Liu, Z. and Liu, A., 2024. Multifaceted aquatic environmental differences between nanoplastics and microplastics: behavior and fate. Environment & Health, 2(10), pp.688-701.

Ward, E., Gordon, M., Hanson, R. and Jantunen, L.M., 2024. Modelling the effect of shape on atmospheric microplastic transport. Atmospheric Environment, 326, p.120458.

Wei, W., Yang, Q., Xiang, D., Chen, X., Wen, Z., Wang, X., Xu, X., Peng, C., Yang, L., Luo, M. and Xu, J., 2023. Combined impacts of microplastics and cadmium on the liver function, immune response, and intestinal microbiota of crucian carp (Carassius carassius). Ecotoxicology and environmental safety, 261, p.115104.

Weis, J.S. and Alava, J.J., 2023. (Micro) plastics are toxic pollutants. Toxics, 11(11), p.935.

Wen, B., Jin, S.R., Chen, Z.Z., Gao, J.Z., Liu, Y.N., Liu, J.H. and Feng, X.S., 2018. Single and combined effects of microplastics and cadmium on the cadmium accumulation, antioxidant defence and innate immunity of the discus fish (Symphysodon aequifasciatus). Environmental Pollution, 243, pp.462-471.

Witczak, A., Przedpełska, L., Pokorska-Niewiada, K. and Cybulski, J., 2024. Microplastics as a threat to aquatic ecosystems and human health. Toxics, 12(8), p.571.

Wu, H., Gao, T., Hu, A. and Wang, J., 2024. Network complexity and stability of microbes enhanced by microplastic diversity. Environmental science & technology, 58(9), pp.4334-4345.

Wu, P., Lin, S., Cao, G., Wu, J., Jin, H., Wang, C., Wong, M.H., Yang, Z. and Cai, Z., 2022. Absorption, distribution, metabolism, excretion and toxicity of microplastics in the human body and health implications. Journal of Hazardous Materials, 437, p.129361.

Xu, L., Bai, X., Li, K., Zhang, G., Zhang, M., Hu, M. and Huang, Y., 2024. Human exposure to ambient atmospheric microplastics in a megacity: spatiotemporal variation and associated microorganism-related health risk. Environmental Science & Technology, 58(8), pp.3702-3713.

Xu, Q., Peng, X., Guo, Y., Hao, Q., Hou, Y. and Yang, C., 2023. Effects of polystyrene microplastics on dechlorane plus bioaccumulation in the thick-shell mussel. Frontiers in Environmental Science, 11, p.1163075.

Yan, Z., Zhou, Y., Zhu, P., Bao, X. and Su, P., 2023. Polystyrene nanoplastics mediated the toxicity of silver nanoparticles in zebrafish embryos. Frontiers in Marine Science, 10, p.1195125.

Yang, L., Zhang, Y., Kang, S., Wang, Z. and Wu, C., 2021. Microplastics in soil: A review on methods, occurrence, sources, and potential risk. Science of the Total Environment, 780, p.146546.

Yarahmadi, A., Heidari, S., Sepahvand, P., Afkhami, H. and Kheradjoo, H., 2024. Microplastics and environmental effects: investigating the effects of microplastics on aquatic habitats and their impact on human health. Frontiers in Public Health, 12, p.1411389.

Yoon, D.Y., Sundararajan, P.R. and Flory, P.J., 1975. Conformational characteristics of polystyrene. Macromolecules, 8(6), pp.776-783.

Yuan, Z., Nag, R. and Cummins, E., 2022. Human health concerns regarding microplastics in the aquatic environment-From marine to food systems. Science of the Total Environment, 823, p.153730.

Yuan, Z.X., Deng, S., Chen, L., Hu, Y., Gu, J. and He, L., 2021. pH‐driven entrapment of enrofloxacin in casein‐based nanoparticles for the enhancement of oral bioavailability. Food Science & Nutrition, 9(8), pp.4057-4067.

Yücel, N., 2023. Detection of microplastic fibers tangle in deep-water rose shrimp (Parapenaeus longirostris, Lucas, 1846) in the northeastern Mediterranean Sea. Environmental Science and Pollution Research, 30(4), pp.10914-10924.

Zettler, E.R., Mincer, T.J. and Amaral-Zettler, L.A., 2013. Life in the “plastisphere”: microbial communities on plastic marine debris. Environmental science & technology, 47(13), pp.7137-7146.

Zhou, B., Wang, J., Zhang, H., Shi, H., Fei, Y., Huang, S., Tong, Y., Wen, D., Luo, Y. and Barceló, D., 2020. Microplastics in agricultural soils on the coastal plain of Hangzhou Bay, east China: Multiple sources other than plastic mulching film. Journal of Hazardous Materials, 388, p.121814.

Zhu, J., Chu, W., Luo, J., Yang, J., He, L. and Li, J., 2022. Dental materials for oral microbiota dysbiosis: an update. Frontiers in cellular and infection microbiology, 12, p.900918.