## Assessment of Physicochemical Parameters of Hanumanthapura Lake, Tumakuru District, Karnataka, India

# ABSTRACT

Lakes are the most important freshwater ecosystems for agricultural, domestic and ecological purpose. They are important suppliers of drinking water, irrigation, fisheries and the preservation of biodiversity. Ecological health and the potability of freshwater are affected by water quality, which is evaluated based on various physicochemical parameters and the concentration of nutrients. However, degradation of this important natural resource due to increasing anthropogenic pressures including agriculture runoff and domestic waste. The present study examines the physicochemical parameters of Hanumanthapura Lake from Dec- 2023 to Nov-2024 at five different sites of the Lake. Temperature, pH, EC, Turbidity, Total alkalinity, DO, Calcium hardness, Magnesium hardness Total hardness, BOD, Free carbon di- oxide, Phosphate, Chloride and TDS parameters were estimated in the laboratory using standard methods of Trivedi and Goel (1986) and APHA (2005). The result revealed that, water temperature ranged from 24.08–25.00 °C, atmospheric temperature from 25.63–26.34 °C and the pH ranges from 7.84–8.05. DO values varied from 2.87–3.41 mg/L, while, BOD level fluctuated between 19.59–23.68 mg/L. However, total alkalinity content of the water deviated from 160.10-186.08 mg/L and TDS ranges from 391.92–625.00 mg/L, magnesium deviated from 29.63–36.32 mg/L. Hanumanthapura lake has been contaminated as a result of ongoing human disputes, and agricultural runoff from the surrounding area. It concludes that, if precautionary measures are not taken, Hanumanthapura Lake water is not used for potable and agricultural purposes.

Key words: APHA, Biochemical oxygen Demand, Temperature, Anthropogenic pressure, freshwater.

# INTRODUCTION

Water is a renewable resource for persistence of all living creatures on the earth sphere. Natural fresh water bodies like lakes rivers and wetlands are the important source of to fulfil water scarcity for domestic, fishing and agricultural activities. (Eliku and Leta, 2018.). Physicochemical parameters are the fundamental indicator used to analyses freshwater quality and health of aquatic ecosystems. The parameters include physical characteristics such as colour, temperature, electrical conductivity and turbidity, chemical parameters include dissolved oxygen, pH, total dissolved solids, alkalinity, hardness and nutrient concentrations. The previous studies on water quality of lakes in Tumakuru highlights that need for regular assessment and monitoring to provide sustainable freshwater resources. Variation in physicochemical parameters of temperature, pH, dissolved oxygen, turbidity, alkalinity and nutrient levels leads to eutrophication and aquatic pollution that threaten the aquatic life of organisms and human use. In India, as in many others countries of the world, freshwater systems are the backbone for sustaining livelihood and ecological integrity. However, degradation of this important natural resource base is an emerging environmental issue in the country. Worldwide, lake water quality is lost due to increasing anthropogenic pressures including agriculture runoff and domestic waste. This Scientific study provides the knowledge to effectively preserve and manage water resources for the well-being of future generations.

# MATERIALS AND METHODS:

# STUDY AREA:

Fig-1 Satellite view of Hanumanthapura lake

Hanumanthapura lake is located in the village of Hanumanthapura, situated in the Madhugiri Taluk of Tumakuru district, Karnataka, India. Among the 200 tanks in Madhugiri Taluk, the present study specifically focuses on Hanumanthapura Lake. Lake supports irrigation across 282 hectares of surrounding agricultural land, fishing and meet local water demands. Geographically the lake is positioned within the coordinates 13˚49ʹ24.30ʺ N to 77˚13ʹ29.75ʺ E Latitude and Longitude. This water body falls under the North Pennar basin sub-basin Dodderi. The catchment area covers 19.89 Km² and average rainfall 600-800mm. The live capacity of the water body is 121.05 m.cft, which spread over 21.05 hectares.

Water samples were collected from five different sites monthly in a Plastic can and analyzed pH, Temperature, Turbidity, parameters and fixation of Dissolved Oxygen were carried out in each site of the study area (APHA, 2005). Electrical Conductivity, Total Dissolved Solids, Total Hardness, Calcium Hardness, Magnesium Hardness, Total Alkalinity, Free carbon dioxide, Phosphate, Chloride and Biological Oxygen Demand were analyzed in the laboratory accord into standard procedures of Trivedi and Goel (1986) and APHA (2005).

# RESULT AND DISSCUSSION:

Table-1 Physicochemical parameters at different Sites of Hanumanthapura Lake.

| Parameters/Sites | Units | Site-1 | Site-2 | Site-3 | Site-4 | Site-5 | Mean ± SD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Water Temperature | °C | 24.14 | 24.08 | 24.12 | 24.13 | 25.00 | 24.29 ±0.40 |
| Atmospheric Temperature | °C | 26.34 | 25.63 | 25.75 | 25.83 | 25.73 | 25.85 ±0.28 |
| Electrical Conductivity | µs/cm. | 616.07 | 580.73 | 728.99 | 733.19 | 692.72 | 670.34 ±68.68 |
| Turbidity | NTU | 232.95 | 211.58 | 214.56 | 193.98 | 212.38 | 213.09 ±13.83 |
| pH | - | 7.84 | 8.05 | 8.04 | 8.04 | 8.05 | 8.01 ±0.09 |
| Total Alkalinity | Mg/L | 175.62 | 171.27 | 186.08 | 178.17 | 160.10 | 174.25 ±9.57 |
| Dissolved Oxygen | Mg/L | 3.41 | 3.18 | 3.22 | 2.87 | 3.08 | 3.15 ±0.20 |
| Free Co2 | Mg/L | 0.17 | 0.18 | 0.19 | 0.23 | 1.29 | 0.41 ±0.49 |
| Calcium Hardness | Mg/L | 46.64 | 47.55 | 48.94 | 51.58 | 38.41 | 46.62 ±4.95 |
| Magnesium hardness | Mg/L | 34.59 | 29.63 | 31.38 | 31.81 | 36.32 | 32.74 ±2.67 |
| Total Hardness | Mg/L | 78.47 | 77.54 | 79.40 | 84.25 | 77.53 | 79.44 ±2.80 |
| Chloride | Mg/L | 66.41 | 87.92 | 85.85 | 82.45 | 80.69 | 80.66 ±8.45 |
| Phosphate | Mg/L | 0.28 | 0.24 | 0.29 | 0.24 | 0.23 | 0.26 ±0.02 |
| BOD | Mg/L | 22.30 | 21.99 | 21.10 | 23.68 | 19.59 | 21.73 ±1.51 |
| Total dissolved Solids | Mg/L | 391.92 | 455.32 | 625.00 | 500.00 | 555.00 | 505.45 ±89.69 |

Atmospheric temperature (Fig-2) varied from 25.63–26.34 °C (mean ± SD: 25.85 °C

±0.28), while Water temperature (Fig-3) ranged between 24.08–25.00 °C (mean ± SD: 24.29 °C

±0.40). These values suggest a stable thermal regime, lake ecosystem, which influences dissolved oxygen solubility and metabolic activity of aquatic organisms. Total dissolved solids (Fig-16) ranged from 391.92–625.00 mg/L (mean ±SD: 505.45 mg/L ±89.69). Electrical conductivity (Fig-4)) showed substantial variation 580.73–733.19 µS/cm (mean ±SD: 670.34 µS/cm ±68.68). Elevated EC and TDS at Site-3 and Site-4 indicate higher ionic load,

possibly due to anthropogenic inputs mainly mining activities and geological influences. (Ebrahimi Sarindizaj and Nikoo, 2025).

Turbidity (Fig-5) values ranged from 193.98–232.95 NTU (mean ±SD: 213.09 NTU

±13.83) are relatively high, leads to increased significant suspended particulate matters. It impairs light penetration, affecting primary productivity and benthic habitats of biological diversity. Total alkalinity (Fig-7) averaged 174.25 mg/L (SD ±9.57), indicating moderate buffering capacity against acidification. pH (Fig-6) remained slightly alkaline from 7.84–8.05; mean ±SD: 8.01 ±0.09, consistent with carbonate buffering (Kwon and Park, 2025).

Dissolved oxygen (Fig-8) was low 2.87–3.41 mg/L (mean ±SD: 3.15 mg/L ±0.20), below optimum levels for delicate aquatic flora and fauna. Free CO₂ (Fig-9) showed sharp increase at Site-5 (1.29 mg/L vs. <0.23 mg/L at other sites), due to domestic organic pollution and respiration dominance of microscopic organisms (Shyam et al., 2025)

Total hardness in fig-12 value ranges from 77.53–84.25 mg/L (mean ±SD: 79.44 mg/L

±2.80) was influenced by calcium hardness (Fig-10) values ranged from 38.41–51.58 mg/L; (mean: 46.62 mg/L ±4.95) and magnesium hardness in fig-11 varies from (29.63–36.32 mg/L; mean ±SD: 32.74 mg/L ±2.67) were within the permissible limits. Chloride (Fig-13) concentrations (66.41–87.92 mg/L; mean: 80.66 mg/L ±8.45) indicates moderate salinity stress (Mohan and Piryadarshinee, 2023).

Phosphate (Fig-14) levels were low (0.23–0.29 mg/L; mean ±SD: 0.26 mg/L ±0.02), suggesting limited eutrophication risk. Biochemical oxygen demand (Fig-15) ranged from 19.59–23.68 mg/L (mean ±SD: 21.73 mg/L ±1.51), exceeding typical thresholds for clean water (<3 mg/L), indicating significant organic pollution in the lake (Kavitha and Prasad, 2024).

Analysis of physicochemical parameters in lake at all sites highlights the significant concerns about potability of water and its ecological implications. International standards for potable water require standard limits in dissolved oxygen, turbidity and ionic balance. In this study high turbidity from 193.98 to 232.95 NTU, Low dissolved oxygen (DO) levels varied from 2.87–3.41 mg/L and elevated biochemical oxygen demand 19.59–23.68 mg/L indicate substantial organic pollution, rendering the water unsuitable for direct human consumption without treatment and make stress conditions in aquatic organisms (Basavaraj and Girish, 2024)

Light penetration and nutrient availability are crucial for plankton diversity. Moderate concentration of phosphate 0.23–0.29 mg/L stimulate growth of phytoplankton, while high

turbidity level restricts photosynthesis leading to imbalances in plankton diversity. Elevated free CO₂ (1.29 mg/L) at site-5 suggests localized respiration dominance favoring tolerant species while reducing overall productivity. Variation in plankton composition can destabilize the food chain and directly affecting fish growth and populations.

Fishes are more sensitive to deficiency of oxygen in aquatic ecosystem. Freshwater organisms require dissolved oxygen levels above 5 mg/L but observed concentrations below this threshold. High chloride and hardness values cause further osmoregulatory stress. Effect of ionic enrichment, organic pollution and turbidity generate poor environmental conditions for fish health and habitat. This research highlights that water is not potable for human consumption and attitudes environmental risks. Effective management measures like pollution control and regulation of nutrient are essential to protect both aquatic and human health.

# CONCLUSION:

The present investigation of physicochemical parameters across five sites demonstrates that the water quality is not potable in its current state due to elevated turbidity, high biochemical oxygen demand and critically low dissolved oxygen concentrations. These conditions reflect significant organic pollution and suspended particulate matter which compromise both human health standards and ecological balance.

From the point of ecological perspective, observed nutrient enrichment and ionic load exert direct impacts on plankton diversity altering species composition and reduces photosynthesis efficiency under high turbidity. This cause imbalances in the food chain, finally affecting population of fishes that depend on plankton as primary food source. Low dissolved oxygen levels lead to severe threat for fish growth reproduction and survival. Moderate hardness and chloride concentrations increase further physiological stress on biological diversity of Hanumanthapura Lake.

Study highlights that there are challenges by ensuring safe drinking water through effective treatment and lowers the pollution level in lake which simultaneously safeguarding aquatic biodiversity. conservation management practices like reducing organic inputs from surrounding area which regulating nutrient levels and regular monitoring of physicochemical parameters are essential to restore environmental integrity and achieve sustainable water resource for future utilization. This study underscores the importance of continuous monitoring and maintenance of water quality parameters to evaluate both potability and ecological health of aquatic ecosystem.

COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

# REFERENCES

- APHA - American Public Health Association (2005). Standard Methods for the Examination of Water and Wastewater (21st ed.). American Public Health Association, Washington, DC.

- Basavaraj, S. K., and Girish Kadadevaru, G. (2024). Assessment of Physico-chemical Parameters and Zooplankton Community at Gopalaswamy Tank, Chitradurga, Karnataka. Indian Journal of Science and Technology, Volume 17(4), (2024) pp368-372.

- Chidiac, S., El Najjar, P., Ouaini, N., El Rayess, Y., and El Azzi, D. (2023). A comprehensive review of water quality indices (WQIs): history, models, attempts and perspectives. Reviews in Environmental Science and Biotechnology, Volume 22, pp349–395. Springer Nature. https://doi.org/10.1007/s11157-023-09650-7

- Ebrahimi Sarindizaj, E., and Nikoo, M. R. (2025). Lakes and Reservoir Water Quality Indices: Evolution, Applications, and Future Directions. Water Air and Soil Pollution. Springer Nature. 237, pp 1-26. https://doi.org/10.1007/s11270-025-08689-2

- Eliku, T., and Leta, S. (2018). Spatial and seasonal variation in physicochemical parameters and heavy metals in Awash River, Ethiopia. Applied Water Science. Volume 8, 177. https://doi.org/10.1007/s13201-018-0803-x

- Kavitha, and Prasad, S., Kulkarni. (2024). Assessment of Physicochemical Quality and Challenges of Municipal Sewage Water in Manchalapur Village in the Karnataka, India. International Journal of Environment and Climate Change, Volume 14 (4), pp69-79.

- Kwon, H., and Park, S. R. (2025). Modeling stream water quality variance and interactions across watersheds: a multilevel and spatial approach. Aquatic Sciences. 87(4), pp102, Springer Nature. https://doi.org/10.1007/s00027-025-01224-9.

- Mohan, B., and Piryadarshinee, S. (2023). Phytoplankton as bio indicators of water quality in two perennial lakes of Coimbatore district, Tamil Nadu, India. International Journal of Entomology Research, Volume 8, pp10-17.

- Shyam Gupta, S., Singh, A., Rajesh, K., and Gola, P. (2025). Study of Some Physicochemical Parameters of Paliwal Park Lake Water at Agra, Uttar Pradesh, India. Asian Journal of Advances in Research, Volume 8(1), pp97-100.

- Trivedi, R. K., and Goel, P. K. (1986). Chemical and biological methods for water pollution studies. Environmental Publication, Volume 15, pp342 - 346.

26.40

26.34

26.20

26.00

oC

25.80

25.60

25.40

25.75	 25.63

25.83

25.73

Site-1 Site-2 Site-3 Site-4 Site-5

25.20

## Atmospheric Temperature

Fig-2 Variation of Atmospheric Temperature in Hanumanthapura lake.

25.20

25.00

24.80

oC

24.60

24.40

24.20

24.00

23.80

23.60

25.00

24.14

24.08

24.12

24.13

Water Temperature

Site-1 Site-2 Site-3 Site-4 Site-5

Fig-3 Variation of Water Temperature in Hanumanthapura lake.

800.00

Values in µs/cm

700.00

600.00

500.00

400.00

300.00

200.00

100.00

0.00

728.99 733.19

616.07

580.73

692.72

Site-1 Site-2 Site-3 Site-4 Site-5

## Electrical Conductivity

Fig-4 Variation of Electrical Conductivity in Hanumanthapura lake.

250.00

200.00

Values in NTU

150.00

100.00

50.00

0.00

232.95

211.58  214.56

212.38

193.98

Turbidity

Site-1 Site-2 Site-3 Site-4 Site-5

Fig-5 Variation of Turbidity in Hanumanthapura lake.

8.10

8.05

8.00

pH units

7.95

7.90

7.85

7.80

7.75

7.70

8.05	8.04	8.04	8.05

7.84

pH

Site-1 Site-2 Site-3 Site-4 Site-5

Fig-6 Variation of pH in Hanumanthapura lake.

190.00

185.00

Values in Mg/L

180.00

175.00

170.00

165.00

160.00

155.00

150.00

145.00

186.08

178.17

175.62

171.27

160.10

Total Alkalinity

Site-1 Site-2 Site-3 Site-4 Site-5

Fig-7 Variation of Total Alkalinity in Hanumanthapura lake.

3.50

3.40

Values in Mg/L

3.30

3.20

3.10

3.00

2.90

2.80

2.70

2.60

2.50

3.41

3.18

3.22

3.08

2.87

Dissolved Oxygen

Site-1 Site-2 Site-3 Site-4 Site-5

Fig-8 Variation of Dissolved Oxygen in Hanumanthapura lake.

0.17	0.18

0.19

0.23

1.40

1.29

1.20

Values in Mg/L

1.00

0.80

0.60

0.40

0.20

Site-1 Site-2 Site-3 Site-4 Site-5

0.00

## Free Co2

Fig-9 Variation of Free Co2 in Hanumanthapura lake.

46.64

47.55

48.94

51.58

38.41

60.00

50.00

Values in Mg/L

40.00

30.00

20.00

10.00

Site-1 Site-2 Site-3 Site-4 Site-5

0.00

Calcium Hardness

Fig-10 Variation of Calcium Hardness in Hanumanthapura lake.

40.00

35.00

Values in Mg/L

30.00

25.00

20.00

15.00

10.00

5.00

0.00

34.59

36.32

31.38	31.81

29.63

Magnesium Hardness

Site-1 Site-2 Site-3 Site-4 Site-5

Fig-11 Variation of Magnesium Hardness in Hanumanthapura lake.

86.00

Values in Mg/L

84.00

82.00

80.00

78.00

76.00

84.25

79.40

78.47

77.54

77.53

Site-1 Site-2 Site-3 Site-4 Site-5

74.00

## Total Hardness

Fig-12 Variation of Total Hardness in Hanumanthapura lake.

100.00

Values in Mg/L

80.00

60.00

40.00

20.00

0.00

87.92	85.85	82.45

80.69

66.41

Chloride

Site-1 Site-2 Site-3 Site-4 Site-5

Fig-13 Variation of Chloride in Hanumanthapura lake.

0.24

0.24

0.23

0.30

0.25

0.28

0.29

Values in Mg/L

0.20

0.15

0.10

0.05

Site-1 Site-2 Site-3 Site-4 Site-5

0.00

## Phosphate

Fig-14 Variation of Phosphate in Hanumanthapura lake.

22.30

21.99

21.10

19.59

25.00	23.68

20.00

Values in Mg/L

15.00

10.00

5.00

Site-1 Site-2 Site-3 Site-4 Site-5

0.00

Biological Oxygen Demand

Fig-15 Variation of BOD in Hanumanthapura lake.

700.00

600.00

Values in Mg/L

500.00

400.00

300.00

200.00

100.00

625.00

555.00

500.00

455.32

391.92

Site-1 Site-2 Site-3 Site-4 Site-5

0.00

## Total dissolved Solids

Fig-16 Variation of Total dissolved solids in Hanumanthapura lake.