Original Research Article

Seasonal Variation in Physicochemical Parameters and Plankton Diversity of Nathsagar Reservoir, Maharashtra, India

Abstract

The seasonal variation in water quality and plankton distribution was studied in the Nathsagar Reservoir, Maharashtra, India, to ascertain the role of physico-chemical parameters on plankton dynamics. Sampling was conducted at 10 sites (summer, winter, and monsoon) in  2022. Parameters such as temperature, pH, turbidity, dissolved oxygen, BOD/COD, TDS, electrical conductivity (EC), alkalinity (ALK), hardness, and major ions were tested according to standard methods. Correlations indicated positive associations among temperature, pH, turbidity, BOD, COD, and ions, and Cyanophyceae and Rotifera. In the studied water bodies, this implies that high temperatures, an alkaline environment, and organic enrichment have facilitated phytoplankton blooms and the development of a zooplankton community. By contrast, a significant positive correlation was found between dissolved oxygen and Bacillariophyceae, Cladocera, and Copepoda. The seasonal patterns indicate consistent plankton communities during winter, while pollution-tolerant taxa dominate in the monsoon and summer months. The study emphasises the influence of physicochemical factors on plankton distribution in Nathsagar Reservoir.

Keywords: Physicochemical parameters, Seasonal variation, Plankton diversity, Nathsagar Reservoir.

Introduction

Reservoir ecosystems are strongly influenced by seasonality, leading to major variations in chemistry and biology (Aware & Vishwanath, 2025). Plankton assemblage changes are quickly induced by temperature changes, as well as by nutrient availability, turbidity, and organic matter (the last variables are used to assess environmental quality in aquatic environments), making them suitable for monitoring the health of aquatic ecosystems (Negrete-García et al., 2024; Morán & Huete-Stauffer, 2018). Monsoon runoff, stable winter months, and higher summer temperatures are the most important factors that dictate water quality in tropical reservoirs and thereby control the composition and abundance of plankton communities.

Nathsagar Reservoir (Jayakwadi) is reportedly one of the major reservoirs in Maharashtra, and its potential uses include irrigation, fisheries, and a haven for migratory water birds (Jayakwadi Dam – Birds of Aurangabad, 2016). It is of ecological, social, and economic importance; however, few data are available on the seasonal impact on this reservoir's water quality (Vishwanath, 2025).

The physicochemical properties of water were interfering with the manipulation of the plankton assemblage. Phytoplankton, as primary producers, are influenced by changes in temperature and nutrient concentrations, and zooplankton, indirectly, depend on water chemistry and phytoplankton availability (Pandey & Kashyap, 2022; Sugumaran et al., 2020). Therefore, fluctuations in plankton abundance and composition are driven by interactions among physical, chemical, and biological processes under the natural conditions of the reservoir ecosystem (Neelgund & Kadadevaru, 2021). Shifts at one trophic level always affect the whole trophic web, emphasising the overall interdependence of aquatic ecosystems (Khan et al., 25).

Understanding seasonal relationships between plankton and water quality is crucial for evaluating ecosystem health and promoting sustainable conservation and management, especially amid growing anthropogenic impacts and climate variability. Observing plankton communities can provide valuable information on ecological responses to environmental change and on sustainable reservoir management (Annamalai et al., 2023; Das et al., 2024; Merkli et al., 2025).

Objectives of the Study

- To study the seasonal variation in physicochemical parameters of Nathsagar Reservoir at ten selected sites.

- To assess the Seasonal variability of phytoplankton and zooplankton populations.

- To study the correlation between physicochemical parameters and the plankton diversity in the reservoir.

2. Materials and Methods

2.1 Study Area:

Figure 1. Study Area

Nathsagar Reservoir (Jayakwadi Dam) is located in Aurangabad district, Maharashtra, India (19°32′–19°38′ N; 75°15′–75°22′ E). It is one of the largest artificial reservoirs in the state and supports an important wetland ecosystem that supports fisheries, agriculture, and migratory waterbirds. Ten sampling sites were selected alongside Nathsagar reservoir, viz. Galnimb, Warkhed, Ramdoha, Khamgaon-1, Bhavi Nimgaon, Jayakwadi (South), Jayakwadi (North), Khanapur, Erandgaon, and Lakhefal.

2.2 Physicochemical Analysis

Water samples were collected seasonally from 10 selected sites in the Nathsagar Reservoir during the monsoon, winter, and summer seasons of 2022. Sampling was carried out during morning hours to minimise diurnal variation. Clean, pre-washed polyethene bottles were used for sample collection, and the bottles were rinsed with reservoir water prior to sampling. Samples were transported to the laboratory and analysed within 24 hours.

Physicochemical parameters were analysed following standard methods prescribed by the American Public Health Association (APHA, 2017). In situ measurements included water temperature, recorded with a calibrated thermometer; pH, measured with a portable digital pH meter; and electrical conductivity (EC), total dissolved solids (TDS), measured with a digital conductivity meter.

Laboratory analysis included chloride, alkalinity (titrimetric method), total hardness, bicarbonate, calcium, magnesium, biochemical oxygen demand (BOD, 5-day incubation method), and chemical oxygen demand (COD, dichromate reflux method). Dissolved oxygen (DO) was estimated using Winkler’s iodometric method. All glassware and instruments were properly cleaned and calibrated before analysis. Each parameter was analysed in triplicate to ensure accuracy and reliability, and the mean values were used for further interpretation. Results were expressed in standard units (mg/L or µS/cm), as recommended by APHA.

2.3 Plankton Diversity

Plankton samples were collected seasonally from all sampling sites by filtering 50 litres of reservoir water through a standard plankton net having a mesh size of 25 µm. The collected samples were immediately transferred to clean, labelled bottles and preserved in 4% formalin solution to prevent decomposition and maintain the structural integrity of organisms.

In the laboratory, plankton samples were thoroughly mixed and allowed to settle. Phytoplankton and zooplankton were identified up to species level using standard taxonomic keys and reference manuals (Prescott, 1962; Edmondson, 1959; Battish, 1992). Identification was confirmed by repeated microscopic examination.

Quantitative estimation of plankton was carried out using a Sedgwick–Rafter counting chamber under a compound microscope. For each sample, multiple counts were performed to minimise counting error, and the average value was considered for analysis. Plankton abundance was expressed as cells per litre (cells/L) for phytoplankton and individuals per litre (ind./L) for zooplankton.

Six dominant species from each plankton group were selected as representative bioindicators to evaluate seasonal and spatial variation. The abundance data were used to analyse seasonal trends, group-wise distribution, and ecological relationships between plankton communities and physicochemical parameters. Correlation analysis was performed to assess the influence of water quality variables on plankton distribution and abundance.

- Results:

3.1 Seasonal Trends of Water Parameters and Plankton Diversity

Seasonal variation in physicochemical parameters was evident across all sampling sites of Nathsagar Reservoir. Water temperature ranged from 30.6–32.4 °C during the monsoon, 23.4–25.1 °C during summer, and was moderate during winter. Turbidity was relatively higher during the monsoon (7.5–9.2 NTU) compared to the summer (5.0–6.5 NTU). The pH of the reservoir remained slightly alkaline throughout the study period, ranging from 7.34 to 8.70 in the monsoon and from 6.75 to 8.60 in summer. Elevated values of BOD (6.50–7.67 mg/L) and COD (41.0–46.9 mg/L) were recorded during monsoon, while comparatively lower values were observed during summer (BOD: 5.10–6.01 mg/L; COD: 31.0–36.9 mg/L). Total dissolved solids and electrical conductivity also showed higher values during monsoon (TDS: 497–589 mg/L; EC: 0.72–0.81 mS/cm) than summer (TDS: 397–489 mg/L; EC: 0.54–0.62 mS/cm). Dissolved oxygen ranged from 6.20–6.80 mg/L during the monsoon and increased during summer (7.60–8.20 mg/L), with maximum values observed during winter.

Figure 2. Seasonal variation in physicochemical parameters of Nathsagar Reservoir during Summer 2022.

Figure 3. Seasonal variation in physicochemical parameters of Nathsagar Reservoir during Monsoon 2022.

Figure 4. Seasonal variation in physicochemical parameters of Nathsagar Reservoir during Winter 2022.

3.4. Plankton Analysis:

Plankton communities exhibited pronounced seasonal variation across the reservoir. Phytoplankton abundance was highest during winter (2,800–4,400 cells/L), followed by summer (2,600–4,300 cells/L) and monsoon (2,100–3,600 cells/L). Cyanophyceae dominated during summer, showing peak densities across all sites, whereas winter exhibited a more balanced representation of Cyanophyceae, Chlorophyceae, and Bacillariophyceae. Zooplankton abundance also reached maximum values during winter (430–600 ind./L), followed by monsoon (360–570 ind./L), and comparatively lower values during summer (380–510 ind./L). Rotifera were dominant in summer, while Cladocera and Copepoda were more abundant in winter. These results indicate clear seasonal fluctuations in plankton communities across Nathsagar Reservoir.

- ZOOPLANKTON:

(Seasonal abundance – Ind/L)

## Figure 5. Monsoon 2022 – Zooplankton.

## Figure 6. Winter 2022 – Zooplankton.

## Figure 7. Summer 2022 – Zooplankton

# B) PHYTOPLANKTON

(Seasonal abundance – cells/L)

## Figure 8. Monsoon 2022 – Phytoplankton

## Figure 9 Winter 2022 – Phytoplankton.

## Figure 10. Summer 2022 – Phytoplankton

4. Discussion

Seasonal variation in plankton diversity is strongly influenced by the reservoir's physicochemical conditions. The winter season supports greater diversity due to favourable environmental conditions, such as moderate temperatures, higher dissolved oxygen, and lower turbidity. Conversely, the summer season exhibits reduced diversity, as elevated temperatures and nutrient concentrations promote bloom-forming cyanobacteria, leading to competitive exclusion and reduced evenness among plankton communities.

Correlation of Physicochemical Parameters with Plankton

Plankton abundance and composition in Nathsagar Reservoir were closely associated with seasonal fluctuations in physicochemical parameters. Temperature, pH, turbidity, BOD, COD, TDS, electrical conductivity, alkalinity, and chloride showed positive associations with Cyanophyceae and Rotifera, indicating that elevated temperature, alkaline conditions, increased organic load, and higher ionic concentrations favour bloom-forming phytoplankton and tolerant zooplankton groups. In contrast, dissolved oxygen showed a strong positive relationship with Bacillariophyceae, Cladocera, and Copepoda, reflecting their preference for well-oxygenated, comparatively stable aquatic conditions. Increased turbidity and organic enrichment during the monsoon and summer were associated with declines in diatoms and larger zooplankton, while promoting cyanobacterial dominance. Winter conditions, characterised by moderate temperature, higher dissolved oxygen, and reduced organic stress, supported a more balanced plankton community with enhanced zooplankton density. Overall, these relationships suggest that nutrient enrichment and organic loading are key drivers of seasonal plankton dynamics in the reservoir.

5. Conclusion

Seasonal changes in water quality have a big effect on plankton diversity in Nathsagar Reservoir. Winter has the most stable ecosystem, while summer shows more organic matter and more plankton blooms. These patterns could affect fish, waterbirds, and other higher levels in the food web.

Data Availability

Data are available within the article.

COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

References:

- APHA (2017). Standard Methods for the Examination of Water and Wastewater. 23rd Ed., American Public Health Association, Washington, DC.

- Wetzel, R.G. (2001). Limnology: Lake and River Ecosystems. 3rd Ed., Academic Press, San Diego.

- Odum, E.P. (1996). Fundamentals of Ecology. 3rd Ed., W.B. Saunders, Philadelphia.

- Pielou, E.C. (1966). The measurement of diversity in different types of biological collections. Journal of Theoretical Biology, 13: 131–144.

- Trivedy, R.K., & Goel, P.K. (1986). Chemical and Biological Methods for Water Pollution Studies. Environmental Publications, Karad, India.

- Prescott, G.W. (1962). Algae of the Western Great Lakes Area. Wm. C. Brown Co., Dubuque.

- Edmondson, W.T. (1959). Freshwater Biology. 2nd Ed., John Wiley & Sons, New York.

- Needham, J.G., & Needham, P.R. (1962). A Guide to the Study of Freshwater Biology. Holden-Day, San Francisco.

- Battish, S.K. (1992). Freshwater Zooplankton of India. Oxford & IBH, New Delhi.

- Michael, R.G. (1984). Ecological studies of freshwater plankton. Indian Journal of Ecology, 11(2): 245–259.

- Ganapati, S.V. (1960). Ecology of the freshwater plankton. Hydrobiologia, 16: 257–269.

- Bais, V.S., & Agrawal, N.C. (1990). Seasonal variation of plankton in Sagar Lake, India. Journal of Freshwater Biology, 2: 49–54.

- Das, S.M., & Srivastava, V.K. (1959). Studies on the seasonal variation of plankton. Hydrobiologia, 14: 206–214.

- Pandey, J., & Pandey, U. (2003). Accumulation of heavy metals in fish. Environmental Monitoring and Assessment, 84: 19–26.

- Talling, J.F., & Lemoalle, J. (1998). Ecological Dynamics of Tropical Inland Waters. Cambridge University Press.

- Reynolds, C.S. (2006). The Ecology of Phytoplankton. Cambridge University Press.

- Saha, S.B., & Choudhury, M. (2012). Plankton diversity in wetlands. International Journal of Environmental Sciences, 3(2): 876–885.

- Vollenweider, R.A. (1968). Scientific fundamentals of eutrophication. OECD Technical Report.

- APHA (2005). Standard Methods for the Examination of Water and Wastewater. 21st Ed., APHA, Washington, DC.

- Aware, S., & Vishwanath, T. (2025). Seasonal dynamics of reservoir ecosystems. Journal of Aquatic Ecology, 40(1), 1–12.

- Negrete-García, G., López, M., & Ramírez, A. (2024). Plankton responses to environmental gradients in freshwater ecosystems. Ecological Indicators, 155, 110987.

- Morán, X. A. G., & Huete-Stauffer, T. M. (2018). Temperature-driven changes in plankton communities. Scientific Reports, 8, 1–10.

- Birds of Aurangabad. (2016). Jayakwadi Dam – Birds of Aurangabad.

- Pandey, R., & Kashyap, S. (2022). Influence of physicochemical parameters on plankton communities. International Journal of Limnology, 58(2), 45–54.

- Sugumaran, P., Suresh, M., & Kumar, R. (2020). Plankton diversity in tropical freshwater bodies. Journal of Environmental Biology, 41(3), 567–574.

- Neelgund, Y. F., & Kadadevaru, G. G. (2021). Seasonal plankton dynamics in reservoirs. Indian Journal of Ecology, 48(1), 123–130.

- Annamalai, S., Ramesh, K., & Kumar, P. (2023). Seasonal variation of plankton diversity in freshwater lakes. Water, 15(3), 456.

- Das, R., Sen, S., & Ghosh, A. (2024). Plankton as bioindicators of water quality. Frontiers in Environmental Science, 12, 100000.

- Merkli, A., Schmid, M., & Keller, B. (2025). Climate-driven changes in plankton communities. Hydrobiologia, 852, 1–15.

- Jadhav, J., Mujumdar, M., Awale, M., Patil, R., & Kuchekar, S. (2026). Comparative assessment of soil quality in drought-prone and irrigated areas and estimation of soil quality index: A novel approach. Discover Soil, 6, Article 12.

- Sobha Rani, S. (2018). Seasonal changes in water quality parameters of Malidevi estuary (Nellore) East Coast of Andhra Pradesh, India. Int J Current Innov Adv Res, 1(1), 1-6.

- Kurhe, Anil. (2013). Study of seasonal variations in physico-chemical parameters and nutrients in water of Bhatye estuary, Ratnagiri central, West coast of India. Journal of Environmental Sciences. Volume 3,. 1671-1677. 10.6088/ijes.2013030500035.