ANALYSIS OF SPERM QUALITY AND KINEMATIC PARAMETERS OF ROHU CARP (Labeo rohita) USING A CASA SYSTEM

Abstract

The quality of sperm is a key factor inducing fertilization success and the efficiency of seed production in carp aquaculture. The current research evaluated the quality and kinematic characteristics of rohu carp (Labeo rohita) sperm through a computer-assisted sperm analysis (CASA) system to deliver an objective evaluation of sperm functional ability. Semen samples were collected from hormonally stimulated mature males and examined for motility characters and specific kinematic parameters with a Biovis CASA 2000 system. A comparative evaluation of two semen lots showed notable variations in total and progressive motility, velocity metrics, and trajectory features. Lot 2 showed notably greater overall motility (98%), a higher percentage of rapidly progressive sperm, and increased velocity metrics such as curvilinear velocity (VCL), average path velocity (VAP), and straight-line velocity (VSL), as well as enhanced linearity and straightness, suggesting enhanced sperm swimming effectiveness and fertilization capability. The noted changes in CASA parameters highlight the impact of biological and methodological factors on sperm functionality. The research validates the reliability and sensitivity of CASA in evaluating rohu sperm quality and sets standard kinematic profiles for this species. The results endorse the use of CASA as a uniform instrument for assessing brood stock, evaluating semen quality, and enhancing breeding and cryopreservation methods in carp hatchery management.

Key words: Labeo rohita, CASA technology, sperm kinematics, sperm motility and semen quality.

Introduction

The key component of freshwater aquaculture in India is carp culture, which is dominated by rohu (Labeo rohita) and catla (Catla catla) (Laxmappa, 2015). Rohu (Labeo rohita), the most popular species among Indian large carps, is also widely cultivated in other Asian nations, particularly on the Indian subcontinent (Khan et al., 2004). It breeds seasonally and spawns from April to September during the south-west monsoon (Sanders & Peterson, 2011). Because it lacks certain environmental cues necessary for neurological stimulation to generate gonadotropin-releasing hormone and subsequent spawning, L. rohita does not breed in captivity. Therefore, in order to promote and synchronize spawning (Piska & Naik, 2007) for the mass generation of fish seed (Khan & Chattah, 2016), first pituitary extract (1970s) and subsequently Ovaprim, a synthetic equivalent (1980s) is being used.

Sperm quality, a measure of sperm's capacity to successfully fertilize an egg, varies greatly and is influenced by a number of external factors, including environmental temperature, feed quality, and feeding schedule (Adewumi et al., 2005; Islam and Akhter, 2011). Sperm morphology, semen production indicators (such as sperm volume and density), sperm motility, and seminal plasma composition are some of the factors that are used to evaluate the quality of sperm (Stoss, 1983; Billard et al., 1995; Linhart et al., 2000; Rurangwa et al., 2004; Alavi and Cosson, 2006).

About 20 years ago, new technology called computer-aided sperm analysis (CASA) was introduced for assessing sperm motility. It has drawn a lot of attention because of its speed, ease of use, repeatability, and objectivity (Fauvel et al., 2010). The CASA system uses a variety of photomicrography exposure and video-micrographs techniques to see and digitize successive images of sperm. According to Fauvel et al., (2010), the CASA has provided accurate and significant information on sperm quantity, kinetics, and dynamics. Furthermore, to provide accurate information on sperm quality, computer-assisted sperm analysis (CASA) systems are being employed more and more to evaluate motility characteristics like linearity (LIN), average path velocity (VAP), and curvilinear velocity (VCL) (Sahu et al., 2023). It is acknowledged that, in contrast to microscopic subjective procedures, CASA technology has offered an accurate assessment of sperm motility.

MATERIALS AND METHODS

Source of Animal

The present study was conducted using ten sexually mature male rohu (Labeo rohita) obtained from the Live Fish Germplasm Resource Centre, ICAR–NBFGR. Individual body weight were recorded and PIT tags were injected into the dorsal region of the species for proper identification. In the hatchery healthy male brood fish were hold separately in shadowed tanks and supply with well aerated water. Brood stock were fed a commercial diet at 2% of body weight per day, and feeding was stopped 24 h prior to stripping. To keep the temperature steady and the levels of ammonia and nitrites at zero, well water was used maintained between 6 and 8 ppm of oxygen. Fish were transported to the hatchery 12–24 h prior to stripping and acclimatized in 5000 L tanks under continuous showering for 6 h before hormonal induction.

Semen Collection

The male brood fish were induced with WOVA-FH (Biostadt India Limited, Mumbai) at a dose of 0.2 mg/kg body weight. Prior to semen collection, the male brood fish were anesthetized using phenoxyethanol (Bernath et al., 2016) at 0.4 ml/L. The abdominal surface was carefully wiped with tissue paper to remove moisture and water if any, to avoid unexpected activation of the spermatozoa. The milt was collected from the fishes after 4 hours of hormonal administration by hand stripping method in dry plastic sperm collection box. Milt collected from the fishes was kept separately for estimating the quality of milt.

Refrigerated (non-frozen) Storage & Shipping of Semen Samples

Collected milt samples were immediately kept on crushed ice (4°C) above the aluminium foil and transported to the laboratory as early as possible for quality assessment. For each male, separate milt boxes were used for milt collection with proper labelling to avoid cross contamination.

Evaluation of Semen Quality

Subjective Sperm Motility

Sperm motility was initially determined subjectively using a light microscope (OLYMPUS, CX43RF, Japan) at 20X magnification. A 2 µl aliquot of sperm sample was activated with 8µl of distilled water on a glass slide.  Motility was defined visually as the percentage of spermatozoa moving in a forward motion. Only samples exhibiting ≥80% motility were selected for CASA analysis. An arbitrary scoring system modified form Kurokara (1984) was used to evaluate sperm motility.

CASA based sperm motility and kinematics

Sperm motility and kinematic parameters were analyzed using a Biovis CASA 2000 system (Expert Vision Labs, Mumbai). For analysis, 10µl of milt sample was diluted with 590 µl of extenders [stock solution 1.0 g/10 ml]: Extender 9C (6500 µl/100 ml NaCl, 1420 µl/100 ml KCl, 300 µl/100 ml CaCl2, 200 µl/100 ml MgSO4.7H2O and 540 µl/100 ml Glucose), Extender 9B (6500 µl/100 ml NaCl, 1420 µl/100 ml KCl, 300 µl/100 ml CaCl2, 200 µl/100 ml MgSO4.7H2O and  5000 µl/100 ml Glycine) and Extender 7 (NaCl 7500, KCl 200, CaCl2 200 and NaHCO3 200. From this mixture, 0.2µl of milt solution was loaded onto a Shukratara chamber and activated with 0.8µl of distilled water. The percentage of motility of the spermatozoa were evaluated microscopically at 20 X magnification. Those milt samples showing more than 80% motility alone were selected for further processing. Carp sperm movement was characterized by motility characteristics, including motile (%), immotile (%), rapid prog (%), slow prog (%), curvilinear velocity (VCL), average path velocity (VAP), straight line velocity (VSL), linearity (LIN%), straightness (STR%), wobble (WOB%), beat cross frequency (BCF), amplitude of lateral head displacement (ALH), and distance related movement parameters.

Statistical Analysis

Data were analyzed using IBM SPSS Statistics version 25. Results are expressed as mean ± standard deviation (SD). Differences between groups were analyzed using Duncan’s multiple range test, and statistical significance was accepted at p < 0.05.

Results and Discussion

This study was carried out with computer assisted sperm analysis (CASA) system to determine sperm motility and kinematic parameters between two semen lots (Table 1) in rohu fish.

| Parameter | Lot 1 (Mean ± SD) | Lot 2 (Mean ± SD) |
| --- | --- | --- |
| Motility (%) | 0.94±0.04 | 0.98±0.02 |
| Immotile (%) | 0.05±0.04 | 0.01±0.02 |
| Rapid Prog (%)* | 0.55±0.18* | 0.81±0.09* |
| Slow Prog (%) | 0.27±0.14 | 0.10±0.08 |
| VCL- Curvilinear Velocity (µm/sec) | 85.08±24.03 | 105.98±23.77 |
| VAP- Average Path Velocity (µm/sec) | 58.34±13.42 | 73.10±11.32 |
| VSL- Straight Line Velocity (µm/sec)* | 48.68±12.03* | 64.96±9.30* |
| LIN- Linearity (%) | 0.57±0.03 | 0.61±0.05 |
| STR- Straightness (%)* | 0.81±0.03* | 0.87±0.01* |
| WOB- Wobble (%) | 0.69±0.03 | 0.69±0.05 |
| BCF- Beat Cross Frequency (Hz) | 4.26±1.46 | 4.80±0.14 |
| ALH- Amplitude of Lateral Head displacement (µm) | 34.62±5.13 | 30.20±3.83 |
| DNC- Distance of Net Curvilinear displacement (sqµm/sec) | 1769.38±303.85 | 2165.44±857.78 |
| DNM Distance of Normalized mean path (sqµm/sec) | 35.88±3.54 | 32.72±9.79 |

Table 1. Comparative Analysis of Sperm Motility and Kinematic Parameters between Lot 1 and Lot 2 in Rohu Milt

Lot 2 exhibited significantly higher total motility (98 ± 2%) compared to Lot 1 (94 ± 4%) suggesting a higher proportion of migrating sperm. Rapid progressive sperm were also significantly higher in Lot 2 (81 ± 9%) than in Lot 1 (55 ± 18%), indicating superior fertilization potential. Lot 2, superior overall performance is further supported by its reduced immotile (0.01±0.02) than Lot 1 (0.05±0.04) and slow Prog was reported lower in Lot 2 (0.10±0.08%) than Lot 1 (0.27±0.14).

Velocity parameters VCL (105.98 µm/sec), VAP (73.10 µm/sec), and VSL (64.96 µm/sec) were consistently higher in Lot 2, suggesting faster and more directed sperm movement. Additionally, higher linearity (LIN %) and straightness (STR %) values (0.61±0.05%) and (0.87±0.01%) observed in Lot 2 indicate efficient forward progression which is essential for successful fertilization. Lot 2 exhibits a little higher beat-cross frequency BCF (Hz) (4.80±0.14) than Lot 1 (4.26±1.46) indicating more vigorous flagellar motility, even though both lots had comparable wobble (WOB%) values Lot 1’s (0.69±0.03) and Lot 2’s (0.69±0.05). Although Lot 1 showed slightly higher ALH (4.62±5.13µm) values than Lot 2 (30.20±3.83µm) which suggests more lateral sperm head movement may reflect less efficient swimming trajectories. Overall, the superior motility and kinematic performance of Lot 2 suggests better sperm quality.

| Literature | Our findings | Ozgur et al. (2019) | Varkonyi et al. (2019) | Cejko et al. (2014) |
| --- | --- | --- | --- | --- |
| VCL (µm/sec) | 105.98±23.77 | 114.13±10.41 | 119±10 | 190.0 ± 46.2 |
| VSL (µm/sec) | 64.96±9.30 | 48.44±5.10 | 98±7 | 133.5 ± 41.6 |
| VAP (µm/sec) | 73.10±11.32 | 45.65±7.19 | - | - |
| LIN (%) | 0.61±0.05 | 19.74±8.17 | 82±4 | 51.5 ± 6.8 |
| ALH (µm) | 34.62±5.13 | 20.91±6.13 | 1.4±0.3 | 1.5 ± 0.2 |

Table 2. Comparison of CASA-based sperm kinematic parameters of Labeo rohita with previous studies

Our findings showed that the values were differed from those of other researchers (Table 2). The differences in the investigated values may be due to different computer assisted sperm analyzer systems (Boryshpolets et al 2013), differences in the diet of fish (Izquierdo et al 2001), stress factors of mature fish (Kime, D. E., & Nash, J. P. (1999), breeding season and age of mature fish (And, V. V., & Zohar, Y. (1999). Brood stock genetics, environmental factors, hormonal treatments, handling, and activating solution can all cause variations in sperm kinematics, demonstrating the need for standard evaluation techniques.

Conclusion

The current research shows the reliability of computer-assisted sperm analysis (CASA) in objectively assessing sperm quality and kinematic parameters in rohu carp (Labeo rohita). Differences in motility and velocity characteristics among semen batches suggest variations in sperm functional capability. Future investigations need to concentrate on determining species-specific CASA reference values and verifying essential kinematic parameters by examining their relationship with fertilization success, hatching rates, and post-thaw outcomes. These research efforts will enhance the use of CASA in selecting brood stock, cryopreservation, and optimizing hatcheries in carp aquaculture.

Disclaimer

The authors declare that this research work is original and has not been published previously, nor is it under consideration for publication elsewhere. The study was conducted following standard ethical guidelines and institutional norms for the care and use of experimental animals. The opinions, interpretations, and conclusions expressed in this manuscript are solely those of the authors and do not necessarily reflect the views of the affiliated institutions. The publisher and editors are not responsible for any use of the information presented in this article.

Competing Interest

The authors declare that there are no competing interests regarding the publication of this paper. All authors have approved the final version of the manuscript and agree with its submission to the journal.

COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

References

Adewumi, A. A., Olaleye, V. F., & Adesulu, E. A. (2005). Egg and sperm quality of the African catfish, Clarias gariepinus (Burchell) broodstock fed differently heated soybean-based diets. Research Journal of Agriculture and Biological Sciences, 1(1), 17-22.

Afzal Khan, M., Jafri, A. K., & Chadha, N. K. (2004). Growth and body composition of rohu, Labeo rohita (Hamilton), fed compound diet: Winter feeding and rearing to marketable size. Journal of Applied Ichthyology, 20(4), 265-270.

Alavi, S. M. H., Cosson, J., & Kazemi, R. (2006). Semen characteristics in Acipenser persicus in relation to sequential stripping. Journal of Applied Ichthyology, 22, 400-405.

And, V. V., & Zohar, Y. (1999). Age‐related sperm quality of captive striped bass Morone saxatilis. Journal of the World Aquaculture Society, 30(1), 65-72.

Billard R, Cosson J, Perchec G, Linhart O. Biology of sperm and artificial reproduction in carp. Aquaculture, 1995; 129:95-112.

Boryshpolets, S., Kowalski, R. K., Dietrich, G. J., Dzyuba, B., & Ciereszko, A. (2013). Different computer-assisted sperm analysis (CASA) systems highly influence sperm motility parameters. Theriogenology, 80(7), 758-765.

Cejko, B. I., Krejszeff, S., Judycka, S., Sarosiek, B., Dietrich, M., Kucharczyk, D., & Kowalski, R. K. (2014). Sperm quality and selected biochemical markers of seminal plasma at the beginning of the reproductive period of common carp, Cyprinus carpio L. Aquaculture International, 22(1), 111-122.

Fauvel, C., Suquet, M., & Cosson, J. (2010). Evaluation of fish sperm quality. Journal of Applied Ichthyology, 26(5), 636-643.

Islam, M. S., & Akhter, T. (2011). Tale of fish sperm and factors affecting sperm motility: a review. Advances in Life Sciences, 1(1), 11-19.

Izquierdo, M. S., Fernandez-Palacios, H., & Tacon, A. G. J. (2001). Effect of broodstock nutrition on reproductive performance of fish. Aquaculture, 197(1-4), 25-42.

Khan, M. N., & Chattah, A. M. (2016). Introduction to aquaculture in Pakistan. In K. Fitzsimmins (Ed.), Aquaculture handbook: Fish farming and nutrition in Pakistan (pp. 1–18). Pakistan: SoyPak (ASA/WISHH).

Kime, D. E., & Nash, J. P. (1999). Gamete viability as an indicator of reproductive endocrine disruption in fish. Science of the Total Environment, 233(1-3), 123-129.

Kurokura, H., Hirano, R., Tomita, M., & Iwahashi, M. (1984). Cryopreservation of carp sperm. Aquaculture, 37(3), 267-273.

Laxmappa, B. (2015). Catfish production in India: present status and prospects.

Linhart, O., Rodina, M., & Cosson, J. (2000). Cryopreservation of sperm in common carp Cyprinus carpio: sperm motility and hatching success of embryos. Cryobiology, 41(3), 241-250.

Özgür, M. E., Okumuş, F., & Kocamaz, A. F. (2019). A novel computer assisted sperm analyzer for assessment of spermatozoa motility in fish; BASA-Sperm Aqua. El-Cezeri, 6(1), 208-219.

Piska, R. S. V., & Naik, J. K. S. (2007). Fresh water aquaculture fisheries. II year paper I intermediate vocational course. State Institute of Vocational Education and Board of Intermediate.

Rurangwa, E., Kime, D. E., Ollevier, F., & Nash, J. P. (2004). The measurement of sperm motility and factors affecting sperm quality in cultured fish. Aquaculture, 234(1-4), 1-28.

Sahu, S., Meher, P. K., and Swain, P.2023. Advancements in CASA system for analyzing sperm motility in freshwater fish species. Aquaculture and Fisheries, 8(2): 134–140.

Sanders, J. D., & Peterson, S. B. (2011). Carp: Habitat, Management, and Diseases. Nova Science Publishers.

Stoss, J., & Holtz, W. (1983). Cryopreservation of rainbow trout (Salmo gairdneri) sperm: III. Effect of proteins in the diluent, sperm from different males and interval between sperm collection and freezing. Aquaculture, 31(2-4), 275-282.

Várkonyi, L., Bokor, Z., Molnár, J., Fodor, F., Szári, Z., Ferincz, Á. & Bernáth, G. (2019). The comparison of two different extenders for the improvement of large‐scale sperm cryopreservation in common carp (Cyprinus carpio). Reproduction in Domestic Animals, 54(3), 639-645.