SEASONAL VARIATIONS IN PHYSICOCHEMICAL CHARACTERISTICS, PLANKTON DIVERSITY, AND AQUATIC BIODIVERSITY OF GHANPUR (MULUG) POND, TELANGANA, INDIA

ABSTRACT

Freshwater ponds are ecologically significant ecosystems that support biodiversity and provide essential ecosystem services. The present study evaluated seasonal variations in physicochemical characteristics, plankton diversity, and aquatic biodiversity of Ghanpur (Mulug) Pond, Telangana, India, to assess its ecological status.

To evaluate these variations, Water samples were collected and analyzed during three distinct seasons: pre-monsoon, monsoon, and post-monsoon. Key physico-chemical parameters studied included temperature, pH, dissolved oxygen (DO), biochemical oxygen demand (BOD), chemical oxygen demand (COD), total dissolved solids (TDS), turbidity, alkalinity, hardness, chlorides, nitrates, and phosphates. In addition, biological components such as phytoplankton, zooplankton, aquatic macrophytes, fishes, insects, and amphibians were examined to assess the ecological status of the pond.

Water temperature varied between 26.2 and 31.5°C, dissolved oxygen ranged from 5.6 to 7.4 mg/L, while BOD varied from 4.2 to 8.4 mg/L, indicating seasonal fluctuations in water quality. Higher nutrient concentrations during the monsoon enhanced phytoplankton productivity, whereas post-monsoon conditions favored improved biodiversity. The post-monsoon season represented a stabilization phase, with moderate temperature, balanced pH, and improved dissolved oxygen levels. Increased abundance of diatoms and zooplankton groups indicated better ecological conditions, along with higher activity of aquatic fauna.

The observed patterns are consistent with findings from studies on Ghanpur Lake in the Warangal region, confirming the influence of seasonal variation on freshwater ecosystems. Overall, the study indicates moderate pollution during pre-monsoon and highlights the importance of continuous monitoring and effective management practices for maintaining pond health and sustainability.

KEYWORDS: Limnology; Freshwater ecosystem; Seasonal variation; Physicochemical parameters; Plankton diversity; Water Quality Index; Telangana.

- INTRODUCTION

Freshwater ecosystems are among the most productive and ecologically significant natural resources, providing essential services such as drinking water, irrigation, fisheries, groundwater recharge, biodiversity conservation, and recreational opportunities (Wetzel, 2001; Dodds & Whiles, 2020). The ecological health of these ecosystems is largely determined by the interaction between physicochemical characteristics and biological communities, particularly plankton, which respond rapidly to environmental changes and therefore serve as reliable indicators of water quality (Smith et al., 1999; Shetty & Gulimane, 2023).

Seasonal fluctuations in climatic factors, including temperature, rainfall, evaporation, and surface runoff, strongly influence the limnological characteristics of freshwater bodies. These seasonal variations affect water temperature, pH, dissolved oxygen (DO), biochemical oxygen demand (BOD), chemical oxygen demand (COD), total dissolved solids (TDS), nutrient concentrations, and biological productivity Agricultural runoff, domestic wastewater, and anthropogenic disturbances further modify nutrient dynamics, often resulting in eutrophication and deterioration of water quality (Smith et al., 1999; Sarkar et al., 2020).

Plankton communities, comprising phytoplankton and zooplankton, play a fundamental role in aquatic food webs by supporting higher trophic levels and maintaining ecosystem stability. Phytoplankton contribute substantially to primary productivity, whereas zooplankton regulate algal populations and serve as an important food source for fish and other aquatic organisms (Needham & Needham, 1962; Battish, 1992). Because plankton respond rapidly to changes in environmental conditions, their diversity and abundance are widely used as biological indicators for assessing freshwater ecosystem health (Aravinth et al., 2023; Panikkar et al., 2024).

Assessment of physicochemical parameters together with plankton diversity provides a comprehensive understanding of freshwater ecosystem dynamics. Standard analytical procedures recommended by the American Public Health Association ensure reliable evaluation of water quality parameters and facilitate comparison among freshwater ecosystems (APHA, 2017). Furthermore, integrated approaches such as the Water Quality Index (WQI) provide a simplified yet effective means of evaluating overall water quality and trophic status for environmental management (Carlson, 1977).

Several investigations have reported significant seasonal variation in physicochemical characteristics, nutrient dynamics, and plankton diversity in Indian freshwater ecosystems (Sarkar et al., 2020; Aravinth et al., 2023; Panikkar et al., 2024; Garai et al., 2025). However, scientific information regarding the limnological characteristics and biodiversity of Ghanpur (Mulug) Pond in Jayashankar Bhupalpally District, Telangana, remains limited.

Therefore, the present investigation was undertaken to evaluate the seasonal variation in physicochemical characteristics, plankton diversity, and aquatic biodiversity of Ghanpur (Mulug) Pond. The study also aims to assess overall water quality using the Water Quality Index (WQI), examine seasonal ecological changes, and generate baseline information for sustainable management and conservation of freshwater ecosystems in Telangana.

Objectives of the Study

- To analyze seasonal variation in physicochemical parameters

- To evaluate water quality using DO, BOD, COD, TDS, and nutrients

- To study phytoplankton and zooplankton diversity

- To assess aquatic flora and fauna

- To compare results with similar freshwater studies

- To suggest conservation strategies

2. MATERIALS AND METHODS

2.1 Study Area

The present investigation was conducted at Ghanpur (Mulug) Pond, located in Jayashankar Bhupalpally District, Telangana, India. The pond is an important freshwater ecosystem that supports irrigation, fisheries, groundwater recharge, livestock, and domestic activities of the surrounding villages. Seasonal rainfall, agricultural runoff, and local drainage influence the hydrological and ecological characteristics of the pond (Wetzel, 2001; Dodds & Whiles, 2020; Garai et al., 2025).

FIG 1. Study Area

2.2 Water Sampling

Water samples were collected monthly from three representative sampling stations during March 2025–February 2026, covering pre-monsoon, monsoon, and post-monsoon seasons. Samples were collected in pre-cleaned polyethylene bottles following standard limnological sampling procedures to minimize contamination and ensure analytical accuracy (APHA, 2017; ISO, 2023).

2.3 Physicochemical Analysis

Water temperature, pH, dissolved oxygen (DO), biochemical oxygen demand (BOD), chemical oxygen demand (COD), total dissolved solids (TDS), turbidity, alkalinity, hardness, chloride, nitrate, and phosphate were analyzed according to the Standard Methods for the Examination of Water and Wastewater (APHA, 2017). Instrument calibration, reagent blanks, duplicate analyses, and quality-control procedures were followed throughout the study (APHA, 2017; WHO, 2022).

2.4 Plankton Collection and Identification

Plankton samples were collected using a standard plankton net (64-µm mesh) and preserved in 4% formalin solution. Identification of phytoplankton and zooplankton was carried out under a compound microscope using standard taxonomic keys (Needham & Needham, 1962; Battish, 1992). Recent classifications and ecological interpretations were compared with contemporary studies on freshwater plankton diversity (Aravinth et al., 2023; Panikkar et al., 2024).

2.5 Biodiversity Indices

The diversity and community structure of phytoplankton and zooplankton were assessed using standard ecological diversity indices. Species abundance data collected during each season were used to calculate the Shannon–Wiener Diversity Index (H′), Simpson Diversity Index (1 − D), Margalef Richness Index (Dmg), and Pielou's Evenness Index (J′). These indices provide quantitative measures of species diversity, richness, dominance, and distribution within the plankton community.

The indices were calculated using the following equations:

Shannon–Wiener Diversity Index (H′)

H′ = −Σ (Pi × ln Pi)

where Pi = ni/N

Simpson Diversity Index

1 − D = 1 − Σ Pi²

Margalef Richness Index

Dmg = (S − 1) / ln N

Pielou's Evenness Index

J′ = H′ / ln S

where:

S = Total number of species

N = Total number of individuals

ni = Number of individuals of the ith species

Pi = Relative abundance of the ith species (ni/N)

These indices were calculated separately for each season using standard ecological methods to evaluate seasonal changes in plankton diversity and ecosystem health. (Sarkar et al., 2020; Garai et al., 2025).

2.6 Water Quality Index (WQI)

The overall water quality of Ghanpur (Mulug) Pond was evaluated using the Weighted Arithmetic Water Quality Index (WQI) method. This approach integrates multiple physicochemical parameters into a single numerical value, facilitating the assessment of seasonal water quality and comparison among freshwater ecosystems.

Equation (1): Water Quality Index

WQI = Σ(Qi × Wi) / ΣWi

where:

Qi = Quality rating of the ith water quality parameter

Wi = Unit weight assigned to the ith parameter

Equation (2): Quality Rating

Qi = [(Vi − Vo) / (Si − Vo)] × 100

where:

Vi = Measured value of the parameter

Vo = Ideal value of the parameter (0 for most parameters; 7.0 for pH and 14.6 mg/L for dissolved oxygen)

Si = Standard permissible value of the parameter according to WHO/APHA guidelines

2.7 Statistical Analysis

All physicochemical and biological data were expressed as mean ± standard deviation (Mean ± SD) based on monthly observations collected during each season. One-way Analysis of Variance (ANOVA) was performed to determine significant seasonal differences among the pre-monsoon, monsoon, and post-monsoon periods. When significant differences were detected (p < 0.05), Tukey's Honestly Significant Difference (HSD) post hoc test was applied to identify pairwise differences among seasons. Statistical analyses were performed using IBM SPSS Statistics version 26.0 (IBM Corp., Armonk, NY, USA), and graphical representations were prepared using Microsoft Excel 2021. Statistical significance was considered at p < 0.05.

3. RESULTS AND DISCUSSION

3.1 Seasonal Variation in Physicochemical Parameters

The seasonal variations in the physicochemical characteristics of Ghanpur (Mulug) Pond are presented in Table 1a & 1b The measured parameters exhibited considerable seasonal variation owing to differences in climatic conditions, rainfall, evaporation, and surface runoff. Water temperature was highest during the pre-monsoon season, whereas dissolved oxygen increased during the monsoon because of rainfall-induced aeration. Biochemical oxygen demand (BOD), chemical oxygen demand (COD), total dissolved solids (TDS), alkalinity, hardness, and chlorides were comparatively higher during the pre-monsoon season because of evaporation and concentration of dissolved substances. In contrast, nitrate and phosphate concentrations reached their maximum during the monsoon season owing to agricultural runoff.

One-way ANOVA indicated significant seasonal differences (p < 0.05) for most physicochemical parameters, including water temperature, dissolved oxygen, biochemical oxygen demand, chemical oxygen demand, turbidity, nitrate, and phosphate concentrations. Tukey's HSD test further demonstrated that the pre-monsoon season differed significantly from the monsoon and post-monsoon seasons for most parameters, indicating a pronounced seasonal influence on water quality.

Table 1a. Seasonal variation in physicochemical parameters of Ghanpur (Mulug) Pond.

| Parameter | Pre-Monsoon | Monsoon | Post-Monsoon | F -Values | P- Values |
| --- | --- | --- | --- | --- | --- |
| Temperature (°C) | 31.5 ± 0.8 | 26.2 ± 0.6 | 27.4 ± 0.5 | 18.52 | <0.001 |
| pH | 8.1 ± 0.2 | 7.2 ± 0.1 | 7.6 ± 0.2 | 9.84 | 0.004 |
| Dissolved Oxygen (mg/L) | 5.6 ± 0.3 | 7.4 ± 0.2 | 6.8 ± 0.3 | 14.36 | <0.001 |

Table 1b: Seasonal variation in physicochemical parameters of Ghanpur (Mulug) Pond.

| Parameter | Pre-Monsoon | Monsoon | Post-Monsoon | Seasonal Trend / Interpretation |
| --- | --- | --- | --- | --- |
| BOD (mg/L) | 8.4 | 4.2 | 5.3 | Highest in pre-monsoon indicating organic load |
| COD (mg/L) | 52 | 28 | 36 | Indicates chemical pollution, highest in summer |
| TDS (mg/L) | 365 | 215 | 275 | Concentrated during pre-monsoon due to evaporation |
| Turbidity (NTU) | 28 | 58 | 34 | Highest in monsoon due to runoff |
| Alkalinity (mg/L) | 168 | 122 | 145 | Higher in summer due to salt concentration |
| Hardness (mg/L) | 210 | 155 | 180 | Higher in pre-monsoon due to mineral concentration |
| Chlorides (mg/L) | 72 | 48 | 56 | Increased in summer due to evaporation |
| Nitrates (mg/L) | 1.2 | 2.4 | 1.6 | Highest in monsoon due to agricultural runoff |
| Phosphates (mg/L) | 0.7 | 1.5 | 0.9 | Indicates nutrient enrichment in monsoon |

3.2 Seasonal Relationship between Water Temperature and Dissolved Oxygen

The relationship between water temperature and dissolved oxygen during different seasons is summarized in Table 2. A clear inverse relationship was observed, with higher water temperature corresponding to lower dissolved oxygen concentration. Improved dissolved oxygen during the monsoon reflects enhanced atmospheric aeration and dilution effects.

Table 2. Seasonal relationship between water temperature and dissolved oxygen.

| Season | Temperature | DO | Ecological Interpretation |
| --- | --- | --- | --- |
| Pre-monsoon | High | Low | Oxygen stress due to high temperature |
| Monsoon | Low-Moderate | High | Better aeration and mixing |
| Post-monsoon | Moderate | Moderate | Stable ecological condition |

3.3 Seasonal Variation in Organic Pollution Indicators

The seasonal behaviour of the major organic pollution indicators is presented in Table 3. Both BOD and COD reached maximum values during the pre-monsoon season, indicating increased organic and chemical pollution. Reduced values during the monsoon season demonstrate the dilution effect of rainfall.

Table 3. Seasonal variation in organic pollution indicators (BOD and COD).

| Season | BOD (mg/L) | COD (mg/L) | Interpretation |
| --- | --- | --- | --- |
| Pre-monsoon | High | High | High organic and chemical pollution |
| Monsoon | Low | Low | Dilution of pollutants |
| Post-monsoon | Moderate | Moderate | Partial recovery |

3.4 Seasonal Variation in Nutrient Concentrations

The seasonal distribution of nitrate and phosphate concentrations is presented in Table 4. Both nutrients increased considerably during the monsoon season because of agricultural runoff and surface drainage, indicating nutrient enrichment of the pond.

Table 4. Seasonal variation in nutrient concentrations.

| Parameter | Pre-Monsoon | Monsoon | Post-Monsoon | Interpretation |
| --- | --- | --- | --- | --- |
| Nitrates | 1.2 | 2.4 | 1.6 | Peak during monsoon due to runoff |
| Phosphates | 0.7 | 1.5 | 0.9 | Indicates nutrient enrichment |

3.5 Seasonal Diversity of Phytoplankton

The composition and seasonal abundance of phytoplankton are presented in Table 5. Chlorophyceae dominated throughout the investigation, while Cyanophyceae were comparatively abundant during the pre-monsoon season, indicating nutrient enrichment.

Table 5. Seasonal diversity of phytoplankton.

| Group | Examples | Pre-Monsoon | Monsoon | Post-Monsoon | Ecological Significance |
| --- | --- | --- | --- | --- | --- |
| Chlorophyceae | Chlorella, Spirogyra | Moderate | High | High | Indicates productive water |
| Bacillariophyceae | Navicula, Diatoms | Low | Moderate | High | Indicator of good water quality |
| Cyanophyceae | Oscillatoria, Anabaena | High | Moderate | Low | Indicates eutrophic conditions |
| Euglenophyceae | Euglena | Moderate | Moderate | Low | Organic pollution indicator |

3.6 Seasonal Diversity of Zooplankton

The seasonal abundance of zooplankton is presented in Table 6. Rotifera remained dominant throughout the study, whereas Cladocera and Copepoda increased during the post-monsoon season, reflecting improved ecological conditions.

Table 6. Seasonal diversity of zooplankton.

| Group | Examples | Pre-Monsoon | Monsoon | Post-Monsoon | Ecological Significance |
| --- | --- | --- | --- | --- | --- |
| Rotifera | Brachionus | High | Moderate | Moderate | Pollution tolerant |
| Cladocera | Daphnia | Low | Moderate | High | Clean water indicator |
| Copepoda | Cyclops | Moderate | High | High | Stable ecosystem |
| Ostracoda | Cypris | Low | Moderate | Moderate | Bottom dwellers |

3.7 . The seasonal biodiversity indices

The seasonal biodiversity indices of plankton communities are presented in Table 7. The Shannon–Wiener Diversity Index (H′) increased from the pre-monsoon to the post-monsoon season, indicating a progressive increase in species diversity under favourable environmental conditions. Similarly, the Simpson Diversity Index (1 − D) showed higher values during the post-monsoon season, reflecting lower dominance and greater community stability.

Margalef's Richness Index indicated an increase in species richness following the monsoon period, while Pielou's Evenness Index demonstrated a more uniform distribution of plankton species during the post-monsoon season. These findings suggest that improved physicochemical conditions and enhanced nutrient balance following the monsoon promoted greater biological diversity and ecological stability in Ghanpur (Mulug) Pond.

Table 7. Seasonal Biodiversity Indices of Plankton Communities

| Season | Season Shannon Index (H′) | Simpson Index (1−D) | Margalef Richness | Pielou Evenness |
| --- | --- | --- | --- | --- |
| Pre-monsoon | 1.86 | 0.71 | 2.34 | 0.76 |
| Monsoon | 2.14 | 0.81 | 2.82 | 0.84 |
| Post-monsoon | 2.48 | 0.89 | 3.21 | 0.91 |

3.8 Seasonal Aquatic Faunal Diversity

The diversity of aquatic fauna recorded during the investigation is presented in Table 8. Fish, amphibians, aquatic insects, molluscs, and birds collectively indicate a productive freshwater ecosystem.

Table 8. Seasonal aquatic faunal diversity.

| Group | Examples | Seasonal Trend | Ecological Role |
| --- | --- | --- | --- |
| Fish | Catla, Rohu | Monsoon/Post Monsoon | Food chain |
| Amphibians | Frogs | Monsoon | Indicator species |
| Aquatic Insects | Beetles | Throughout the year | Nutrient cycling |
| Molluscs | Snails | Post-monsoon | Detritus feeders |
| Watter Birds | Kingfishers | Monsoon/Post | Top predators |

3.9 Seasonal Ecological Status

The overall ecological condition of the pond during different seasons is summarized in Table 9.

Table 9: Seasonal Ecological Status of Ghanpur (Mulug) Pond

| Season | Key Features | Ecological Status |
| --- | --- | --- |
| Pre-monsoon | High temp, Elevated BOD/COD, low DO | Pollution stress |
| Monsoon | High nutrients, increased turbidity, high DO | Highly productive but seasonally unstable Overall water quality remained good throughout the study |
| Post-monsoon | Moderate physiochemical parameters, high biodiversity | Stable ecosystem |

3.10. Water Quality Index Assessment

The seasonal Water Quality Index (WQI) values demonstrated that the overall water quality of Ghanpur (Mulug) Pond remained within the good category throughout the study period. The pre-monsoon season recorded the highest WQI value (42.8), reflecting comparatively lower water quality due to elevated temperature, increased biochemical oxygen demand (BOD), chemical oxygen demand (COD), total dissolved solids (TDS), and reduced dissolved oxygen concentration. These conditions indicate the concentration of dissolved pollutants resulting from high evaporation rates and limited water inflow.

During the monsoon season, the WQI decreased to 36.5, indicating an improvement in water quality. Increased rainfall enhanced dilution, atmospheric aeration, and water circulation, leading to higher dissolved oxygen concentrations and lower organic pollution. However, agricultural runoff contributed to elevated nitrate and phosphate concentrations, indicating moderate nutrient enrichment.

The post-monsoon season exhibited the lowest WQI value (29.4), representing the best overall water quality during the investigation. Moderate physicochemical conditions, improved dissolved oxygen levels, balanced nutrient concentrations, and enhanced plankton diversity reflected greater ecological stability and recovery following the monsoon period. These findings suggest that Ghanpur (Mulug) Pond maintained moderately good ecological health and remained suitable for sustaining freshwater biodiversity throughout the study.

Table 10. Seasonal Water Quality Index (WQI) of Ghanpur (Mulug) Pond

| Season | WQI | Water Quality Status |
| --- | --- | --- |
| Pre-monsoon | 42.8 | Good |
| Monsoon | 36.5 | Good |
| Post-monsoon | 29.4 | Good (Approaching Excellent) |

3.13. DISCUSSION

The observed seasonal fluctuations in water temperature, pH, dissolved oxygen (DO), biochemical oxygen demand (BOD), chemical oxygen demand (COD), total dissolved solids (TDS), and nutrient concentrations indicate the strong influence of climatic conditions, rainfall, and anthropogenic activities on the limnological characteristics of Ghanpur (Mulug) Pond. Similar seasonal variations have been reported in tropical freshwater ecosystems by Sarkar et al. (2020) and Garai et al. (2025).

The statistical analysis confirmed that seasonal variation significantly influenced the physicochemical characteristics of Ghanpur (Mulug) Pond. Significant differences detected by one-way ANOVA (p < 0.05) support the interpretation that climatic conditions, rainfall, and runoff are major factors controlling water quality and plankton dynamics. Similar statistically significant seasonal variations have been reported in other tropical freshwater ecosystems.

The comparatively lower dissolved oxygen values recorded during the pre-monsoon season may be attributed to elevated water temperature, increased microbial decomposition, and enhanced biological oxygen demand. The present DO values (5.6–7.4 mg/L) are comparable with those reported by Aravinth et al. (2023) (5.8–7.2 mg/L), but lower than Garai et al. (2025), possibly because of higher organic loading in Ghanpur Pond.

The higher BOD and COD values observed during the pre-monsoon season indicate increased organic and chemical pollution resulting from elevated evaporation, concentration of dissolved substances, and enhanced microbial activity. The subsequent reduction in BOD and COD during the monsoon season reflects the dilution effect of rainfall and increased water circulation, which improve overall water quality. Similar seasonal trends have been reported in tropical freshwater ecosystems, where climatic conditions strongly influence organic pollution levels.

Higher concentrations of nitrate and phosphate during the monsoon season indicate nutrient enrichment through agricultural runoff and surface drainage from the surrounding catchment. Excess nutrient loading accelerates primary productivity and may promote eutrophication under favourable environmental conditions (Smith et al., 1999).

The dominance of Chlorophyceae followed by Bacillariophyceae and Cyanophyceae suggests that the pond supports moderate nutrient availability and healthy primary productivity. Similar phytoplankton succession has been documented in freshwater ponds and reservoirs by Shetty and Gulimane (2023). The increased abundance of Bacillariophyceae during the post-monsoon season indicates improved ecological conditions, whereas higher Cyanophyceae abundance during the pre-monsoon season reflects comparatively nutrient-enriched conditions.

The zooplankton community was dominated by Rotifera, Cladocera, and Copepoda, indicating favourable ecological conditions and adequate food availability. Seasonal fluctuations in zooplankton diversity were closely associated with changes in physicochemical characteristics and phytoplankton abundance. Comparable findings have been reported by Battish (1992), Sharma and Sharma (2019), and Panikkar et al. (2024).

The biodiversity indices further supported the seasonal variation observed in plankton communities. Higher Shannon, Simpson, Margalef, and Pielou index values during the post-monsoon season indicate improved species diversity, richness, and community stability resulting from favourable environmental conditions. Conversely, comparatively lower diversity during the pre-monsoon season may be attributed to elevated temperature, reduced dissolved oxygen, and increased organic load. These findings are consistent with previous limnological studies reporting positive relationships between improved water quality and plankton diversity in tropical freshwater ecosystems.

The Water Quality Index (WQI) indicated that the pond water remained within acceptable quality limits during most of the study period, although slight deterioration was observed during the pre-monsoon season due to increased organic loading and reduced dilution capacity. Similar applications of WQI in freshwater ecosystem assessment have been reported for evaluating seasonal water quality and ecological status of freshwater ecosystems.

Overall, the findings demonstrate that Ghanpur (Mulug) Pond maintained satisfactory ecological integrity throughout the investigation, despite pronounced seasonal variations in physicochemical parameters and biological communities. Elevated nutrient concentrations and organic load during the pre-monsoon and monsoon seasons indicated moderate environmental stress; however, improved dissolved oxygen levels, balanced plankton succession, and enhanced aquatic biodiversity during the post-monsoon period reflected the ecosystem's resilience and natural recovery capacity. These findings suggest that the pond continues to function as a productive freshwater ecosystem, although sustained environmental monitoring and scientifically informed management strategies are essential to mitigate future anthropogenic pressures and ensure long-term ecological sustainability as recommended by the United Nations Environment Programme.

4. Conclusion

The study showed clear seasonal variation in the physicochemical and biological characteristics of Ghanpur (Mulug) Pond. Pre-monsoon conditions were marked by higher water temperature, biochemical oxygen demand, chemical oxygen demand, total dissolved solids, hardness, alkalinity and chlorides, together with lower dissolved oxygen. These conditions indicate seasonal concentration of dissolved substances and moderate organic stress. During the monsoon, rainfall and runoff improved dilution and aeration but also increased turbidity, nitrate and phosphate concentrations. Post-monsoon conditions were comparatively stable, with moderate temperature, improved dissolved oxygen and higher plankton diversity indices. Phytoplankton and zooplankton communities responded to these seasonal changes, suggesting their usefulness as biological indicators of pond health. The Water Quality Index indicated good water quality across the study period, with the best status in the post-monsoon season. Overall, Ghanpur Pond remains ecologically productive, but regular monitoring, control of agricultural runoff, prevention of waste discharge and protection of aquatic biodiversity are needed for long-term sustainability.

Limitations

This study was limited by incomplete quantitative presentation of biological data. Plankton abundance and aquatic faunal diversity were reported mainly through broad seasonal categories rather than detailed species-wise counts. The Water Quality Index and biodiversity indices were presented without full calculation tables, parameter weights, standards or raw abundance values. Sampling-station details also require clarification because the text and map differ in the number of stations. The study was conducted over one annual cycle, and longer-term monitoring would be needed to confirm trends. Future studies should include station-wise monthly data, full species lists, quantitative plankton density and detailed statistical analysis.

ABBREVIATIONS:

The following abbreviations are used throughout this manuscript: DO, Dissolved Oxygen; BOD, Biochemical Oxygen Demand; COD, Chemical Oxygen Demand; TDS, Total Dissolved Solids; WQI, Water Quality Index; ANOVA, Analysis of Variance; APHA, American Public Health Association; NTU, Nephelometric Turbidity Unit; and MSL, Mean Sea Level.

Declaration of AI Use

This manuscript was prepared through the combined contributions of all author(s), including contributions to the study design, data, content development, results, interpretation, and related scholarly work. The author(s) acknowledge the use of Grammarly and ChatGPT to assist with grammar checking, language refinement, reference formatting. These AI-assisted tools were not used as authors and did not replace the intellectual contributions or scholarly judgment of the author(s). All AI-assisted outputs, including content, references, and interpretations, were carefully reviewed, revised, verified, and approved by the author(s). The author(s) accept full responsibility for the accuracy, integrity, and final content of the manuscript.

5. Ethical Approval

For this limnological study, use Ethical approval was not required for this study as it involved environmental monitoring of freshwater ecosystems and did not include human participants or experimental animals.

COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

REFERENCES

- APHA. (2017). Standard methods for the examination of water and wastewater (23rd ed.). American Public Health Association.

- Battish, S. K. (1992). Freshwater zooplankton of India. Oxford & IBH Publishing.

- Central Pollution Control Board. (2019). Status of water quality in India. Ministry of Environment, Forest and Climate Change, Government of India.

- Dodds, W. K., & Whiles, M. R. (2020). Freshwater ecology: Concepts and environmental applications of limnology (3rd ed.). Academic Press.

- Kalff, J. (2002). Limnology: Inland water ecosystems. Prentice Hall.

- Dodds, W. K., & Smith, V. H. (2016). Nitrogen, phosphorus, and eutrophication in streams. Inland Waters, 6(2), 155–164. https://doi.org/10.5268/IW-6.2.909

- National Institute of Hydrology. (2023). Freshwater ecosystem monitoring report of India. National Institute of Hydrology, Roorkee, India.

- Reynolds, C. S. (2006). The ecology of phytoplankton. Cambridge University Press.

- United Nations Environment Programme. (2022). Global freshwater ecosystem assessment report. United Nations Environment Programme.Carlson, R. E. (1977). A trophic state index for lakes. Limnology and Oceanography, 22(2), 361–369. https://doi.org/10.4319/lo.1977.22.2.0361

- Sharma, A., & Sharma, M. (2019). Zooplankton diversity in relation to physico-chemical parameters in subtropical pond of Jammu, Jammu and Kashmir, India. Biosciences Biotechnology Research Asia, 16(2), 425–439. https://doi.org/10.13005/bbra/2758

- Reynolds, C. S. (2006). The ecology of phytoplankton. Cambridge University Press.

- Smith, V. H., Tilman, G. D., & Nekola, J. C. (1999). Eutrophication: Impacts of excess nutrient inputs on freshwater, marine, and terrestrial ecosystems. Environmental Pollution, 100(1–3), 179–196. https://doi.org/10.1016/S0269-7491(99)00091-3

- United Nations Environment Programme. (2022). Global freshwater ecosystem assessment report. United Nations Environment Programme.

- Wetzel, R. G. (2001). Limnology: Lake and river ecosystems (3rd ed.). Academic Press.

- Wetzel, R. G., & Likens, G. E. (2021). Limnological analyses (4th ed.). Springer Nature.

- World Health Organization. (2022). Guidelines for drinking-water quality (4th ed., incorporating the first and second addenda). World Health Organization.

- Carlson, R. E. (1977). A trophic state index for lakes. Limnology and Oceanography, 22(2), 361–369. https://doi.org/10.4319/lo.1977.22.2.0361

- Sarkar, R., Ghosh, A. R., & Mondal, N. K. (2020). Comparative study on physicochemical status and diversity of macrophytes and zooplanktons of two urban ponds of Chandannagar, West Bengal, India. Applied Water Science, 10, 63. https://doi.org/10.1007/s13201-020-1146-y

- Aravinth, A., Kannan, R., Chinnadurai, G., Manickam, N., Perumal, P., & Santhanam, P. (2023). Temporal changes in plankton diversity in relation to hydrographical characteristics at Perumal Lake, Cuddalore District, Tamil Nadu, India. The Journal of Basic and Applied Zoology, 84, 13. https://doi.org/10.1186/s41936-023-00337-7

- Panikkar, P., Khan, F., Ramya, V. L., Saha, A., Vijaykumar, M. E., Sarkar, U. K., Jesna, P. K., & Das, B. K. (2024). Assessment of plankton diversity and physico-chemical characteristics for sustainable fish production in a tropical reservoir in South India. Indian Journal of Fisheries, 71(3). https://doi.org/10.21077/ijf.2024.71.3.147246-05

- Shetty, K., & Gulimane, K. (2023). Application of microalgal diversity in assessing the water quality of freshwater ponds. Environmental Monitoring and Assessment, 195(5), 595. https://doi.org/10.1007/s10661-023-11116-w

- Garai, S., Mondal, A., Dey, I., Bose, R., & Pal, R. (2025). Decoding the influence of water quality and seasonal shifts on phytoplankton communities in Eastern Indian freshwater waterbodies. Limnology and Freshwater Biology, 2025(3), 284–297. https://doi.org/10.31951/2658-3518-2025-A-3-284

- International Organization for Standardization. (2023). ISO 5667-1:2023 Water quality—Sampling—Part 1: Guidance on the design of sampling programmes and sampling techniques. International Organization for Standardization.

- World Health Organization. (2022). Guidelines for drinking-water quality (4th ed., incorporating the first and second addenda). World Health Organization.

- Garai, S., Mondal, A., Dey, I., Bose, R., & Pal, R. (2025). Decoding the influence of water quality and seasonal shifts on phytoplankton communities in Eastern Indian freshwater waterbodies. Limnology and Freshwater Biology, 2025(3), 284–297.

- Panikkar, P., Khan, F., Ramya, V. L., Saha, A., Vijaykumar, M. E., Sarkar, U. K., Jesna, P. K., & Das, B. K. (2024). Assessment of plankton diversity and physico-chemical characteristics for sustainable fish production in a tropical reservoir in South India. Indian Journal of Fisheries, 71(3). https://doi.org/10.21077/ijf.2024.71.3.147246-05

- Ansari, A. A., Gill, S. S., Abbas, Z. K., Naeem, M., & others. (2023). Freshwater ecosystem health assessment using physicochemical and biological indicators. Environmental Monitoring and Assessment, 195, 1086.

- Aravinth, A., Kannan, R., Chinnadurai, G., Manickam, N., Perumal, P., & Santhanam, P. (2023). Temporal changes in plankton diversity in relation to hydrographical characteristics at Perumal Lake, Cuddalore District, Tamil Nadu, India. The Journal of Basic and Applied Zoology, 84, 13. https://doi.org/10.1186/s41936-023-00337-7

- Panikkar, P., Khan, F., Ramya, V. L., Saha, A., Vijaykumar, M. E., Sarkar, U. K., Jesna, P. K., & Das, B. K. (2024). Assessment of plankton diversity and physico-chemical characteristics for sustainable fish production in a tropical reservoir in South India. Indian Journal of Fisheries, 71(3). https://doi.org/10.21077/ijf.2024.71.3.147246-05

- Garai, S., Mondal, A., Dey, I., Bose, R., & Pal, R. (2025). Decoding the influence of water quality and seasonal shifts on phytoplankton communities in Eastern Indian freshwater waterbodies. Limnology and Freshwater Biology, 2025(3), 284–297.