Concentrations and Risk Assessment of Dioxins and Furans (PCDD/Fs) in Sediments of the Lower Reaches of River Ethiope, Delta State, Nigeria

# ABSTRACT

# Dioxins and furans (PCDD/Fs) are persistent contaminants that can accumulate in aquatic sediments and contribute to ecological and human exposure. This study assessed PCDD/F concentrations in sediments from the lower reaches of River Ethiope, Delta State, Nigeria, and evaluated the associated ecological and human health risks. Twenty sediment samples were collected from sampling points near petroleum-handling facilities. Sediment extracts were prepared by ultrasonic solvent extraction and analysed by gas chromatography–mass spectrometry. Total concentrations of the seventeen analysed PCDD/F congeners ranged from 9.53 to 57.5 µg kg⁻¹, with OCDD as the dominant dioxin and OCDF as the dominant furan. Computed ∑TEQ values ranged from 1.34 to 8.52 ng TEQ g⁻¹ and exceeded the Canadian sediment quality guideline applied in the study. Non-cancer risk estimates yielded hazard index values of 34.1–216.6 for children and 4.77–30.32 for adults. Total cancer risk values ranged from 3.38 × 10⁻⁴ to 2.15 × 10⁻³ for children and from 2.37 × 10⁻⁴ to 1.50 × 10⁻³ for adults. Children consistently had higher estimated risks than adults across the sampled locations. The findings indicate substantial PCDD/F contamination of the sampled sediments and support continued environmental monitoring, measures to limit contaminant inputs, and actions to reduce direct exposure to contaminated sediment in communities using the river.

# Key words: Dioxins and Furans, aquatic sediments, health risks, organic pollutants, environmental pressure, freshwater river, cancer risks, Inhalation exposure

# Introduction

Sediments function as important environmental reservoirs for persistent contaminants, including dioxins and furans (PCDD/Fs), thereby governing their environmental cycling and serving as pathways through which humans and other organisms are exposed. PCDD/Fs are not intentionally manufactured but arise as unwanted by-products of combustion and thermal processes, including metal smelting, cement production, coal-fired power generation, and the incineration of chlorine-containing waste (Themba et al., 2023; Meng et al., 2016). Because of their hydrophobicity and capacity to travel long distances in air and water, these compounds persist and disperse widely once released into the environment (Klees et al., 2015). Owing to this persistence and toxicity, PCDD/Fs were among the original twelve substances targeted for elimination under the 2001 Stockholm Convention on persistent organic pollutants, and they accumulate and increase in concentration as they move up the food chain (Baqar et al., 2017). Recent sediment studies have further documented PCDD/Fs and related dioxin-like compounds in aquatic sediments, with congener patterns being used to examine contamination distributions and potential sources (Chitsaz et al., 2025; Kannan et al., 2022; Liu et al., 2023).

The lower reaches of River Ethiope, in Delta State, Nigeria, form a vital waterway that supports fishing, farming, and the transport of petroleum products. The river is nonetheless under considerable environmental pressure: it periodically overflows its banks, carrying domestic and industrial waste into the surrounding floodplain during the rainy season. The lower reaches of the River Ethiope are flanked by nine different tank farms, which include Rain Oil, Cybernetics Oil and Gas, Dutchess Energy, Nepal Oil and Gas, Othinel Brooks Energy, Prudent Energy, Gas Black Light Energy, and Fredo Energy Limited, all of which handle petrol, diesel, and kerosene on a daily basis. Product transfer through riverbank jetties creates a recurring risk of leakage into both the water column and the underlying sediment. The area is further impacted by sand dredging, illegal fuel bunkering, artisanal crude oil refining, palm oil processing, and timber sawing, while runoff from adjoining farmland adds to the contaminant load entering the river. Together, these activities have raised concern among communities that depend on the river, given the potential for adverse effects on public health. Comparable studies in Nigerian aquatic systems have applied sediment-based ecological and human health risk frameworks to persistent organochlorine contaminants, supporting integrated evaluation of contaminant occurrence and exposure pathways in river environments (Akpan et al., 2026; Okechukwu & Onwukeme, 2025). Despite the range of petroleum-handling and other anthropogenic activities described along the lower reaches of River Ethiope, the combined evaluation of sediment PCDD/F concentrations, toxic equivalency, and associated ecological and human health risks remains insufficiently characterised for this study area. The present study therefore addresses this local evidence gap through an integrated assessment of PCDD/F contamination and risk in the river sediments. The objectives of the study were to determine the concentrations of the seventeen PCDD/F congeners in sediments collected from the lower reaches of River Ethiope, compute the toxic equivalency (TEQ) values of PCDD/Fs in the sediments and evaluate them against ecological guideline values recommended by the CCME and WHO, and assess the non-cancer and cancer health risks posed to children and adults through ingestion and dermal contact with PCDD/F-contaminated sediments in the study area.

# Materials and Methods

## Description of Study Area

This study was conducted in the Oghareki-Oghara community, within Ethiope West Local Government Area of Delta State, Nigeria (Latitude 5°57'2"N, Longitude 5°38'25"E). The community lies along the lower course of River Ethiope, a freshwater river that originates at Umuaja in Ukwuani Local Government Area and flows more than 50 kilometres, passing through Abraka, Mosogar, and Sapele before reaching Oghareki, where it joins the Benin River. Residents of the area depend heavily on the river for fishing, washing, bathing, swimming, and other domestic and occupational activities. Figure 1 is a map of the study area.

## Sample Collection

Twenty sediment samples were obtained from twenty sampling points positioned at distances of 100 metres to 1 kilometre from the tank farms, using an Ekman grab sampler. Each sample was wrapped in aluminium foil, sealed in a labelled zip-lock polyethylene bag, and stored in an ice-packed cooler for transport to the laboratory.

Figure 1: Map of the study area showing the sampling points

## Extraction of Sediment Samples for Dioxins and Furans

Sediment extracts for PCDD/F analysis were prepared following the ultrasonic solvent extraction procedure of US EPA Method 3550C (US EPA, 2007). A 10.0 g portion of each sediment sample was combined with an equal mass of anhydrous Na₂SO₄ and ultrasonically extracted at 30 °C for 30 minutes using 50 mL of a 1:1 (v/v) hexane/dichloromethane mixture. The filtered extract was subjected to two further rounds of sonication, each with a fresh acetone/dichloromethane mixture. Combined extracts were concentrated to 1 mL by rotary evaporation, and then cleaned up by solid-phase extraction on 2 g of partially deactivated (5%) aluminium oxide, eluting sequentially with 15 mL hexane, 15 mL of a 9:1 hexane/dichloromethane mixture, and 20 mL of a 4:1 hexane/dichloromethane mixture. The pooled eluate was concentrated under a gentle nitrogen stream to approximately 0.5 mL. PCBs, PBDEs, and PCDD/F congeners in the resulting extract were quantified using gas chromatography (Hewlett-Packard 6890, Palo Alto, CA, USA) coupled to a mass-spectrometric detector, with separation achieved on an HP-5 cross-linked phenyl-methyl siloxane capillary column (0.25 µm film thickness, 0.25 mm × 30 m). Helium carrier gas was delivered at a linear velocity of 30 cm/s. The oven temperature programme began at 200 °C and was ramped at 4 °C/min to a final temperature of 310 °C, while the injector was held at 250 °C, 200 °C, and 240 °C for the respective injections; sample introduction was by 2.0 µL splitless injection. Quantification relied on an external calibration curve constructed from PCDD/F standard solutions at five concentration levels.

## Quality Control/Assurance

Method performance was verified by monitoring recoveries of isotopically labelled PCDD/F surrogate standards. Previously analysed sediment samples were also fortified with PCDD/F standards at three concentration levels, each in triplicate, and re-analysed alongside the study samples using the same extraction and quantification procedure.

## Assessment of Human Health Risk from Exposure to PCDD/Fs in Sediments

Human health risk from sediment-associated PCDD/Fs was evaluated for three exposure routes － ingestion, dermal contact, and inhalation － using exposure models developed by the United States Environmental Protection Agency (USEPA). These models, previously applied to assess PAH exposure risk in soil, dust, and sediment (Yang et al., 2012; Tesi et al., 2016; Iwegbue & Obi, 2016; Odali et al., 2024), were adapted here for PCDD/F congeners.

## Non-Cancer and Cancer Risk Assessment from PCDD/Fs

Non-cancer and cancer risks from dermal contact and inhalation exposure were calculated using the standard USEPA equations below (USEPA, 1989; USEPA, 2009).

For non-cancer risk:

Hazard index (HI) = HQ=HQing + HQ dermal  		(1)

HQ = CDIncRfD                                                                              (2)

CDIing-nc =       C × IngR x EF x ED  BW x ATnc × 10-6                                      (3)

CDIdermal-nc = C x SA x AF x ABS x EF x ED BW x ATnc × 10-6 		            (4)

Cancer risk assessment:

ILCRing = C × IngR × EF × ED ×CF × SFOBW ×ATca				(5)

ILCRderm = C ×SA ×AF ×ABS ×EF ×ED ×CF ×SFO ×GIABSBW ×ATca		(6)

Where CDIing, CDIinh, and CDIderm are the chronic daily intakes for ingestion, inhalation, and dermal contact, respectively; ILCRing, ILCRinh, and ILCRderm are the incremental lifetime cancer risks for ingestion, inhalation, and dermal contact, respectively. ILCR values were computed for each exposure route using the parameters in Tables 1 and 2. The WHO 2005 TEFs listed in Table 2 follow Van den Berg et al. (2006).

Table 1: Values of variables used for estimation of human health risk

| Variable | Unit | Definition | Child | Adult | Reference |
| --- | --- | --- | --- | --- | --- |
| C | mg/kg | PCDD/F toxic-equivalency (TEQ) concentration in sediments | - | - | Table 4 |
| ABS | - | Dermal absorption factor (default for chlorinated POPs; no TCDD-specific value available) | 0.14 | 0.14 | USEPA, 2004 (RAGS Part E) |
| AF | mg/cm² | Sediment-to-skin adherence factor | 0.2 | 0.07 | USEPA, 2011 |
| BW | kg | Average body weight | 15 | 60 | Iwegbue et al. (2015) |
| ED | year | Exposure duration | 6 | 30 | USEPA, 2001 |
| EF | day/yr | Exposure frequency | 350 | 350 | USEPA, 2001 |
| IngR | mg/day | Sediment ingestion rate | 200 | 100 | USDOE, 2011 |
| SA | cm²/event | Skin surface area | 2800 | 5700 | USDOE, 2011 |
| ATnc | day | Averaging time (non-carcinogenic) = ED × 365 | 2190 | 10950 | USDOE, 2011 |
| ATca | day | Averaging time (carcinogenic) = LT × 365 | 20075 | 20075 | USDOE, 2011 |
| LT | year | Lifetime | 55 | 55 | WHO, 2018 |
| CF | kg/mg | Mass-unit conversion factor | 1 × 10⁻⁶ | 1 × 10⁻⁶ | USEPA, 1989 |
| GIABS | - | Gastrointestinal absorption factor (default; no TCDD-specific value) | 1 | 1 | USEPA, 2004 (RAGS Part E) |
| RfDo | mg/kg/d | Oral reference dose for 2,3,7,8-TCDD, applied to ∑TEQ | 7 × 10⁻¹⁰ | 7 × 10⁻¹⁰ | USEPA IRIS, 2012 |
| SFO | (mg/kg/d)⁻¹ | Oral cancer slope factor for 2,3,7,8-TCDD, applied to ∑TEQ | 1.3 × 10⁵ | 1.3 × 10⁵ | CalEPA, 1986/2002; USEPA RSL |

Table 2: Toxicological parameters of the investigated PCDD/F congeners used for health risk assessment

| Congener | WHO (2005) TEF |
| --- | --- |
| 2,3,7,8-TCDD | 1 |
| 1,2,3,7,8-PCDD | 1 |
| 1,2,3,4,7,8-HxCDD | 0.1 |
| 1,2,3,6,7,8-HxCDD | 0.1 |
| 1,2,3,7,8,9-HxCDD | 0.1 |
| 1,2,3,4,6,7,8-HpCDD | 0.01 |
| OCDD | 0.0003 |
| 2,3,7,8-TCDF | 0.1 |
| 1,2,3,7,8-PCDF | 0.03 |
| 2,3,4,7,8-PCDF | 0.3 |
| 1,2,3,4,7,8-HxCDF | 0.1 |
| 1,2,3,6,7,8-HxCDF | 0.1 |
| 1,2,3,7,8,9-HxCDF | 0.1 |
| 2,3,4,6,7,8-HxCDF | 0.1 |
| 1,2,3,4,6,7,8-HpCDF | 0.01 |
| 1,2,3,4,7,8,9-HpCDF | 0.01 |
| OCDF | 0.0003 |

# Results and Discussion

## Concentrations of Dioxins and Furans (PCDD/Fs) in Sediments

Concentrations of the seventeen analysed PCDD/F congeners are presented in Table 3 and Figure 2. Total concentrations varied significantly among sampling points (p < 0.05), likely reflecting differences in the strength and history of individual contamination sources as well as post-depositional transformation. Values ranged from 9.53 to 57.5 µg/kg, with the highest total recorded at sampling point 5 and the lowest at point 18. Across the congener profile, OCDD was the dominant dioxin and OCDF the dominant furan. These totals considerably exceed the 253.9–344.2 ng/kg reported for river sediment near a steel-recycling village in northern Vietnam (Nguyen et al., 2024), the 500–5000 ng/kg range reported for lake and reservoir sediments in the UK (Rose & McKay, 1996), and the mean values of 335 and 941 ng/kg reported for urban sediments elsewhere (Loyola-Sepúlveda et al., 2018).

Table 3: Summary statistics of dioxins and furans (µg/kg) in sediments of the lower reaches of River Ethiope, Delta State

| Congener | Mean | SD | Median | Min | Max |
| --- | --- | --- | --- | --- | --- |
| 2,3,7,8-TCDF | 0.93 | 0.96 | 0.48 | 0.30 | 3.90 |
| 2,3,7,8-TCDD | 0.93 | 0.58 | 0.72 | 0.34 | 2.44 |
| 1,2,3,7,8-PCDF | 1.14 | 0.97 | 1.00 | 0.12 | 4.30 |
| 2,3,4,7,8-PCDF | 1.03 | 0.88 | 0.83 | 0.03 | 3.43 |
| 1,2,3,7,8-PCDD | 1.30 | 1.11 | 0.73 | 0.22 | 3.94 |
| 1,2,3,4,7,8-HxCDF | 1.21 | 0.91 | 1.39 | 0.07 | 3.52 |
| 1,2,3,6,7,8-HxCDF | 1.61 | 1.53 | 1.41 | 0.16 | 6.31 |
| 2,3,4,6,7,8-HxCDF | 1.70 | 1.58 | 1.50 | 0.14 | 6.58 |
| 1,2,3,4,7,8-HxCDD | 2.04 | 2.05 | 1.27 | 0.22 | 7.07 |
| 1,2,3,6,7,8-HxCDD | 2.00 | 1.46 | 1.92 | 0.23 | 5.66 |
| 1,2,3,7,8,9-HxCDD | 1.21 | 0.68 | 1.03 | 0.39 | 2.49 |
| 1,2,3,7,8,9-HxCDF | 1.74 | 1.32 | 1.30 | 0.45 | 4.56 |
| 1,2,3,4,6,7,8-HpCDF | 2.01 | 1.66 | 1.84 | 0.32 | 6.20 |
| 1,2,3,4,6,7,8-HpCDD | 2.63 | 1.70 | 2.05 | 0.70 | 5.97 |
| 1,2,3,4,7,8,9-HpCDF | 2.03 | 1.29 | 1.51 | 0.56 | 4.76 |
| OCDD | 2.57 | 1.71 | 1.77 | 0.85 | 6.23 |
| OCDF | 2.29 | 1.94 | 1.47 | 0.45 | 7.45 |
| Total | 28.4 | 17.7 | 21.0 | 9.53 | 57.5 |

Fig 2: Concentrations in ug/kg of PCCD/Fs in sediments of the lower reaches of River Ethiope

## Computed TEQ for PCDD/Fs in Sediments

Table 4 presents the computed TEQ values for the PCDD/F congeners in the analysed sediments. ∑TEQ values ranged from 1.34 to 8.52 ng TEQ g⁻¹, with 2,3,7,8-TCDD and 1,2,3,7,8-PCDD contributing most strongly to the total TEQ at most sampling points. These values exceed both the 21.5 pg TEQ g⁻¹ guideline of the Canadian Council of Ministers of the Environment (CCME, 2001) and the 20 ng WHO-TEQ kg⁻¹ benchmark of the World Health Organization, indicating a high potential ecological risk to organisms exposed to PCDD/F-contaminated sediment in the lower reaches of River Ethiope.

## Non-Cancer Risk of PCDD/Fs in Sediments

Non-cancer risk was estimated by applying the verified USEPA-IRIS oral reference dose for 2,3,7,8-TCDD (7 × 10⁻¹⁰ mg/kg-day; USEPA, 2012) to the ∑TEQ concentration at each sampling point, following the standard toxic-equivalency approach for dioxin-like compounds, since only 2,3,7,8-TCDD has an established dose–response value. Table 5 summarises the resulting non-cancer risk estimates. Hazard Index (HI) values ranged from 34.1 to 216.6 for children and 4.8 to 30.3 for adults, with children consistently showing higher HI values than adults — a pattern attributable to children's greater hand-to-mouth activity and lower body weight relative to adults. Because every HI value exceeds the threshold of 1, both child and adult populations face an elevated non-carcinogenic health risk from exposure to PCDD/F-contaminated sediment, with children at appreciably higher risk than adults.

| Table 4: Computed TEQ (ng TEQ g-1) for dioxins and furans in sediments of Lower reaches of River Ethiope |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| 2,3,7,8-TCDF | 5E-02 | 1E-01 | 4E-01 | 3E-01 | 2E-01 | 8E-02 | 7E-02 | 4E-02 | 4E-02 | 4E-02 | 5E-02 | 5E-02 | 3E-02 | 3E-02 | 4E-02 | 4E-02 | 5E-02 | 4E-02 | 7E-02 | 2E-01 |
| 2,3,7,8-TCDD | 2.44 | 7E-01 | 1.27 | 1.72 | 1.78 | 5E-01 | 1.68 | 5E-01 | 3E-01 | 1.05 | 1.28 | 6E-01 | 8E-01 | 5E-01 | 7E-01 | 5E-01 | 7E-01 | 3E-01 | 6E-01 | 5E-01 |
| 1,2,3,7,8-PCDF | 5E-02 | 6E-02 | 5E-02 | 1E-01 | 6E-02 | 3E-02 | 6E-02 | 2E-02 | 3E-02 | 5E-03 | 3E-02 | 4E-02 | 3E-02 | 4E-02 | 5E-03 | 4E-03 | 4E-03 | 4E-03 | 3E-02 | 2E-02 |
| 2,3,4,7,8-PCDF | 6E-01 | 5E-01 | 3E-01 | 1E+00 | 6E-01 | 3E-01 | 5E-01 | 5E-01 | 2E-01 | 8E-03 | 1E-02 | 2E-01 | 2E-01 | 4E-01 | 1E-01 | 1E-01 | 1E-02 | 2E-02 | 1E-01 | 3E-01 |
| 1,2,3,7,8-PCDD | 1.87 | 2.96 | 2.22 | 2.35 | 3.94 | 2.32 | 2.33 | 5E-01 | 5E-01 | 2E-01 | 4E-01 | 3E-01 | 1.54 | 1.82 | 0.96 | 4E-01 | 2E-01 | 5E-01 | 3E-01 | 3E-01 |
| 1,2,3,4,7,8-HxCDF | 4E-01 | 2E-01 | 2E-01 | 2E-01 | 1E-01 | 2E-01 | 1E-01 | 1E-01 | 3E-02 | 7E-03 | 7E-02 | 2E-01 | 2E-01 | 1E-01 | 1E-01 | 7E-02 | 1E-02 | 3E-02 | 1E-02 | 2E-02 |
| 1,2,3,6,7,8-HxCDF | 2E-01 | 6E-01 | 3E-01 | 2E-01 | 5E-02 | 9E-02 | 4E-01 | 2E-01 | 1E-01 | 2E-01 | 4E-02 | 1E-01 | 5E-02 | 2E-01 | 5E-02 | 8E-02 | 4E-02 | 1E-01 | 2E-02 | 3E-02 |
| 1,2,3,4,7,8-HxCDD | 2E-01 | 4E-01 | 4E-01 | 5E-01 | 5E-01 | 7E-01 | 3E-01 | 4E-02 | 4E-02 | 2E-01 | 3E-02 | 5E-02 | 3E-01 | 2E-01 | 3E-02 | 9E-02 | 2E-02 | 2E-02 | 3E-02 | 4E-02 |
| 1,2,3,6,7,8-HxCDD | 2E-01 | 4E-01 | 6E-01 | 3E-01 | 4E-01 | 2E-01 | 2E-01 | 2E-01 | 2E-01 | 2E-02 | 2E-01 | 2E-01 | 1E-01 | 1E-01 | 2E-01 | 2E-02 | 4E-02 | 1E-01 | 6E-02 | 4E-02 |
| 1,2,3,7,8,9-HxCDD | 7E-02 | 2E-01 | 2E-01 | 2E-01 | 2E-01 | 2E-01 | 2E-01 | 4E-02 | 1E-01 | 1E-01 | 9E-02 | 5E-02 | 1E-01 | 1E-01 | 5E-02 | 4E-02 | 9E-02 | 4E-02 | 1E-01 | 1E-01 |
| 1,2,3,7,8,9-HxCDF | 3E-01 | 3E-01 | 4E-01 | 4E-01 | 5E-01 | 1E-01 | 2E-01 | 5E-02 | 6E-02 | 2E-01 | 2E-01 | 1E-01 | 2E-01 | 5E-02 | 6E-02 | 1E-01 | 9E-02 | 6E-02 | 6E-02 | 5E-02 |
| 1,2,3,4,6,7,8-HpCDF | 6E-02 | 6E-02 | 3E-02 | 2E-02 | 2E-02 | 3E-02 | 2E-02 | 3E-02 | 1E-02 | 5E-03 | 1E-02 | 2E-02 | 2E-02 | 2E-02 | 2E-02 | 3E-03 | 5E-03 | 3E-03 | 5E-03 | 9E-03 |
| 1,2,3,4,6,7,8-HpCDD | 5E-02 | 5E-02 | 6E-02 | 5E-02 | 5E-02 | 2E-02 | 4E-02 | 2E-02 | 3E-02 | 3E-02 | 1E-02 | 3E-02 | 9E-03 | 2E-02 | 1E-02 | 2E-02 | 8E-03 | 7E-03 | 1E-02 | 1E-02 |
| 1,2,3,4,7,8,9-HpCDF | 4E-02 | 3E-02 | 3E-02 | 4E-02 | 3E-02 | 2E-02 | 5E-02 | 1E-02 | 1E-02 | 6E-03 | 1E-02 | 1E-02 | 1E-02 | 3E-02 | 9E-03 | 2E-02 | 2E-02 | 7E-03 | 1E-02 | 6E-03 |
| OCDD | 5E-02 | 3E-02 | 5E-02 | 6E-02 | 4E-02 | 5E-02 | 3E-02 | 3E-02 | 1E-02 | 2E-02 | 1E-02 | 1E-02 | 1E-02 | 2E-02 | 3E-02 | 9E-03 | 2E-02 | 9E-03 | 1E-02 | 1E-02 |
| ∑TEQ | 6.53 | 6.70 | 6.49 | 7.46 | 8.52 | 4.94 | 6.29 | 2.27 | 1.81 | 2.14 | 2.45 | 2.05 | 3.69 | 3.63 | 2.46 | 1.47 | 1.36 | 1.34 | 1.46 | 1.57 |

Table 5: Non-cancer risk of PCDD/Fs in sediments

| Point | Child HQing | Child HQderm | Child HI | Adult HQing | Adult HQderm | Adult HI |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 119.3 | 46.8 | 166 | 14.91 | 8.33 | 23.24 |
| 2 | 122.4 | 48 | 170.3 | 15.3 | 8.54 | 23.84 |
| 3 | 118.5 | 46.5 | 165 | 14.82 | 8.28 | 23.09 |
| 4 | 136.3 | 53.4 | 189.7 | 17.03 | 9.51 | 26.55 |
| 5 | 155.6 | 61 | 216.6 | 19.45 | 10.87 | 30.32 |
| 6 | 90.2 | 35.4 | 125.6 | 11.28 | 6.3 | 17.58 |
| 7 | 114.9 | 45 | 159.9 | 14.36 | 8.02 | 22.38 |
| 8 | 41.5 | 16.3 | 57.7 | 5.18 | 2.9 | 8.08 |
| 9 | 33.1 | 13 | 46 | 4.13 | 2.31 | 6.44 |
| 10 | 39.1 | 15.3 | 54.4 | 4.89 | 2.73 | 7.62 |
| 11 | 44.7 | 17.5 | 62.3 | 5.59 | 3.12 | 8.72 |
| 12 | 37.4 | 14.7 | 52.1 | 4.68 | 2.61 | 7.29 |
| 13 | 67.4 | 26.4 | 93.8 | 8.42 | 4.71 | 13.13 |
| 14 | 66.3 | 26 | 92.3 | 8.29 | 4.63 | 12.92 |
| 15 | 44.9 | 17.6 | 62.5 | 5.62 | 3.14 | 8.75 |
| 16 | 26.8 | 10.5 | 37.4 | 3.36 | 1.87 | 5.23 |
| 17 | 24.8 | 9.7 | 34.6 | 3.11 | 1.73 | 4.84 |
| 18 | 24.5 | 9.6 | 34.1 | 3.06 | 1.71 | 4.77 |
| 19 | 26.7 | 10.5 | 37.1 | 3.33 | 1.86 | 5.2 |
| 20 | 28.7 | 11.2 | 39.9 | 3.58 | 2 | 5.59 |

## Total Cancer Risk of PCDD/Fs in Sediments

Cancer risk was similarly estimated by applying an oral cancer slope factor of 1.3 × 10⁵ (mg/kg-day)⁻¹ for 2,3,7,8-TCDD to the ∑TEQ concentration at each point. This slope factor is not an IRIS value (since EPA's IRIS carcinogenicity assessment for TCDD remains unresolved for oral exposure) but is the CalEPA (2002)-derived value that USEPA's own Regional Screening Level tables adopt as the standard default in the absence of an IRIS figure (California OEHHA, 2009; USEPA, 2011). Table 6 shows the resulting cancer risk estimates. Total cancer risk (TCR) values ranged from 3.4 × 10⁻⁴ to 2.2 × 10⁻³ for children and 2.4 × 10⁻⁴ to 1.5 × 10⁻³ for adults, again with higher values recorded for children. All TCR values exceed the 10⁻⁶ acceptable risk threshold, by roughly two to three orders of magnitude, across every sampled location, indicating a substantial carcinogenic risk associated with human exposure to PCDD/Fs in the sediments of River Ethiope.

Table 6: Cancer risk of PCDD/Fs in sediments

| Point | Child ILCRing | Child ILCRderm | Child TCR | Adult ILCRing | Adult ILCRderm | Adult TCR |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.18E-03 | 4.64E-04 | 1.65E-03 | 7.40E-04 | 4.13E-04 | 1.15E-03 |
| 2 | 1.21E-03 | 4.76E-04 | 1.69E-03 | 7.59E-04 | 4.24E-04 | 1.18E-03 |
| 3 | 1.18E-03 | 4.61E-04 | 1.64E-03 | 7.35E-04 | 4.11E-04 | 1.15E-03 |
| 4 | 1.35E-03 | 5.30E-04 | 1.88E-03 | 8.45E-04 | 4.72E-04 | 1.32E-03 |
| 5 | 1.54E-03 | 6.06E-04 | 2.15E-03 | 9.66E-04 | 5.39E-04 | 1.50E-03 |
| 6 | 8.96E-04 | 3.51E-04 | 1.25E-03 | 5.60E-04 | 3.13E-04 | 8.73E-04 |
| 7 | 1.14E-03 | 4.47E-04 | 1.59E-03 | 7.13E-04 | 3.98E-04 | 1.11E-03 |
| 8 | 4.12E-04 | 1.61E-04 | 5.73E-04 | 2.57E-04 | 1.44E-04 | 4.01E-04 |
| 9 | 3.28E-04 | 1.29E-04 | 4.57E-04 | 2.05E-04 | 1.15E-04 | 3.20E-04 |
| 10 | 3.88E-04 | 1.52E-04 | 5.40E-04 | 2.43E-04 | 1.35E-04 | 3.78E-04 |
| 11 | 4.44E-04 | 1.74E-04 | 6.18E-04 | 2.78E-04 | 1.55E-04 | 4.33E-04 |
| 12 | 3.72E-04 | 1.46E-04 | 5.17E-04 | 2.32E-04 | 1.30E-04 | 3.62E-04 |
| 13 | 6.69E-04 | 2.62E-04 | 9.31E-04 | 4.18E-04 | 2.34E-04 | 6.52E-04 |
| 14 | 6.58E-04 | 2.58E-04 | 9.16E-04 | 4.11E-04 | 2.30E-04 | 6.41E-04 |
| 15 | 4.46E-04 | 1.75E-04 | 6.21E-04 | 2.79E-04 | 1.56E-04 | 4.35E-04 |
| 16 | 2.67E-04 | 1.04E-04 | 3.71E-04 | 1.67E-04 | 9.31E-05 | 2.60E-04 |
| 17 | 2.47E-04 | 9.67E-05 | 3.43E-04 | 1.54E-04 | 8.61E-05 | 2.40E-04 |
| 18 | 2.43E-04 | 9.52E-05 | 3.38E-04 | 1.52E-04 | 8.48E-05 | 2.37E-04 |
| 19 | 2.65E-04 | 1.04E-04 | 3.69E-04 | 1.65E-04 | 9.24E-05 | 2.58E-04 |
| 20 | 2.85E-04 | 1.12E-04 | 3.96E-04 | 1.78E-04 | 9.94E-05 | 2.77E-04 |

# Conclusion

This study shows that sediments in the lower reaches of River Ethiope carry appreciable concentrations of PCDD/Fs, with total congener levels and TEQ values that exceed international ecological and health-based guidelines. The predominance of OCDD and OCDF, together with elevated hazard index and total cancer risk values for both children and adults, indicates that residents relying on the river for fishing, domestic use, and recreation face a non-negligible risk of adverse health outcomes from ingestion and dermal contact with contaminated sediment, with children consistently at greater risk than adults across all measures evaluated. Given the density of petroleum tank farms and associated activities, including dredging, illegal bunkering, and artisanal refining along this stretch of the river, these findings point to industrial and informal petroleum-handling activities as likely contributors to sediment contamination. Continued monitoring, stricter enforcement of environmental safeguards around tank-farm operations, and public health measures to reduce direct sediment contact are recommended to mitigate these risks. Future work should incorporate source apportionment analysis and a seasonally varied sampling programme to better characterise temporal variability in PCDD/F loading in the river system.

Limitations

The assessment represents conditions at the sampled locations and does not resolve seasonal variability or the relative contributions of individual contaminant sources. Accordingly, the findings should be interpreted as location-specific evidence for the lower reaches of River Ethiope. Future studies should incorporate seasonally distributed sampling and source-apportionment analysis to clarify temporal patterns and contributions from multiple activities along the river. The health-risk estimates should also be interpreted within the exposure assumptions and toxicological parameters used in the assessment.

Declaration of AI Use

This manuscript was prepared through the intellectual contributions of the author(s) to the study design, data analysis, interpretation of results, and scholarly content. Grammarly and ChatGPT were used solely to assist with grammatical refinement and reference formatting during manuscript revision. The author(s) accept full responsibility for the accuracy, integrity, and final content of the manuscript.

COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

# References

Akpan, N. A., Udombeh, R. B., Ekanem, J. O., Ukpong, M. B., & Ntukuyo, A. I. (2026). Health risk reduction of organochlorine pesticides (OCPs) via source contamination identification and control in water, sediment and fish in Ikot Ekpene River, Nigeria. Asian Journal of Geological Research, 9(1), 40–50. https://doi.org/10.9734/ajoger/2026/v9i1226

Baqar, M., Sadef, Y., Ahmad, S. R., Mahmood, A., Qadir, A., Aslam, I., Li, J., & Zhang, G. (2017). Occurrence, ecological risk assessment, and spatio-temporal variation of polychlorinated biphenyls (PCBs) in water and sediments along River Ravi and its northern tributaries, Pakistan. Environmental Science and Pollution Research, 24(36), 27913–27930. https://doi.org/10.1007/s11356-017-0182-0

California Department of Health Services. (1986). Report on chlorinated dioxins and dibenzofurans: Part B. Health effects of chlorinated dioxins and dibenzofurans. California Air Resources Board. https://ww2.arb.ca.gov/sites/default/files/classic/toxics/id/summary/dioxptb.pdf

California Environmental Protection Agency, Office of Environmental Health Hazard Assessment. (2002). Air Toxics “Hot Spots” Program risk assessment guidelines, Part II: Technical support document for describing available cancer potency factors. https://oehha.ca.gov/air/crnr/draft-air-toxics-hot-spots-program-risk-assessment-guidelines-part-ii-2002

California Environmental Protection Agency, Office of Environmental Health Hazard Assessment. (2009). Technical support document for cancer potency factors 2009. https://oehha.ca.gov/air/crnr/technical-support-document-cancer-potency-factors-2009

Canadian Council of Ministers of the Environment. (2001). Polychlorinated dibenzo-p-dioxins and polychlorinated dibenzofurans (PCDD/Fs): Canadian sediment quality guidelines for the protection of aquatic life. In Canadian environmental quality guidelines. https://ccme.ca/en/res/polychlorinated-dioxins-and-furans-pcdd_fs-canadian-sediment-quality-guidelines-for-the-protection-of-aquatic-life-en.pdf

Chitsaz, M., Al Hello, M., Burris, D. R., Francisco, K. L., & Rodenburg, L. A. (2025). Sources of polychlorinated dibenzo-p-dioxins and -furans to sediment in the Newtown Creek Superfund Site. Science of the Total Environment, 958, 177771. https://doi.org/10.1016/j.scitotenv.2024.177771

Nguyen, H. X., Nguyen, X. T., Mai, H. T. H., Nguyen, H. T., Vu, N. D., Pham, T. T. P., Nguyen, T. Q., Nguyen, D. T., Duong, N. T., Hoang, T. A. L., Nguyen, T. N., Le, N. V., Dao, H. V., Truong, N. M., & Bui, M. Q. (2024). A comprehensive evaluation of dioxins and furans occurrence in river sediments from a secondary steel recycling craft village in Northern Vietnam. Molecules, 29(8), 1788. https://doi.org/10.3390/molecules29081788

Iwegbue, C. M. A., Bassey, F. I., Tesi, G. O., Overah, L. C., Onyeloni, S. O., & Martincigh, B. S. (2015). Concentrations and health risk assessment of metals in chewing gums, peppermints and sweets in Nigeria. Journal of Food Measurement and Characterization, 9(2), 160–174. https://doi.org/10.1007/s11694-014-9221-4

Iwegbue, C. M. A., & Obi, G. (2016). Distribution, sources, and health risk assessment of polycyclic aromatic hydrocarbons in dust from urban environment in the Niger Delta, Nigeria. Human and Ecological Risk Assessment: An International Journal, 22(3), 623–638. https://doi.org/10.1080/10807039.2015.1100157

Kannan, V. M., Gopikrishna, V. G., Saritha, V. K., Krishnan, K. P., & Mohan, M. (2022). PCDD/Fs, dioxin-like, and non-dioxin like PCBs in the sediments of high Arctic fjords, Svalbard. Marine Pollution Bulletin, 174, 113277. https://doi.org/10.1016/j.marpolbul.2021.113277

Klees, M., Hiester, E., Brückmann, P., Molt, K., & Schmidt, T. C. (2015). Polychlorinated biphenyls, polychlorinated dibenzo-p-dioxins and dibenzofurans in street dust of North Rhine-Westphalia, Germany. Science of the Total Environment, 511, 72–81. https://doi.org/10.1016/j.scitotenv.2014.12.018

Liu, M., Li, H., Song, A., Peng, P., Liu, H., Hu, J., Sheng, G., & Ying, G. (2023). Polybrominated dibenzo-p-dioxins/furans and their chlorinated analogues in sediments from a historical hotspot for both brominated flame retardants and organochlorine pesticides. Environmental Pollution, 316, 120489. https://doi.org/10.1016/j.envpol.2022.120489

Meng, G., Nie, Z., Feng, Y., Wu, X., Yin, Y., & Wang, Y. (2016). Typical halogenated persistent organic pollutants in indoor dust and the associations with childhood asthma in Shanghai, China. Environmental Pollution, 211, 389–398. https://doi.org/10.1016/j.envpol.2015.12.006

Odali, E. W., Iwegbue, C. M. A., Egobueze, F. E., Nwajei, G. E., & Martincigh, B. S. (2024). Distribution, sources, and risk of polycyclic aromatic hydrocarbons in soils from rural communities around gas flaring points in the Niger Delta of Nigeria. Environmental Science: Processes & Impacts, 26(4), 721–733. https://doi.org/10.1039/D3EM00067B

Okechukwu, V. U., & Onwukeme, V. I. (2025). Ecological and health risk assessment of heavy metals and organochlorine pesticides in water and sediments from Otamiri River in Imo State, Nigeria. Journal of Global Ecology and Environment, 21(1), 29–45. https://doi.org/10.56557/jogee/2025/v21i19080

Loyola-Sepúlveda, R., Salamanca, M. O., Gutiérrez-Baeza, F., Figueroa Sn. M., C. M., Chandia, C. V., Bravo-Linares, C., & Mudge, S. M. (2018). Contributions of dioxins and furans to the urban sediment signature: The role of atmospheric particles. Science of the Total Environment, 615, 751–760. https://doi.org/10.1016/j.scitotenv.2017.09.218

Rose, C. L., & McKay, W. A. (1996). PCDDs (dioxins) and PCDFs (furans) in selected UK lake and reservoir sites—Concentrations and TEQs in sediment and fish samples. Science of the Total Environment, 177(1–3), 43–56. https://doi.org/10.1016/0048-9697(95)04858-8

Tesi, G. O., Iwegbue, C. M. A., Emuh, F. N., & Nwajei, G. E. (2016). Lagdo Dam flood disaster of 2012: An assessment of the concentrations, sources, and risks of PAHs in floodplain soils of the lower reaches of River Niger, Nigeria. Journal of Environmental Quality, 45(1), 305–314. https://doi.org/10.2134/jeq2015.02.0116

Themba, N., Sibali, L. L., & Chokwe, T. B. (2023). A review on the formation and remediations of polychlorinated dibenzo-p-dioxins and dibenzofurans (PCDD/Fs) during thermal processes with a focus on MSW process. Air Quality, Atmosphere & Health, 16, 2115–2132. https://doi.org/10.1007/s11869-023-01394-1

U.S. Department of Energy. (2011). The Risk Assessment Information System (RAIS). Oak Ridge Operations Office. https://rais.ornl.gov/

U.S. Environmental Protection Agency. (1989). Risk assessment guidance for Superfund, Volume I: Human health evaluation manual (Part A) (EPA/540/1-89/002). Office of Emergency and Remedial Response. https://archive.epa.gov/region9/superfund/web/pdf/hhema.pdf

U.S. Environmental Protection Agency. (2001). Risk assessment guidance for Superfund, Volume I: Human health evaluation manual (Part E, Supplemental guidance for dermal risk assessment): Review draft for public comment (EPA/540/R/99/005; OSWER 9285.7-02EP). Office of Emergency and Remedial Response. https://www.epa.gov/risk/risk-assessment-guidance-superfund-rags-part-e

U.S. Environmental Protection Agency. (2004). Risk assessment guidance for Superfund, Volume I: Human health evaluation manual (Part E, Supplemental guidance for dermal risk assessment) (EPA/540/R/99/005; OSWER 9285.7-02EP). Office of Superfund Remediation and Technology Innovation. https://www.epa.gov/risk/risk-assessment-guidance-superfund-rags-part-e

U.S. Environmental Protection Agency. (2007). SW-846 test method 3550C: Ultrasonic extraction (Revision 3). https://www.epa.gov/hw-sw846/sw-846-test-method-3550c-ultrasonic-extraction

U.S. Environmental Protection Agency. (2009). Risk assessment guidance for Superfund, Volume I: Human health evaluation manual (Part F, Supplemental guidance for inhalation risk assessment) (EPA-540-R-070-002; OSWER 9285.7-82). Office of Superfund Remediation and Technology Innovation. https://www.epa.gov/risk/risk-assessment-guidance-superfund-rags-part-f

U.S. Environmental Protection Agency. (2011). Regional screening levels for chemical contaminants at Superfund sites. https://www.epa.gov/risk/regional-screening-levels-rsls

U.S. Environmental Protection Agency. (2012). Integrated Risk Information System (IRIS): 2,3,7,8-Tetrachlorodibenzo-p-dioxin (TCDD)—Chemical assessment summary (CASRN 1746-01-6). https://iris.epa.gov/static/pdfs/1024_summary.pdf

Van den Berg, M., Birnbaum, L. S., Denison, M., De Vito, M., Farland, W., Feeley, M., Fiedler, H., Hakansson, H., Hanberg, A., Haws, L., Rose, M., Safe, S., Schrenk, D., Tohyama, C., Tritscher, A., Tuomisto, J., Tysklind, M., Walker, N., & Peterson, R. E. (2006). The 2005 World Health Organization reevaluation of human and mammalian toxic equivalency factors for dioxins and dioxin-like compounds. Toxicological Sciences, 93(2), 223–241. https://doi.org/10.1093/toxsci/kfl055

World Health Organization. (2018). World health statistics 2018: Monitoring health for the SDGs, sustainable development goals. https://www.who.int/publications/i/item/9789241565585

Yang, B., Xue, N., Zhou, L., Li, F., Cong, X., Han, B., Li, H., Yan, Y., & Liu, B. (2012). Risk assessment and sources of polycyclic aromatic hydrocarbons in agricultural soils of Huanghuai plain, China. Ecotoxicology and Environmental Safety, 84, 304–310. https://doi.org/10.1016/j.ecoenv.2012.07.027