Impact of oil refinery spillage on heavy metal contamination and pollution indices in biotic and abiotic components in Ennore Creek, India

Abstract

Heavy metal contamination has rapidly increased in recent decades, posing a significant effect on the aquatic environment and the organisms inhabiting it. The chief objective of the study is to ascertain concentrations of Cadmium (Cd), Chromium (Cr), Lead (Pb) and Mercury (Hg) in collected water, sediment and biological samples before and after the oil spill at Ennore. The commercially consumed fish Elops saurus and Gerres filamentous were identified using morphometric characteristics by a benchmark protocol. Heavy metal concentrations showed marked variation among stations, with Pb recording the highest levels in water and sediment (75.8 μg/L and 856 mg/kg, respectively), followed by Cr (4.06 μg/L and 164.2 mg/kg), respectively. Among the biological matrices, after the oil spill, Elops saurus showed higher Cr accumulation (5.57 mg/kg), while polychaetes exhibited comparatively elevated Pb levels (1.60 mg/kg). Overall, the elevated metal concentrations, particularly in sediment and biota, indicate potential bioaccumulation and ecological concern in the study area. Pollution indices, including the geoaccumulation index, pollution load index and bioaccumulation factor, indicated escalating pollution after the oil spill. This alerts to the urgent need to improve and protect the aquatic ecosystem health for future generations.

Keywords: Aquatic environment, Elops saurus, accumulation, Gerres filamentous, Pollution indices, Ecological concern.

- Introduction

Heavy metals (HMs) are elements with dense metallic properties and are toxic even at very low levels. Metal elements are pervasively found in waste released from industrial sludge, fertiliser, paints, ash and oil refineries (Wang & Yang, 2016; Panseriya et al., 2020). HMs are present in fractional amounts known as trace elements.  Chromium (Cr), Cobalt (Co), Copper (Cu), Nickel (Ni), Selenium (Se), Zinc (Zn), Molybdenum (Mo) and Iron (Fe), have a vital role in the maintenance of biochemical and physiological function (Prashanth et al., 2015). Essential elements have deleterious effects when taken excessively, as they disturb the absorption of other nutrients and damage the organs (Canli & Atli, 2003). Non-essential elements, for instance, Cadmium (Cd), Mercury (Hg) and Lead (Pb) are not requisite for biological activities; hence, they are considered perilous to humans and also to the aquatic environment as they lead to poor reproduction and growth, causing tissue damage (Otchere, 2019; Liang et al., 2023).

In the recent decade, coastal ecosystems have been tainted by the pollutants of anthropocentric activities, particularly agricultural leachate, industrial expansion, urbanisation and economic activities. Trace toxic elements are eminent pollutants discharged into the ocean and can accumulate in the sediments and marine ecosystems (Liu et al., 2007; Velusamy et al., 2014; Stark et al., 2014; Vetrimurugan et al., 2019). Sediment contamination by heavy metals may influence aquatic ecosystem quality, biosorption and bioaccumulation. Moreover, these metals become immobilised in sediments and may therefore be involved in co-precipitation, absorption and the formation of complexes (Chandrasekaran et al., 2015). When exposed to human-induced activity, sediment absorbs contaminants from various sources like mines, agricultural fields, industries, etc. (da Silva et al., 2025).

Bioaccumulation indicates the accumulation of metal pollution within the environment. Elevated quantities of non-essential elements have been detected in marine animals, including polychaetes, edible mussels, sea stars, clams and fish. This bioaccumulation may cause cellular harm (Singh et al., 2017). Fish are largely employed in metal accumulation experiments since they are a vital species and are regularly consumed by humans (Krishnamurti, 1999; Rashed, 2001)

Fish are known as bioindicators of metal accumulation (de Carvalho et al., 2025). When fish accumulate a large amount of metals, it poses a potential risk of transferring metals to humans (Gu et al., 2015).  Therefore, it is crucial to determine the levels of heavy metals in the organisms to evaluate if the concentration does not fall within acceptable limits or poses health threats to the consumer (Krishnamurti, 1999).

Research gap: Even though some heavy metals research has reported contamination at Ennore, there is limited comparison of heavy metal accumulation in biotic as well as edible fish and their ecological risk before and after the spill.

1.1 Objective

The importance of the present study is to demonstrate the potential risk of heavy metal contamination in water, sediment, polychaetes and Elops saurus and Gerres filamentous fish muscles and its impact on Ennore Creek before and after the oil spill.

- Materials and methods

2.1 Study area location description

The geographical location of the present study area was depicted using QGIS 3.44. Five sampling sites were chosen for analysing heavy metal contamination representation of the geographical location at Ennore Fig. 1. Ennore Creek at 13˚13’52.85’’N, 80˚19’40.51’’E, Nettukuppam at 13˚13’56.79’’N, 80˚19’52.44’’E, Thazhankuppam at 13˚13’38.90’’N, 80˚19’48.92’’E, Periyakuppam at 13˚12’43.15’’N, 80˚19’29.14’’E and Chinnakuppam at 13˚12’26.71’’N, 80˚19’22.19’’E.

Fig. 1. Representation of the geographical location of the five sampling sites

The Ennore region is also considered an industrial hub and this region is also a habitat for aquatic organisms as well as for fishermen and local communities. The tragic oil leakage occurred at the Chennai Petroleum Corporation Limited (CPCL) on December 3, 2023.  Along with the Michaung Cyclone, the heavy downpour flooded the refinery, causing oil to mix with water.

2.2 Sample collection and analysis

Water, sediment, polychaetes and two edible fish samples were collected from five preselected sampling points over an extended period from October 2023 to February 2024, ranging before and after the oil spillage. About 1 litre of water samples (water column and interstitial water) were collected and stored in a sterilised container.  Sediments were randomly collected from 500 m along the coastal regions at a depth of 15 cm in an aluminium box. A bulk of about 50 polychaetes were handpicked arbitrarily, and 10 specimens of 2 different edible fish were collected from station 1 to station 5 and stored separately on dry ice and carried to the Aquatic Sciences and Biotechnology Laboratory (ASBL), Ethiraj College for Women, Chennai, for future analysis. The polychaete and fish samples were rinsed with distilled water to remove dust and debris. Using Days Volume 1878, the collected edible fish species were identified morphometrically as Elops saurus and Gerres filamentous; then, authorised confirmation was done by an expert from the Central Institute of Brackish water Aquaculture, ICAR-CIBA.

2.3 Acid digestion method

100 ml water samples were supplemented with 5 ml of concentrated Nitric acid (HNO3), to which 100 ml of ultrapure water was added to the filtered sample to estimate the concentration of heavy metals (Pandiyan et al., 2020). The soil samples were rinsed with HNO3 (10%), air-dried for 3 days and crushed to a fine powder. About 9 ml of HNO3, along with 3ml of HCl, was added to 0.500 g of the powdered samples.  Tissue samples collected from the polychaetes and fish were analysed according to the AOAC method (2016). Following digestion, the samples were analysed using Inductively Coupled Plasma Optical Emission Spectroscopy (ICP-OES AvioTM 220 Max).

2.4 Heavy metal pollution index (HPI)

HPI was calculated using the following formula (Mohan et al., 1996),

HPI= i=1nQiWii=1nWi

Where,

Wi = unit weight of the ith parameter

Qi = sub-index value of the ith parameter.

2.5 Geoaccumulation index

The geoaccumulation index (Igeo) is a comprehensive method to assess heavy metal pollution within the environment. Igeo can be determined by a specific formula that enables the systematic assessment of the contamination rate.

Igeo = log2[Cn/1.5 x Bn]

Where Cn denotes metal content, Bn is the shale background content of the metal as stated by Turekian & Wedepohl. (1961), and 1.5 is the constant initiation to minimise change of background value because of lithogenic origins. Furthermore, the Igeo value is prorated into seven level classifications according to Muller (1969).

2.6 Pollution Load Index (PLI)

The pollution load index (PLI) was evaluated using the following formula (Tomlinson et al., 1980),

CF = Cmetal/Cbackground

PLI= (CF1 x CF2 x ….  x CFn)1/n

where ‘n’ represents the number of metals present, CF specifies the pollution factor, Cmetal represents the sample metal concentration, and Cbackground indicates the metal concentration of the background. PLI >1 recommends sediment pollution, and PLI < 1 absolutely no pollution.

2.7 Bioaccumulation factor (BAF)

BAF abides by Wang et al. (2017) to find the heavy metal concentration in fish relative to that in water.

BAF = Ci/Cw

Where Ci signifies the mean concentration of pelagic organisms, whereas Cw signifies the mean water concentration.

2.8 Statistical analysis

In order to determine the relationship of heavy metals among water, soil, polychaete and fish species, Pearson correlation was calculated using SPSS version 20.0.  The analysis of variance (ANOVA) was conducted using the JAMOVI version 2.6.44 software.

- Results and Discussion

3.1 Physicochemical parameter

Water is considered to be essential for aquatic organisms, but it gets contaminated due to anthropogenic activities (Sharma et al., 2025). Industrialisation is progressively expanding, which in turn becomes the foundation for pollution and releases toxic elements into the environment. The temperature before and after the oil spill was within the acceptable range of 27.7 ± 0.52 ºC to 33.4 ± 0.55 ºC and 25.08 ± 0.65 to 32.8 ± 0.70ºC, respectively. The water quality before and after the oil spill at 5 different geographical locations was analysed and presented in Table 1.  Normally, the temperature varies depending on the period, seasons and salt concentration of the water (Vincy et al., 2012). Water temperature before and after the oil slick varies by approximately 2 °C in the present study. The oil on the water surface inhibits sunlight penetration causing the temperature to drop at the point of the spill area (Xing et al., 2015). The pH before the oil leak ranged from 8.2 ± 0.26 to 7.3 ± 0.50, but after the spill 8 ± 0.52 to 7.1 ± 0.60. The pH level of water exhibits slight fluctuations following the spillage in the present investigation, which indicates that the pH remains marginally alkaline. Slightly similar change in pH was observed by Chen et al. (2017) in Taiwan after the oil spillage was within the range of 7.95 to 8.07. The salinity before the spill was between 22.12 ± 0.63 to 31.6 ± 0.85 ppt, whereas after the spill, the observed salinity was between 18.1 ± 0.91 to 25.9 ± 0.77 ppt. Hence, salinity after the spillage reduced to 4 to 6 ppt. The water salinity relies on tidal variation and on the ratio of marine and river water (Cheng et al., 2005). The dissolved oxygen (DO) in the present study was <5 mg/l; i.e., the range was from 0.8 ± 0.26 mg/l to 4.7 ± 0.81mg/l whereas after the oil spill it was 0.71 ± 0.12 mg/l to 3.97 ± 0.56 mg/l. The DO levels are reduced slightly after the slick as compared to before the spill. The slight decrease in DO is due to the oil presence, which disturbs the exchange of oxygen in the water bodies and provides less oxygen to the fish (Bulbul & Mishra, 2022).

Table 1. The average, mean and standard deviation of physicochemical parameters of water before and after the oil slick.

| Parameters | ST-1 | ST-2 | ST-3 | ST-4 | ST-5 |
| --- | --- | --- | --- | --- | --- |
| Before oil slick |  |  |  |  |  |
| Temperature (˚C) | 33.4±0.55 | 31.08±0.45 | 29.7±0.80 | 28.5±0.88 | 27.7±0.52 |
| pH | 8.2±0.26 | 8±0.26 | 7.8±0.36 | 7.5±0.43 | 7.3±0.50 |
| Salinity (ppt) | 31.6±0.85 | 29.1±0.72 | 27.01±0.70 | 24.04±0.21 | 22.12±0.63 |
| DO (mg/l) | 4.7±0.81 | 3.89±0.08 | 3.24±0.13 | 2.41±0.31 | 0.8±0.26 |
| After oil slick |  |  |  |  |  |
| Temperature (˚C) | 32.8±0.70 | 29.6±0.10 | 27.9±0.70 | 26.4±0.70 | 25.08±0.65 |
| pH | 8±0.52 | 7.7±0.36 | 7.5±0.52 | 7.2±0.43 | 7.1±0.60 |
| Salinity (ppt) | 25.9±0.77 | 25.7±1.02 | 22.4±1.21 | 20.6±0.45 | 18.1±0.91 |
| DO (mg/l) | 3.97±0.56 | 3.42±0.56 | 3.1±0.55 | 2.49±0.36 | 0.71±0.12 |

3.2 Multi-matrix metal assessment

Many studies on heavy metals in the aquatic environment have been conducted by researchers worldwide. The predominant Cd, Cr, Pb and Hg trace metals in the water bodies prominently enter into the living organisms present within the aquatic environment, in turn causing bioaccumulation of trace elements in the food chain (Loska & Wiechuła, 2003; Ekperusi et al., 2024). The overall mean values of heavy metals in the water in 5 different regions have been evaluated. The Cd (0.02±0.01 to 0.21±0.02 μg/L), Cr (0.15±0.02 to 2.30±0.26 μg/L) and Hg (0.01±0 to 0.12±0.04 μg/L) in the water samples from all the stations were below the permissible limit. But after the spill, the level eventually increased and was above the permissible level; among all the stations, the highest value of Cd, 2.89±0.31 μg/L, was recorded at station 4. For Cr, the highest value was at station 1, which was 100±4 μg/L and for Hg it was at station 3 with 1.13±0.11 μg/L, as represented in Table 2.  Crude oil released into the water bodies contains heavy metals; this eventually gets spread, leading to an elevation in metal concentration (Sharma et al., 2024). On the other hand, Pb before (51.49±2.03 μg/L) and after (320.31±10.49 μg/L) the spill exceeded the permissible range. Chen et al. (2022) reported that lead concentration in the global ocean was high mainly because of anthropogenic activity, which includes industrial emissions. Kibria et al. (2016) investigated 11 heavy metals in the Bangladesh Coastal and Karnafuli River estuary. The study results showed that Hg, Cr, Cd, Ni and Cu were found to be high, thereby impacting the water quality and in turn, affecting the ecology.

Table 2. Mean and standard deviation of heavy metals before and after the oil slick from the five different stations

| Stations | Parameters | Water (μg/L) |  | Sediment (mg/kg) |  | Polychaete (mg/kg) |  | Elops saurus (mg/kg) |  | Gerres filamentous (mg/kg) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BOS | AOS | BOS | AOS | BOS | AOS | BOS | AOS | BOS | AOS |
| ST 1 | Cd | 0.13±0.02 | 2.00±0.47 | 0.24±0.06 | 6.20±0.22 | 0.01±0 | 0.10±0.06 | BDL | 0.11±0.01 | BDL | 0.19±0.01 |
|  | Cr | 2.30±0.26 | 100±4 | 1.13±0.83 | 164.20±6.01 | 0.20±0.17 | 5.50±0.06 | BDL | 0.41±0.02 | BDL | 1.20±0.27 |
|  | Pb | 75.80±2 | 320.31±10.49 | 3.56±0.59 | 856±17.74 | BDL | 1.60±0.05 | BDL | 0.21±0.02 | BDL | 0.23±0.01 |
|  | Hg | 0.12±0.04 | 1±0.17 | 0.13±0.02 | 1.30±0.02 | BDL | 0.10±0.09 | BDL | 0.06±0.06 | BDL | 0.10±0.01 |
| ST 2 | Cd | 0.06±0.03 | 1.75±0.35 | 0.60±0.06 | 4.35±0.19 | 0.03±0.02 | 0.13±0.07 | BDL | 0.18±0.03 | BDL | 0.24±0.06 |
|  | Cr | 1.13±0.10 | 64.62±3.71 | 4.06±1.00 | 127.98±5.47 | 0.49±0.13 | 3.57±0.49 | 0.01±0 | 0.61±0.07 | BDL | 1.03±0.12 |
|  | Pb | 47.01±2.53 | 157.64±6.26 | 7.62±0.51 | 657.81±15.11 | 0.07±0.05 | 1.15±0.07 | 0.03±0.02 | 0.62±0.15 | 0.01±0 | 0.52±0.15 |
|  | Hg | 0.01±0 | 0.84±0.35 | 0.08±0.01 | 1.49±0.08 | 0.01±0 | 1.13±0.10 | 0.01±0 | 1.10±0.01 | BDL | 1.05±0.03 |
| ST 3 | Cd | 0.21±0.02 | 2.41±0.14 | 0.82±0.03 | 5.11±0.18 | 0.07±0 | 0.11±0.02 | 0.01±0.01 | 0.11±0.01 | BDL | 0.10±0 |
|  | Cr | 1.35±0.02 | 52.74±2.73 | 1.13±0.03 | 120.67±5.21 | 0.42±0.02 | 5.25±0.13 | 0.18±0.02 | 1.12±0.03 | 0.13±0.02 | 1.15±0.04 |
|  | Pb | 51.49±2.03 | 262.66±7.09 | 3.19±0.16 | 600.57±14.63 | 0.39±0.01 | 1.60±0.05 | 0.10±0 | 0.21±0.02 | 0.08±0 | 0.22±0.03 |
|  | Hg | 0.06±0.01 | 1.13±0.11 | 0.17±0.02 | 1.28±0.04 | 0.40±0.02 | 1.11±0.01 | BDL | 0.05±0 | BDL | 0.08±0.01 |
| ST 4 | Cd | 0.04±0.01 | 2.89±0.31 | 0.68±0.03 | 2.69±0.83 | 0.04±0.01 | 0.17±0.01 | BDL | 0.10±0.01 | BDL | 0.13±0.01 |
|  | Cr | 2.24±0.12 | 67.81±3.15 | 3.06±0.20 | 136.09±6.01 | 0.52±0.05 | 1.69±0.15 | 0.16±0.06 | 1.16±0.05 | 0.24±0.03 | 1.13±0.03 |
|  | Pb | 32.52±1.56 | 124.05±5.66 | 2.59±0.15 | 587.20±13.58 | 0.25±0.02 | 1.40±0.04 | 0.02±0.01 | 0.46±0.11 | 0.07±0.05 | 0.24±0.04 |
|  | Hg | 0.07±0.02 | 1.07±0.02 | 0.10±0.01 | 1.38±0.10 | 0.06±0.01 | 0.20±0.02 | 0.03±.01 | 1.13±0.11 | 0.02±.01 | 1.09±.04 |
| ST 5 | Cd | 0.02±0.01 | 1.55±0.02 | 0.18±0.01 | 1.39±0.04 | 0 | 0.10±0.2 | 0.01±0 | 0.10±0.01 | BDL | 0.09±0.01 |
|  | Cr | 0.15±0.02 | 49.41±2.53 | 1.08±0.02 | 109.96±4.41 | 0.33±0.02 | 2.17±0.04 | BDL | 2.09±0.03 | 0.08±0 | 1.83±0.07 |
|  | Pb | 22.99±1.53 | 123.55±5.21 | 1.42±0.03 | 540.00±12.46 | 0.23±0 | 0.42±0.01 | 0.09±0 | 0.36±0.02 | 0.07±0 | 0.39±0.01 |
|  | Hg | 0.02±0 | 0.44±0.02 | 0.06±0.02 | 1.15±0.02 | 0.39±0.01 | 1.14±0.02 | 0.00±0 | 1.10±0.02 | 0.05±0.01 | 1.12±0.01 |

Note: BOS- Before Oil Spill, AOS- After Oil Spill

The mean concentrations of Cd, Cr, Pb and Hg before the spill were under the limit, except for Cd (0.82±0.03 mg/kg) and Hg (0.17±0.02 mg/kg) at station 3. Sediment in the present study has a metal concentration 2 times higher than in water.  The metals Cd, Cr, Pb and Hg were exceeded the permissible level after the oil spill, with the maximum of Pb pollution. The detailed values of heavy metal pollution are represented in Table 2. Here, the maximum value was for Pb (540.00±12.46 mg/kg to a maximum of 856±17.74 mg/kg). The oil ensnared in the sediments causes ecological disturbance, which can manifest as a reduction in density and biomass to a decline in diversity (Lee & Lin, 2013). Heavy metals get settled in the sediment for a longer period of time because sediment acts as a sink for most of the pollutants in the environment (Xu et al., 2022). Steffy et al. (2013) provide evidence from the Deepwater Horizon oil spill that Cr and Pd increased after the spill settled in the sediment.

The interpretation of metals in polychaetes suggests that after the oil spill all the four metal were drastically increased above the permissible limit, with each minimum to maximum mean values being Cd (0.10±0.2 to 0.17±0.01 mg/kg), Cr (1.69±0.15 to 5.50±0.06 mg/kg), Pb (0.42±0.01 to 1.60±0.05 mg/kg) and Hg (0.10±0.09 to 1.14±0.02 mg/kg). The polychaete is normally observed to have elevated metal levels compared to fish because polychaetes have close contact with the sediment and their feeding habits suggest they are more prone to benthic contamination exposure (Karthikeyan et al., 2020).

The two varieties of fish Elops saurus and Gerres filamentous, were analysed for metal abundance in the present study and the levels were significantly below the permissible level given by FAO. (2022) before the spillage. Whereas the metal concentration was slightly above the allowable limit after the oil leak.  The mean values for E. saurus after the spill were for Cd (0.10±0.01 to 0.18±0.03 mg/kg), Cr (0.41±0.02 to 2.09±0.03 mg/kg), Pb (0.21±0.02 to 0.62±0.15 mg/kg) and Hg 0.05±0 to 1.13±0.11 mg/kg). The mean concentrations for Cd (0.09±0.01 to 0.24±0.06 mg/kg), Cr (1.03±0.12 to 1.83±0.07 mg/kg), Pb (0.23±0.01 to 0.52±0.15 mg/kg) and Hg (0.08±0.01 to 1.12±0.01 mg/kg) in G. filamentous.

The fish exposed to oil contamination have a risk of effects when the metal concentration increases. Cd can injure the hypothalamus-pituitary-gonadal axis and cause hormone changes (Vinanthi Rajalakshmi et al., 2023), malformed larvae were found in experimental Danio rerio fish (Tarasco et al., 2019).  Other researchers experimented with different fish and found abnormal germ cells, low hatching rates and mortality in Gymnotus carapo and Gasterosteus aculeatus (Vergilio et al., 2015; Hani et al., 2019). Cr presence in fish muscles can cause allergic reactions and affect immunity (Bakshi et al., 2018). In other cases, chronic exposure in fish may lead to anaemia (Ray et al., 2025). Chronic exposure to lead results in neurological dysfunction in fish (Zhu et al., 2016; Lee et al., 2019) and in humans who feed on fish. Moreover, mercury in fish becomes lethal to an extent and also shows apoptosis; oxygen depletion in the fish disturbs electron transport (Carocci et al., 2013).

3.3 Heavy Metal Pollution Index (HPI)

HPI value was calculated for four metals Cd, Cr, Pb and Hg following the methodology of Mohan et al. (1996). Before the oil spillage, the metal concentration was <100, indicating no pollution, whereas after the spill the concentration exceeded >100 hence, the water bodies were polluted. The HPI of metals shows that lead was about 220.4 at station 1, 108.47 at station 2 and 180.73 at station 5, which was highest among other metals. Overall, the station-wise calculated values indicate that station 1 has the maximum contamination (307.25), followed by station 3 (278.36), then station 4 (182.94), station 2 (181.42) and the least was station 5 (128.5). The rise in metal contamination of metals may be due to the crude oil spill because there is a chance of trace elements being discharged from the spill (Chaharlang et al., 2016); or the metal might mobilise along with the oil and settle in sediment, contaminated water and aquatic species; then, possibilities of anthropogenic activities can also contribute to the contamination (Santos-Echeandía et al., 2008).

3.4 Geoaccumulation index (Igeo)

The Igeo index was used to assess sediment metal contamination according to Muller, (1969) classification; the study obtained results ranging from uncontaminated to heavily to extremely contaminated. The Igeo index was depicted in Fig. 2.  BOS, the Igeo class for Cd, Cr, Pb and Hg at stations 1 to 5 was recorded as class 0, indicating a pollution-free environment, except for Cd (0.42, 0.87, 0.6) at stations 2, 3, and 4, which signify class 1, indicating soil that is uncontaminated to moderately contaminated. The untreated waste from the industries, metal manufacturing plant, steel, chromium plating and chemical fabrication acts as a source of pollution (Keshav & Rama, 2016). After the oil slick, Cr at stations 1 and 5 (-0.08 and -0.3) were analysed to be the lowest Igeo represents class 0, no pollution. Heavy to extremely polluted levels were seen at Pb (ST1: 4.38, ST2: 4.45, ST3: 4.32, ST4: 4.29, ST5: 4.17), suggesting class 5 contamination. Cd was classified under class 4 at stations 1 to 3, which have heavily polluted sediments; class 3 at station 4, which has moderate to high contamination; and class 2 at station 5, which has moderate pollution. The concentration of Cr at stations 1 to 4, which are uncontaminated to moderately contaminated (class 1). Hg concentration falls under class 2 at stations 1 to 4 and under class 1 at station 5. The Pb level after the spill may be elevated because it accumulates in fine particles of sediment and organic matter (Jayaprakash et al., 2008).

Fig. 2. Geoaccumulation values derived from the sediment of the study area.

3.5 Pollution Load Index (PLI)

Heavy metals Cd, Cr, Pb and Hg in sediment from station 1 (0.15), station 2 (0.30), station 3 (0.21), station 4 (0.22) and station 5 (0.09) were >1 stipulate no pollution before the oil spill.  Conversely, post the spill, station 1 (8.51), station 2 (7.09), station 3 (6.89), station 4 (6.86) and station 5 (4.58) metal pollution levels were above 1, implying greater cumulative pollution occurred at all five stations. This is probably the result of a low-energy aquatic environment allowing the metals to settle down on the sediment surface (Yan BeiZhan et al., 2016).

3.6 Bioaccumulation factor

BAF values in polychaetes show that the water before and after the spill ranged from 0.01 to 19.50 and after the spill from 0.01 to 2.59, indicating relatively low accumulation of heavy metals in the polychaete samples compared to the water. For Elops saurus and Gerres filamentous, they show comparatively minimal accumulation relative to their concentrations in the surrounding water. A similar study was reported by Olayinka-Olagunju et al. (2021) in Clarias gariepinus muscle show low accumulation and mainly due to feeding behaviour and for each species it differs.

3.7 Pearson correlation

A strong positive correlation in the water between temperature and salinity (r = 0.89); pH and DO (r = 0.75) and Salinity and DO (r = 0.84) are the same in sediment, polychaetes, and fish.  The correlation of metals in water: Cd and Cr (r = 0.89); Cr and Hg (r = 0.89); Pb and Hg (r = 0.83). Similarly, a very strong positive correlation was seen between Cd and Hg (r = 0.95).  In contrast, the physicochemical parameters show a strong negative correlation with the heavy metals in both water and sediment. Correlation alone does not determine the source of contamination, but it can also influence metal mobility (Nishitha et al., 2023). Heavy metals in water show high statistical significance < 0.001. In sediment, a strong positive correlation was seen for Cd and Pb (r = 0.91) and for Cr and Hg (r = 0.98).  (Zhang et al., 2022) reported in the sediment that there was a negative correlation between Hg, Cr and Cd and salinity. On the other hand, temperature, pH and DO show a negative correlation with heavy metals (Zhang et al., 2018). The p-values for polychete metals Cd- Cr, Cd-Hg, Cr-Hg and Pb-Hg were <0.05, which is statistically significant, whereas Cd-Pb and Cr-Pb were <0.001, which is highly statistically significant. A very strong correlation was observed for Cr-Pb (r = 0.88), whereas a weak correlation was seen for Pb-Hg (r = 0.36). A very strong correlation was seen for Cr-Pb (r = 0.88), whereas a weak correlation was seen for Pb-Hg (r = 0.36). The physicochemical parameters do sometimes influence the change in metal accumulation in Diopatra neapolitana (Giménez et al., 2022). The p-values for polychetes and Elops saurus Cd- Cr, Cd-Hg, Cr-Hg and Pb-Hg were <0.05, which is statistically significant, whereas Cd-Pb and Cr-Pb were <0.001, which is highly statistically significant. E. saurus, a very strong positive correlation was found between Cd-Pb (r = 0.84) and Pb-Hg (r = 0.88) then a strong correlation among the metals Cr-Hg (r = 0.69) and a weak correlation between Cr-Pb (r = 0.6). Similarly, for G. filamentous, Cd-Pb (r = 0.82) represents a very strong correlation; a weak correlation was observed for Cr-Pb (r = 0.8) The p-values for all the metals were <0.001, which is highly statistically significant.

4. Conclusions

The present study significantly provides a pronounced increase in heavy metal concentration in the environment as a direct consequence of the oil spill. Trace amounts of non-essential heavy metals can be toxic to many living organisms. In our study, trace metals such as Cd, Cr, Pb, and Hg were compared with the biotic and abiotic components of the environment and these metals were among the most increased after the oil spill. Igeo and PLI indices used to statistically analyse metal pollution in the sediment reveal most of the contamination. BAF values were determined to find the level of metal accumulation in the fish, showing slight accumulation. Overall, the toxic contamination was found more in sediment Pb> Cr> Cd> Hg. Bioremediation and diligent monitoring of waste discharges from industries ensure minimal contamination and reduce the impact level on the coastal area as well as the terrestrial ecosystem, which in turn reduces the risk of bioaccumulation of toxic substances in humans as well as the environment.

Declaration of AI Use

This manuscript was prepared through the intellectual contributions of the author(s) to the study design, data analysis, interpretation of results, and scholarly content. Grammarly and ChatGPT were used solely to assist with grammatical refinement and reference formatting during manuscript revision. The author(s) accept full responsibility for the accuracy, integrity, and final content of the manuscript.

COMPETING INTERESTS DISCLAIMER:

Authors have declared that they have no known competing financial interests OR non-financial interests OR personal relationships that could have appeared to influence the work reported in this paper.

References

- AOAC. (2016). Official Methods of Analysis of the Association of Official Analytical Chemists. 20th Edition, AOAC Inc., Washington DC.

- Bakshi, A., & Panigrahi, A. K. (2018). A comprehensive review on chromium induced alterations in fresh water fishes. Toxicology reports, 5, 440-447. https://doi.org/10.1016/j.toxrep.2018.03.007.

- Bulbul Ali, A., & Mishra, A. (2022). Effects of dissolved oxygen concentration on freshwater fish: A review. 10 (4), 113-127.  https://doi.org/10.22271/fish.2022.v10.i4b.2693.

- Canli, M., & Atli, G. (2003). The relationships between heavy metal (Cd, Cr, Cu, Fe, Pb, Zn) levels and the size of six Mediterranean fish species. Environmental pollution, 121(1), 129-136.https://doi.org/10.1016/S0269-7491(02)00194-X.

- Carocci, A., Rovito, N., Sinicropi, M. S., & Genchi, G. (2013). Mercury toxicity and neurodegenerative effects. Reviews of environmental contamination and toxicology, 1-18. https://doi.org/10.1007/978-3-319-03777-6_1

- Chandrasekaran, A., Mukesh, M. V., Chidambaram, S., Singarasubramanian, S. R., Rajendran, S., Muthukumarasamy, R., & Tamilselvi, M. (2015). Assessment of heavy metal distribution pattern in the sediments of Tamirabarani river and estuary, east coast of Tamil Nadu, India. Environmental Earth Sciences, 73(5), 2441-2452. https://doi.org/10.1007/s12665-014-3593-y

- Chaharlang, B. H., Bakhtiari, A. R., Mohammadi, J., & Farshchi, P. (2016). Geochemical partitioning and pollution assessment of Ni and V as indicator of oil pollution in surface sediments from Shadegan wildlife refuge, Iran. Marine pollution bulletin, 111(1-2), 247-259. https://doi.org/10.1016/j.marpolbul.2016.06.109

- Chen, C. C., Tew, K. S., Ho, P. H., Ko, F. C., Hsieh, H. Y., & Meng, P. J. (2017). The impact of two oil spill events on the water quality along coastal area of Kenting National Park, southern Taiwan. Marine pollution bulletin, 124(2), 974-983. https://doi.org/10.1016/j.marpolbul.2017.02.034

- Chen, M., Carrasco, G., Park, E., Morgan, K., Tay, S.H.X., Tanzil, J., Ooi, S.K., Zhou, K. & Boyle, E.A. (2022). Monsoonal variations of lead (Pb) in coastal waters around Singapore. Marine Pollution Bulletin, 179, 113654. https://doi.org/10.1016/j.marpolbul.2022.113654

- Cheng, K. M., Hu, C. Q., Liu, Y. N., Zheng, S. X., & Qi, X. J. (2005). Dietary magnesium requirement and physiological responses of marine shrimp Litopenaeus vannamei reared in low salinity water. Aquaculture Nutrition, 11(5), 385-393. https://doi.org/10.1111/j.1365-2095.2005.00364.x

- Da Silva, C. C., da Costa, B. V. M., de Araújo, C. M. V., & de Souza Santos, L. P. (2025). Long-term variation of Ecotoxicological quality of sediment in an estuarine system with industrial and port activities. Marine Pollution Bulletin, 215, 117854. https://doi.org/10.1016/j.marpolbul.2025.117854

- De Carvalho, D., Guimarães, I. M., Ferreira, F. F., Dergam, J. A., Moreira, M. Z., & Pompeu, P. S. (2025). Linking trophic ecology and metal bioaccumulation to assess a widespread fish as a bioindicator following a large-scale mining disaster. Environmental Science and Pollution Research, 32(51), 29192-29206. https://doi.org/10.1007/s11356-025-37248-9

- Ekperusi, A. O., Michael, A., Chukwurah, C. H., & Sunday, N. M. (2024). Evaluation of heavy metals and their potential risk to human health from seafood in Escravos Estuary, Southern Nigeria. Marine Pollution Bulletin, 208, 117014. https://doi.org/10.1016/j.marpolbul.2024.117014

- FAO (2022). The State of World Fisheries and Aquaculture 2022. Towards Blue Transformation. Rome, FAO. https://doi.org/10.4060/cc0461en

- Giménez, V., Cardoso, P., Sá, C., Patinha, C., Ferreira da Silva, E., Figueira, E., & Pires, A. (2022). Interplay of seasonality, major and trace elements: Impacts on the polychaete Diopatra neapolitana. Biology, 11(8), 1153.  https://doi.org/10.3390/biology11081153

- Gu, Y. G., Lin, Q., Wang, X. H., Du, F. Y., Yu, Z. L., & Huang, H. H. (2015). Heavy metal concentrations in wild fishes captured from the South China Sea and associated health risks. Marine pollution bulletin, 96(1-2), 508-512. https://doi.org/10.1016/j.marpolbul.2015.04.022

- Hani, Y. M., Turies, C., Palluel, O., Delahaut, L., Bado-Nilles, A., Geffard, A., Dedourge-Geffard, O., & Porcher, J.-M. (2019). Effects of a chronic exposure to different water temperatures and/or to an environmental cadmium concentration on the reproduction of the threespine stickleback (Gasterosteus aculeatus). Ecotoxicology and Environmental Safety, 174, 48–57. https://doi.org/10.1016/j.ecoenv.2019.02.032

- Jayaprakash, M., Jonathan, M. P., Srinivasalu, S., Muthuraj, S., Ram-Mohan, V., & Rajeshwara-Rao, N. (2008). Acid-leachable trace metals in sediments from an industrialized region (Ennore Creek) of Chennai City, SE coast of India: an approach towards regular monitoring. Estuarine, Coastal and Shelf Science, 76(3), 692-703. https://doi.org/10.1016/j.ecss.2007.07.035

- Karthikeyan, P., Marigoudar, S. R., Mohan, D., Nagarjuna, A., & Sharma, K. V. (2020). Ecological risk from heavy metals in Ennore estuary, South East coast of India. Environmental Chemistry and Ecotoxicology, 2, 182-193. https://doi.org/10.1016/j.enceco.2020.09.004

- Keshav Krishna, A., & Rama Mohan, K. (2016). Distribution, correlation, ecological and health risk assessment of heavy metal contamination in surface soils around an industrial area, Hyderabad, India. Environmental Earth Sciences, 75(5), 411. https://doi.org/10.1007/s12665-015-5151-7

- Kibria, G., Hossain, M. M., Mallick, D., Lau, T. C., & Wu, R. (2016). Trace/heavy metal pollution monitoring in estuary and coastal area of Bay of Bengal, Bangladesh and implicated impacts. Marine pollution bulletin, 105(1), 393-402. https://doi.org/10.1016/j.marpolbul.2016.02.021

- Krishnamurti, A.J., & Nair, V.R. (1999). Concentration of metals in shrimps and crabs from Thane-Bassein creek system, Maharashtra.

- Lee, L. H., & Lin, H. J. (2013). Effects of an oil spill on benthic community production and respiration on subtropical intertidal sandflats. Marine pollution bulletin, 73(1), 291-299. https://doi.org/10.1016/j.marpolbul.2013.05.006

- Lee, J. W., Choi, H., Hwang, U. K., Kang, J. C., Kang, Y. J., Kim, K. I., & Kim, J. H. (2019). Toxic effects of lead exposure on bioaccumulation, oxidative stress, neurotoxicity, and immune responses in fish: A review. Environmental toxicology and pharmacology, 68, 101-108. https://doi.org/10.1016/j.etap.2019.03.010

- Liang, Y., Wang, R., Sheng, G.D., Pan, L., Lian, E., Su, N., Tang, X., Yang, S. & Yin, D. (2023). Geochemical controls on the distribution and bioavailability of heavy metals in sediments from Yangtze River to the East China Sea: assessed by sequential extraction versus diffusive gradients in thin-films (DGT) technique. Journal of Hazardous Materials, 452, 131253. https://doi.org/10.1016/j.jhazmat.2023.131253

- Liu, J., Dietz, T., Carpenter, S.R., Alberti, M., Folke, C., Moran, E., Pell, A.N., Deadman, P., Kratz, T., Lubchenco, J. & Ostrom, E. (2007).  Complexity of coupled human and natural systems. Science. 317(5844), 1513-1516. https://www.science.org/doi/abs/10.1126/science.1144004

- Loska, K., & Wiechuła, D. (2003). Application of principal component analysis for the estimation of source of heavy metal contamination in surface sediments from the Rybnik Reservoir. Chemosphere, 51(8), 723-733. https://doi.org/10.1016/S0045-6535(03)00187-5

- Mohan, S. V., Nithila, P., & Reddy, S. J. (1996). Estimation of heavy metals in drinking water and development of heavy metal pollution index. Journal of Environmental Science & Health Part A, 31(2), 283-289. https://doi.org/10.1080/10934529609376357

- Muller, G.M.M.G.M.G.M.G.P. (1969). Index of geoaccumulation in sediments of the Rhine River.

- Nishitha, D. S., Sudheer, A. K., Arun, K., Amrish, V. N., Mahesh, G., Udayashankar, H. N., & Balakrishna, K. (2023). Risk assessment and spatio-temporal distribution of dissolved trace metals in Swarna, Sharavati and Kali estuaries, South-West Coast of India. Environmental Science and Pollution Research, 30(4), 9914-9931. https://doi.org/10.1080/15320383.2025.2524754

- Olayinka-Olagunju, J. O., Dosumu, A. A., & Olatunji-Ojo, A. M. (2021). Bioaccumulation of heavy metals in pelagic and benthic fishes of Ogbese River, Ondo State, South-Western Nigeria. Water, Air, & Soil Pollution, 232(2), 44. https://doi.org/10.1007/s11270-021-04987-7

- Pandiyan, J., Mahboob, S., Jagadheesan, R., Elumalai, K., Krishnappa, K., Al-Misned, F., Kaimkhani, Z.A. & Govindarajan, M. (2020). A novel approach to assess the heavy metal content in the feathers of shorebirds: A perspective of environmental research. Journal of King Saud University-Science, 32(7), 3065-3071. https://doi.org/10.1016/j.jksus.2020.08.014.

- Panseriya, H. Z., Gosai, H. B., Sankhwal, A. O., Sachaniya, B. K., Gavali, D. J., & Dave, B. P. (2020). Distribution, speciation and risk assessment of heavy metals: geochemical exploration of Gulf of Kachchh, Gujarat, India. Environmental Earth Sciences, 79(10), 213. https://doi.org/10.1007/s12665-020-08972-x

- Prashanth, L., Kattapagari, K. K., Chitturi, R. T., Baddam, V. R. R., & Prasad, L. K. (2015). A review on role of essential trace elements in health and disease. Journal of Dr. YSR University of Health Sciences, 4(2), 75-85. https://doi.org/10.4103/2277-8632.158577.

- Rashed, M. N. (2001). Monitoring of environmental heavy metals in fish from Nasser Lake. Environment international, 27(1), 27-33. https://doi.org/10.1016/S0160-4120(01)00050-2

- Ray, A., Mondal, D., Chakraborty, N., & Ganguly, S. (2025). Toxicity effects of hexavalent chromium on hematological, biochemical and digestive enzyme profiles of Labeo rohita (Hamilton, 1822). Ecotoxicology, 34(1), 76-88. https://doi.org/10.1007/s10646-024-02815-2

- Santos-Echeandía, J., Prego, R., & Cobelo-García, A. (2008). Influence of the heavy fuel spill from the Prestige tanker wreckage in the overlying seawater column levels of copper, nickel and vanadium (NE Atlantic ocean). Journal of Marine Systems, 72(1-4), 350-357. https://doi.org/10.1016/j.jmarsys.2006.12.005

- Sharma, K., Shah, G., Singhal, K., & Soni, V. (2024). Comprehensive insights into the impact of oil pollution on the environment. Regional Studies in Marine Science, 74, 103516. https://doi.org/10.1016/j.rsma.2024.103516

- Sharma, M., Kant, R., Sharma, A. K., & Sharma, A. K. (2025). Exploring the impact of heavy metals toxicity in the aquatic ecosystem. International Journal of Energy and Water Resources, 9(1), 267-280. https://doi.org/10.1007/s42108-024-00284-1

- Singh, A., Singh, D., & Yadav, H. (2017). Impact and assessment of heavy metal toxicity on water quality, edible fishes and sediments in lakes: a review. Trends Biosci, 10(8), 1551-1560.

- Stark, J. S., Kim, S. L., & Oliver, J. S. (2014). Anthropogenic disturbance and biodiversity of marine benthic communities in Antarctica: a regional comparison. PloS one, 9(6), e98802. https://doi.org/10.1371/journal.pone.0098802

- Steffy, D. A., Nichols, A. C., Morgan, L. J., & Gibbs, R. (2013). Evidence that the Deepwater Horizon oil spill caused a change in the nickel, chromium, and lead average seasonal concentrations occurring in sea bottom sediment collected from the eastern Gulf of Mexico continental shelf between the years 2009 and 2011. Water, Air, & Soil Pollution, 224(11), 1756. https://doi.org/10.1007/s11270-013-1756-1

- Tarasco, M., Cardeira, J., Viegas, M. N., Caria, J., Martins, G., Gavaia, P. J., Cancela, M. L., & Laizé, V. (2019). Anti-osteogenic activity of cadmium in zebrafish. Fishes, 4(1), 11. https://doi.org/10.3390/fishes4010011

- Tomlinson, D. L., Wilson, J. G., Harris, C. R., & Jeffrey, D. W. (1980). Problems in the assessment of heavy-metal levels in estuaries and the formation of a pollution index. Helgoländer meeresuntersuchungen, 33(1), 566-575. https://doi.org/10.1007/BF02414780

- Turekian, K. K., & Wedepohl, K. H. (1961). Distribution of the elements in some major units of the earth's crust. Geological society of America bulletin, 72(2), 175-192. https://doi.org/10.1130/0016-7606(1961)72[175:DOTEIS]2.0.CO;2

- Velusamy, A., Kumar, P. S., Ram, A., & Chinnadurai, S. (2014). Bioaccumulation of heavy metals in commercially important marine fishes from Mumbai Harbor, India. Marine pollution bulletin, 81(1), 218-224.

https://doi.org/10.1016/j.marpolbul.2014.01.049

- Vetrimurugan, E., Shruti, V. C., Jonathan, M. P., Roy, P. D., Sarkar, S. K., Rawlins, B. K., & Villegas, L. E. C. (2019). Comprehensive study on metal contents and their ecological risks in beach sediments of KwaZulu-Natal province, South Africa. Marine pollution bulletin, 149, 110555. https://doi.org/10.1016/j.marpolbul.2019.110555

- Vergilio, C. S., Moreira, R. G., Carvalho, C. E. V., & Melo, E. J. T. (2015). Evolution of cadmium effects in the testis and sperm of the tropical fish Gymnotus carapo. Tissue and Cell, 47(2), 132–139. https://doi.org/10.1016/j.tice.2015.02.001

- Vinanthi Rajalakshmi, K. S., Liu, W.-C., Balamuralikrishnan, B., Meyyazhagan, A., Sattanathan, G., Pappuswamy, M., Joseph, K. S., Paari, K. A., & Lee, J.-W. (2023). Cadmium as an endocrine disruptor that hinders the reproductive and developmental pathways in freshwater fish: A review. Fishes, 8(12), 589. https://doi.org/10.3390/fishes8120589

- Vincy, M. V., Rajan, B., & Pradeep Kumar, A. P. (2012). Water quality assessment of a tropical wetland ecosystem with special reference to backwater tourism, Kerala, South India. International Research Journal of Environment Sciences, 1(5), 62-68.

- Wang, Q., Chen, M., Shan, G., Chen, P., Cui, S., Yi, S., & Zhu, L. (2017). Bioaccumulation and biomagnification of emerging bisphenol analogues in aquatic organisms from Taihu Lake, China. Science of the Total Environment, 598, 814-820. https://doi.org/10.1016/j.scitotenv.2017.04.167

- Wang, Q., & Yang, Z. (2016). Industrial water pollution, water environment treatment, and health risks in China. Environmental pollution, 218, 358-365. https://doi.org/10.1016/j.envpol.2016.07.011

- Xing, Q., Li, L., Lou, M., Bing, L., Zhao, R., & Li, Z. (2015). Observation of oil spills through landsat thermal infrared imagery: A case of deepwater horizon. Aquatic Procedia, 3, 151-156. https://doi.org/10.1016/j.aqpro.2015.02.205

- Xu, Q., Wu, B., & Chai, X. (2022). In situ remediation technology for heavy metal contaminated sediment: a review. International Journal of Environmental Research and Public Health, 19(24), 16767. https://doi.org/10.3390/ijerph192416767

- Yan BeiZhan, Y.B., Passow, U., Chanton, J.P., Nöthig, E.M., Asper, V., Sweet, J., Pitiranggon, M., Diercks, A. & Pak, D. (2016). Sustained deposition of contaminants from the Deepwater Horizon spill.  https://doi.org/10.1073/pnas.1513156113

- Zhang, S., Chen, B., Du, J., Wang, T., Shi, H., & Wang, F. (2022). Distribution, assessment, and source of heavy metals in sediments of the Qinjiang River, China. International Journal of Environmental Research and Public Health, 19(15), 9140. https://doi.org/10.3390/ijerph19159140

- Zhang, J., Zhou, F., Chen, C., Sun, X., Shi, Y., Zhao, H., & Chen, F. (2018). Spatial distribution and correlation characteristics of heavy metals in the seawater, suspended particulate matter and sediments in Zhanjiang Bay, China. PLoS One, 13(8), e0201414. https://doi.org/10.1371/journal.pone.0201414

- Zhu, B., Wang, Q., Shi, X., Guo, Y., Xu, T., & Zhou, B. (2016). Effect of combined exposure to lead and decabromodiphenyl ether on neurodevelopment of zebrafish larvae. Chemosphere, 144, 1646-1654. https://doi.org/10.1016/j.chemosphere.2015.10.056